| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G244 |
| --- | --- |

| Title: | Non-CE6a: Luma-based chroma prediction - Model correction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information / Proposal |  |  |
| Author(s) or Contact(s): | Christophe GISQUET Edouard FRANCOIS Canon Research Centre France Rue de la Touche Lambert 35510 Cesson-Sevigne, FRANCE | Email: | christophe.gisquet@crf.canon.fr edouard.francois@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution presents a modification to the Luma-based chroma intra prediction mode, where the alpha parameter values are checked and replaced by constant values, depending on statistics used to compute the alpha values. It is reported that this method obtains more than 4.2% to 4.5% average coding BD-rate gain for U and V components, with a negligible impact on encoder and decoder runtime. Additionally, simplifications to the OLS computation and reduction in the number of bits needed for the chroma prediction samples computation are evaluated, with reportedly around 3.9% chroma BD-rate gain (i.e. around a 0.3% decrease of the former chroma gain, which is below what is measured for equivalent simplifications in HM4.0).

# Introduction

In HM4.0, the luma-based chroma intra prediction mode uses samples from the causal borders of the block to encode, to compute through OLS algorithm linear parameters (alpha, beta) linking luma and chroma samples. This can be related to several well-known publications on inter-component coding such as [1], which illustrates the block adaptability seen in the current HM4.0 coding mode. Other includes:

- Explicit signaling of either the alpha parameter or a corrective value Error: Reference source not found, for instance by transmitting the index in a set of values of the value performing best

- Explicit signaling at the frame level of said parameter [2].

The explicit signaling may be preferable for ill-cases which generate wrong OLS estimation of the linear model parameters. This can be linked to several phenomenons:

- The quantization affects the samples used in the OLS prediction; with the original sample values, the alpha parameter is actually quite better estimated

- Non-stationary signals

The aim of the proposed method is to detect such ill-cases and thus to define when the alpha parameter computed through OLS should not be used as-is. For this, our method compares the alpha value to various thresholds and replaces said value by a predetermined value sent in the slice header or in the adaptive slice parameter set.

# Technical description

## Reminder on LM prediction

Of interest in the current HM4.0 implementation is the following formulas, used in linear least square solution between causal reconstructed data of downsampled luma component and causal chroma component to derive model parameters  and .

(1),

(2),

whereand indicate reconstructed chroma samples and downsampled luma samples around the target block, indicates total samples number of neighboring data.

Obviously, A2 is always positive, while A1 may not. Let’s call uA1 its absolute value.

## Detection and correction

### Detection by uses of thresholds

Ill-cases are detected and classified according to the following 3 cases:

- If uA1 or A2 are below a first threshold T1
In this case quantization has greatly impacted uA1 and/or A2, making the resulting alpha value potentially wrong both in magnitude and sign; the actual optimal alpha value may vary a lot in the blocks under that case.

- If uA1 or A2 in absolute values are below a second threshold T2
In this case alpha magnitude is more trustworthy, but both quantization and image properties are leading to semi-reliable values

3 sub-cases are considered:

  - If uA1 or A2 in absolute values are below a third threshold T3

This detects cases where A1 or A2 magnitude is not reliable

  - Else if A1 is negative

  - Else if A1 is strictly positive

In these 2 later cases sign and magnitude are considered as more reliable

- Other cases
in this case the alpha value is considered as good enough

The detection and classification process is illustrated in Figure 1.

Figure 1: flowchart of the proposed alternate processing

Concerning the thresholds:

- T1 is linked to the potential error due to quantization, and its formula links it to the QP in the following fashion:

T1 = m1×QP<<(g_uiBitIncrement+iCountShift-2)

With m1 a factor between 0 and 3 transmitted in the syntax

- T2 corresponds to the image properties and controls a range of alpha values; in order to avoid overflows, the check is written in the following manner:

uA1 < m2×(A2>>5)

with again m2 a factor between 0 and 7 transmitted in the syntax

- T3 is a hardcoded value, currently implemented in a manner much identical to above:

uA2 < (A2>>6)

The values m1 and m2 could be computed on-the-fly at the decoder side, but several factors make it worthwhile to transmit them instead:

- Computation of the values would be complex on the decoder side, and the resulting values would also suffer from quantization

- They allow tweaking of the thresholds to the image, even allowing to deactivate specific detections

### Corrective measures

Let us list again the various cases detected:

- A case where quantization has greatly impacted uA1 and/or A2, making the resulting alpha value potentially wrong both in magnitude and sign; the actual optimal alpha value may vary a lot in the blocks under that case

- Cases where magnitude at least is more trustworthy, but where both quantization and image properties are leading to semi-reliable values:

  - Where only the magnitude is somewhat reliable

  - Where sign and magnitude are more reliable

- Cases where the alpha value is good enough compared to the correction available

In all cases except 3), the alpha value is set to a predetermined one, found by the encoder and transmitted in the bitstream. More specifically, what is transmitted is the index of the value to use among the following set:

{ -11,  -9,  -7,  -6,  -5, -4, -3, -2, -1, 1, 2,  3,  4,  5,  7,  9 }/2B

where B is 5 for case 1), 7 for case a and 6 for case b, still allowing the fixed-point computations currently used in HM.

The encoder simply evaluates which of the above values performs best. To achieve this, during encoding analysis, the differences of distortions between each evaluated alpha value and an equivalent of the DC mode are estimated over the current block, and, if positive, added to a global score.

### Results

Please note that this uses the bugfix for the number of bits used in alpha to actually set it to 7. One may find results corresponding to HM4.0 (which amounted to using 9 bits of precision) in section. The results have been cross-checked by P. Bordes and P. Salmon from Technicolor and are available in document G.270. The results can also be found in the “JCTVC-G244-Adaptive.xls” file with this contribution.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -14.1% | -16.8% | -0.1% | -14.2% | -16.0% |
| Class B | 0.0% | -1.8% | -1.0% | 0.0% | -2.0% | -1.0% |
| Class C | -0.1% | -1.3% | -1.7% | -0.1% | -1.6% | -1.5% |
| Class D | -0.1% | -1.4% | -1.4% | -0.1% | -1.5% | -1.5% |
| Class E | 0.0% | -2.6% | -1.6% | -0.1% | -2.9% | -1.7% |
| Overall | 0.0% | -4.2% | -4.5% | -0.1% | -4.4% | -4.3% |
|  | 0.0% | -4.1% | -4.4% | -0.1% | -4.3% | -4.2% |
| Enc Time[%] | 100.4% |  |  | 100.5% |  |  |
| Dec Time[%] | 100.2% |  |  | 100.7% |  |  |

Table 1: results of the normal operation mode

The presented method thus offers more than 4% gain on chroma with less than 1% of runtime increase.

### Image inspection

The U component of the 6th frames of various sequences encoded using the AILC profile are presented here.

#### Nebuta, 6th frame, QP 37, AILC

Figure 2: Nebuta 6th frame, QP 37, AILC, our version

Figure 3: Nebuta 6th frame, QP 37, AILC, HM4.0

Most noticeable loss in the fine details of the beard, but also the lines of the mask.

#### Vidyo3, 6th frame, QP 27, AILC

Figure 4: Vidyo3, 6th frame, QP 27, AILC, our version

Figure 5: Vidyo3, 6th frame, QP 27, AILC, HM4.0

Contrast was increased by 30% to let the differences be more noticeable. The diagrams have greatly been wiped out in the HM4.0 version. The output is also more blocky.

#### People on street, 6th frame, QP 37, AILC

|  | Figure 6: People on street, 6th frame, QP 37, AILC, our version |
| --- | --- |
|  | Figure 7: People on street, 6th frame, QP 37, AILC, original |
|  | Figure 8: People on street, 6th frame, QP 37, AILC, HM4.0 |

Except for the board on the right of the image, neither version keeps enough detail from the original image. However, some of the shadows have been wiped out in the HM4.0 version.

## Proposed simplifications to the LM mode

The proposed following simplifications are implemented on top of the LM mode modifications presented in section 2.2.

### Decimation of the borders

JCTVC-G129 reports the results from M. Budagavi (TI) and K. Sato (Sony), for several subsampling of the borders to reduce the OLS computation complexity, of the BD rate variation:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Subsampling factor | Y | U | V | Y | U | V |
| 1:2 for 16x16 | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% |
| 1:2 for 8x8, 16x16 | 0.1% | 0.5% | 0.4% | 0.1% | 0.5% | 0.3% |
| 1:2 for 8x8, 1:4 for 16x16 | 0.1% | 0.8% | 0.5% | 0.1% | 0.7% | 0.3% |

Table 2: results of decimating the borders as reported by M. Budagavi from TI

We have implemented the last one of these “subsampling” scenarios. This results in having always 8 samples on the border, which allow much simpler circuitry.

### Reduction of the number of significant bits in the alpha values

The current design has settled for restricting the number of significant bits of the alpha values to 7. However, because the input sample values can have 8 or 10 bits, this means the computation of the output chroma samples requires intermediate on 17 bits sometimes. We propose to restrict this number of significant bits to 6, so that routinely available DSP blocks can be reused.

### Results

The results below are obtained using the two above simplifications. The cross-check has been performed by M. Budagavi (TI) in document G.XXX. Detailed results are also available in the “JCTVC-G244-Simplified.xls” file with the present contribution.

When using results from section 2.2.3 as reference, the following results are obtained:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.9% | 0.7% | 0.1% | 0.7% | 0.5% |
| Class B | 0.0% | 0.3% | 0.2% | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | 0.3% | 0.0% | 0.0% | 0.2% | 0.1% |
| Overall | 0.0% | 0.3% | 0.2% | 0.0% | 0.3% | 0.2% |
|  | 0.0% | 0.3% | 0.2% | 0.0% | 0.3% | 0.2% |
| Enc Time[%] | 99.8% |  |  | 100.1% |  |  |
| Dec Time[%] | 98.9% |  |  | 100.9% |  |  |

Table 3: results of the simplifications when using new alternate mode as reference

The modification therefore shows much less decrease in coding efficiency than in the HM4.0.

The results when comparing to HM4.0 are:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -13.4% | -16.3% | 0.0% | -13.7% | -15.6% |
| Class B | 0.0% | -1.5% | -0.9% | 0.0% | -1.7% | -0.9% |
| Class C | -0.1% | -1.2% | -1.6% | -0.1% | -1.6% | -1.5% |
| Class D | -0.1% | -1.3% | -1.4% | -0.1% | -1.4% | -1.4% |
| Class E | 0.0% | -2.4% | -1.6% | -0.1% | -2.7% | -1.5% |
| Overall | 0.0% | -3.9% | -4.3% | 0.0% | -4.2% | -4.2% |
|  | 0.0% | -3.9% | -4.2% | 0.0% | -4.1% | -4.0% |
| Enc Time[%] | 100.2% |  |  | 100.6% |  |  |
| Dec Time[%] | 99.0% |  |  | 101.6% |  |  |

Table 4: results of the simplifications with HM4.0 as reference

One may find results when keeping 7 bits of precision in annex.

# Acknowledgements

We would like to express our gratitude towards P. Bordes and P. Salmon from Technicolor and M. Budagavi from Texas Instruments for cross-checking our results.

# Conclusion

This contribution proposes modifications to the chroma intra prediction method present in HM4.0, where the values used in computing the model parameters are checked and, under specific circumstances, the parameters replaced by adaptively determined global parameters. Experimental results show that the proposed method achieves 0.1%, 4.2% and 4.5% average BD-rate reduction respectively for Y, Cb and Cr components in AIHE test with -0.7, 29.5% and 38.9% BD-rate maximum gain for SteamLocomotive sequences. The encoding/decoding measures show that changes in runtime are negligible. In addition, several simplifications are also proposed, leading to a loss of BD-rate of 0.3% and 0.2% for Cb and Cr when compared to the non-simplified modification.

Since proposed modifications achieve solid coding gain without notable complexity increase, and also allow simplifications to the current HM4.0 implementation, we recommend further study of the proposal in the scope of CE6: Intra mode coding improvements.

# Annex: additional experiments

# References

- Efficient RGB Video Coding Using Adaptive Inter-Plane Residual Prediction, Y.-H. Kima, J.-H. Park, J.-W. Kim, B. Choia and J. Paika, IEEE Trans. on Circuits and Systems for Video Technology, pp. 1051-1056, July 2009

- W. Kim, D. Cho, and H. Kim, "Inter-plane prediction for RGB video coding," Proc, IEEE ICIP, pp.785-788, October 2004.

- Intra Prediction Method based on the linear relationship between the channels for YUV 4:2:0 intra coding, S.H. Lee and N.I. Cho, Proc. IEEE ICIP 2009

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

## Syntax modification

The proposed changes are made on the basis of JCTVC-F803d3, with the slice header chosen as the place where to store the alternate alpha information. The use of such syntax in an Adaptive Parameter Set should be easily derived and is thus not presented.

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| lm_chroma_parameters_flag | u(1) |
| if( lm_chroma_parameters_flag ) { |  |
| lm_chroma_param( ) |  |
| } |  |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

#### Alternate LM chroma prediction parameters decoding

| lm_chroma_param( ) { | Descriptor |
| --- | --- |
| quantization_alternate_threshold[0] | u(2) |
| if( quantization_alternate_threshold[0] ) |  |
| quantization_alternate_alpha_index[0] | u(4) |
| image_alternate_threshold[0] | u(3) |
| if( image_alternate_threshold[0] ) { |  |
| image_alternate_low_alpha_index[0] | u(4) |
| image_alternate_positive_alpha_index[0] | u(4) |
| image_alternate_negative_alpha_index[0] | u(4) |
| } |  |
| quantization_alternate_threshold[1] | u(2) |
| if( quantization_alternate_threshold[1] ) |  |
| quantization_alternate_alpha_index[1] | u(4) |
| image_alternate_threshold[1] | u(3) |
| if( image_alternate_threshold[1] ) { |  |
| image_alternate_low_alpha_index[1] | u(4) |
| image_alternate_positive_alpha_index[1] | u(4) |
| image_alternate_negative_alpha_index[1] | u(4) |
| } |  |
| } |  |

## Decoding process modification

##### Specification of Intra_FromLuma prediction mode

Inputs to this process are:

–	the chroma component index uiChromaIdx

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 35.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- Variable k3 and the sample array pY’ are derived as:

k3 = Max( 0, BitDepthC + log2( nS ) – 14 )		(8-1)

pY’[ x, y ] = ( recSamplesL[ 2x, 2y ] + recSamplesL[ 2x, 2y+1 ] ) >> 1, with x, y = -1..2*nS	(8-2)

- Variables L, C, LL, LC and k2 are derived as follows:

L = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k3		(8-3)

C = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k3		(8-4)

LL = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k3	(8-5)

LC = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k3	(8-6)

k2 = log2( (2*nS) >> k3 )		(8-7)

- Variables a and b are derived as:

a1 	 = ( LC << k2 ) – L*C		(8-8)
a2 	 = ( LL << k2 ) – L*L		(8-9)
ua1 = abs(a1) 		(8-10)
uiT1 = (quantization_alternate_threshold[uiChromaIdx]*QP)<<( BitDepthC + log2( nS ) – 10 ) 	(8-11)
if( ua1<uiT1 || a2<uiT1 ) {		(8-12)
    a            = lmAlternateAlpha[ quantization_alternate_alpha_index[uiChromaIdx] ]	(8-13)
    iShift     = 5		(8-14)
} else if( ua1 < image_alternate_threshold[uiChromaIdx] * (a2>>5) ) {	(8-15)
     if( ua1 < (a2>>6) ) {		(8-16)
        a        = lmAlternateAlpha[ alternate_low_alpha[uiChromaIdx] ]	(8-17)
        iShift = 7		(8-18)
     } else if ( a1 < 0 ) {		(8-19)
        a        = lmAlternateAlpha[ alternate_negative_alpha[uiChromaIdx] ]	(8-20)
        iShift = 6		(8-21)
     } else {		(8-22)
        a        = lmAlternateAlpha[ alternate_positive_alpha[uiChromaIdx] ]	(8-23)
        iShift = 6		(8-24)
}				(8-25)
else {			(8-26)
     k 	   = Max( 0, log2( abs( a2 ) ) – 5 ) – Max( 0, log2( abs( a1 ) ) – 14 ) + 2	(8-27)
     a1s	   = a1 >> Max(0, log2( abs( a1 ) ) – 14 )		(8-28)
     a2s	   = abs( a2 >> Max(0, log2( abs( a2 ) ) – 5 ) ) + 1	(8-29)
     iShift = 13		(8-30)
}				(8-31)

[Ed. (WJ): bug in software – a2s should be abs( a2 >> Max(0, log2( abs( a2 ) ) – 5 ) ) – without ‘+1’]

a	= Clip3( -215, 215-1, a1s*lmDiv + ( 1 << ( k1 – 1 ) ) >> k1 )	(8-32)
}				(8-33)

b	= ( L – ( ( a*C ) >> k1 ) + ( 1 << ( k2 – 1 ) ) ) >> k2	(8-34)

where lmDiv is specified in Error: Reference source not found with the input a2s and lmAlternateAlpha below.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * a ) >> iShift ) + b ), with x, y = 0..nS-1	(8-35)

lmAlternateAlpha[16] = [ -11,  -9,  -7,  -6,  -5, -4, -3, -2, -1, 1, 2,  3,  4,  5,  7,  9 ]

# Annex: supplementary results

## Normal mode: use for normal alpha of 9 bits instead of 7bits

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -14.1% | -16.9% | -0.1% | -14.3% | -16.0% |
| Class B | 0.0% | -1.8% | -1.0% | 0.0% | -2.0% | -1.0% |
| Class C | -0.1% | -1.4% | -1.7% | -0.1% | -1.7% | -1.5% |
| Class D | -0.1% | -1.4% | -1.4% | -0.1% | -1.5% | -1.5% |
| Class E | 0.0% | -2.7% | -1.7% | -0.1% | -2.9% | -1.7% |
| Overall | 0.0% | -4.2% | -4.5% | -0.1% | -4.4% | -4.3% |
|  | 0.0% | -4.2% | -4.4% | -0.1% | -4.3% | -4.2% |
| Enc Time[%] | 100.2% |  |  | 100.7% |  |  |
| Dec Time[%] | 99.9% |  |  | 101.8% |  |  |

Table 5: Results of using 9 bits instead of 7 for the normal alpha case

There is therefore no difference in coding efficiency, so the benefit observed in HM4.0 of reducing the number of significant bits no longer applies. This may be interpreted as the fact that such benefit is covered by the alternate handling in this contribution.

## Simplified mode: use for normal alpha of 7 bits instead of 6

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.8% | 0.6% | 0.1% | 0.7% | 0.4% |
| Class B | 0.0% | 0.3% | 0.2% | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.2% | 0.0% | 0.0% | 0.3% | 0.1% |
| Overall | 0.0% | 0.3% | 0.2% | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.3% | 0.2% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 99.8% |  |  | 100.3% |  |  |
| Dec Time[%] | 99.8% |  |  | 101.5% |  |  |

Table 6: Results of using 7 bits instead of 6 for the normal alpha case, with simplifications

The results can be interpreted as the fact that the number of significant bits of the alpha parameter hardly matter in the simplified mode (as well as the normal mode).

## Normal mode: use of non-adaptive alternate alpha values

In this case, we keep the number of significant bits to 7, and we use fixed alpha values, found by taking the most frequent ones in the normal mode (i.e. no R/D optimization over the set of test sequences was performed). The fixed values are still written to the bitstream, so those settings can be interpreted as a fast encoding operating mode.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -11.0% | -11.8% | 0.0% | -11.4% | -10.7% |
| Class B | 0.0% | -1.6% | -0.3% | 0.0% | -2.1% | -0.1% |
| Class C | 0.0% | -1.4% | -0.4% | 0.0% | -1.9% | -0.5% |
| Class D | 0.0% | -1.4% | -0.2% | 0.0% | -1.6% | -0.1% |
| Class E | 0.0% | -2.3% | -0.1% | 0.0% | -4.1% | -0.1% |
| Overall | 0.0% | -3.5% | -2.6% | 0.0% | -4.1% | -2.3% |
|  | 0.0% | -3.4% | -2.5% | 0.0% | -4.1% | -2.2% |
| Enc Time[%] | 99.8% |  |  | 99.9% |  |  |
| Dec Time[%] | 100.1% |  |  | 103.3% |  |  |

Table 7: Results when using non-adaptive alternate alpha values

These default values seem to be more beneficial for U (less than 1% loss) than for V (around 2% loss) when compared to the adaptive case. As no efficient optimization was performed, the results are only indicative of the fact it is still possible to use fixed values and obtain a gain.