| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G245r1 |
| --- | --- |

| Title: | Non-CE6a: Use of chroma phase in LM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Christophe GISQUET Stephane PAUTET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr christophe.gisquet@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution relates to the luma-based chroma prediction mode (LM mode) and consists of two proposals.

The first proposal generalizes the LM mode to handle generic chroma phase configurations. It consists of signaling in the SPS chroma phase information and to use this information in the luma interpolation process. The approach has been tested using new generated test sequences, corresponding to different chroma phase configurations. For the most favorable chroma phase configuration, average BD-rate gains of 0.5% AV* (0.1% Y, 1.6% U, 1.7 % V) AIHE and 0.4% AV* (0.1% Y, 1.5% U, 1.5% V) AILC are reported. For the least favorable chroma phase configuration, average gains of 0.2% AV* (0% Y, 0.6% U, 0.7% V) AIHE and 0.2% AV* (0% Y, 0.4% U, 0.4% V AILC) are reported. RA configuration benefits are also reported to be in same range. Gains up to 1.1% AV* (0.3% luma, 5.6% chroma) are reported on high resolution sequence classes.

The second proposal intends to reduce the number of operations involved in the luma interpolation mainly by removing right shift operations in the luma filtering process. In total, (W.H+W+H) right shifts, 2W additions and W multiplications by 2 (or left shifts) are removed when LM mode is applied, with W and H being the chroma block width and height. The impact on coding efficiency is reportedly negligible for AI, RA and LD configurations.

* AV-BDR is obtained using the following PSNR: PSNRAV = (6*PSNRY + PSNRU + PSNRV)/8

# Introduction

The luma-based chroma prediction mode consists of predicting the chroma signal from the already reconstructed luma signal. This tool explicitly establishes a link between chroma and luma samples, using a linear model.

In its current version, this tool addresses the 4:2:0 format, where chroma samples resolution is half the luma samples resolution in both horizontal and vertical dimensions. Because of the different resolutions of luma and chroma, an interpolation process is applied to luma samples to generate interpolated luma samples at same resolution as the chroma samples to be predicted. This interpolation actually considers that chroma samples are positioned at said nominal position related to luma samples.

The contribution aims at generalizing the LM mode for any chroma phase. It consists of taking into account the chroma samples position (chroma phases) in the luma interpolation process in case of 4:2:0 content. This information is explicitly signaled in the sequence parameter set. The interpolation process of luma samples is modified so that interpolated luma samples are located at same position as chroma samples to be predicted, whatever the considered chroma phase.

# Chroma phase in current HM design

## Chroma phase definition

The chroma phase corresponds to the location of the chroma samples related to the luma samples. In Figure 1 different possible positions of the chroma samples related to the luma samples in a 4:2:0 content are considered. To each position corresponds a couple of horizontal and vertical chroma phases (Cx,Cy), in unit of half luma samples of a frame.

Figure 1: chroma phases in 4:2:0 format.

Chroma phase is already implicitly specified in the current HM WD [1], in the same way as in H.264/MPEG-4 AVC. The syntax element chroma_format_idc defines the chroma format sampling structure. In case of 4:2:0 content, corresponding to chroma_format_idc equal to 1, the nominal vertical and horizontal relative locations of luma and chroma samples in pictures correspond to (Cx= 0, Cy=1).

In addition, in H.264/MPEG-4 AVC, alternative chroma sample relative locations may be indicated in case of 4:2:0 content in video usability information.

Chroma phases are also specified in the SVC extension of H.264/MPEG-4 AVC, in order to accurately control the inter-layer chroma prediction.

## Chroma phase in current LM mode design

In the LM mode, the nominal position is also explicitly considered. The interpolated luma samples RecL’[x,y] used to derive the linear model parameters and to predict the chroma signal are virtually located at the nominal position, as illustrated in Figure 2. The filters used to generate the collocated interpolated luma samples are build based on this assumption:

- For generating the left border and inner luma samples, vertical filter [1,1]/2 is used

  - RecL’[-1,y] = 	( RecL[-1,2y] + RecL[-1,2y+1] ) >> 1	      for the left border samples

  - RecL’[x,y] = 	( RecL[2x,2y] + RecL[2x,2y+1] ) >> 1	      for the inner samples

- For generating the top border luma samples, horizontal filter [1,2,1]/4 is used

  - RecL’[x,-1] = 	( RecL[2x-1,-1] + 2 RecL[2x,-1] + RecL[2x+1,-1] + 2 ) >> 2

Figure 2: position of luma-chroma samples in LM mode.

# Proposal 1: LM mode generalization with chroma phase

In this proposal, it is suggested to generalize the LM mode to others cases than the nominal chroma samples positioning. The 4 cases depicted in Figure 1 are considered. In case of 4:2:0 content, the chroma phase relative to the luma samples position is signaled in the Sequence Parameter Set, as described in section 3.1. The interpolation process of the luma samples is modified so that interpolated luma samples RecL’[x,y] are located at the same position as the chroma samples to be predicted, whatever the considered chroma phase, as described in section 3.2.

## Syntax changes

It is first proposed to insert syntax elements related to chroma phases into the Sequence Parameter Set.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) { |  |
| chroma_phase_x | u(1) |
| chroma_phase_y | u(1) |
| } |  |
| max_temporal_layers_minus1 | u(3) |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

chroma_phase_x specifies the horizontal phase shift of the chroma components in units of half luma samples of a frame (possible values 0,1). If chroma_phase_x is not present, it shall be inferred to be equal to 0.

chroma_phase_y specifies the vertical phase shift of the chroma components in units of half luma samples of a frame (possible values 0,1). If chroma_phase_y is not present, it shall be inferred to be equal to 0.

## Modification of luma signal interpolation process

It is proposed to modify the interpolation filters of luma samples RecL’ as follows.

Let (Cx,Cy) being defined as (chroma_phase_x, chroma_phase_y).

- For generating the left border, the following filter is used

  - RecL’[-1,y] = 	( RecL[-1,2y] + Cy.RecL[-1,2y+1] ) >> Cy

- For generating the inner luma samples, the following filter is used

  - RecL’[x,y] = 	( RecL[2x,2y] + Cx.RecL[2x+1,2y] +

Cy.RecL[2x,2y+1] + Cx.Cy.RecL[2x+1,2y+1] ) >> (Cx+Cy)

- For generating the top border luma samples, horizontal filter [1,2,1]/4 is used

  - RecL’[x,-1] = 	( ( RecL[2x-1,-1] + 2RecL[2x,-1] + RecL[2x+1,-1] + 2 ) >> 2 +

Cx*( RecL[2x,-1] + 2RecL[2x+1,-1] + RecL[2x+2,-1] + 2 ) >> 2 ) >>

Cx

This is actually a generalization of the current interpolation process. When Cx and Cy are equal to 0, these equations correspond to the current process.

## Test conditions

To evaluate the proposed solution, new test sequences with specific chroma phases are required.

Three types of sequences have been generated:

- From 4:2:0 test sequences, with re-phasing of chroma samples.

- From 4:4:4 content, with downsampling of chroma components with specific chroma samples phases.

- From 4:2:0 test sequences, with downsampling of luma and chroma components, and re-phasing of chroma samples.

The process for generating these sequences is described in Annex.

In total, 10 classes are generated, for each considered chroma phases, grouped in 2 test sets:

- Test set 1

  - Class A’: 	chroma re-phased version of the 4:2:0 class A sequences

  - Class B’: 	chroma re-phased version of the 4:2:0 class B sequences

  - Class C’: 	chroma re-phased version of the 4:2:0 class C sequences

  - Class D’: 	chroma re-phased version of the 4:2:0 class C sequences

  - Class E’: 	chroma re-phased version of the 4:2:0 class E sequences

- Test set 2

  - Class AP: 	downsampled version of the 4:2:0 class A sequences

  - Class BP: 	downsampled version of the 4:2:0 class B sequences

  - Class CP: 	downsampled version of the 4:2:0 class C sequences

  - Class EP: 	downsampled version of the 4:2:0 class E sequences

  - Class P: 	4:4:4 1080p sequences converted to 4:2:0 with re-phased downsampled chroma

## Results

The proposed tool is evaluated for the different 3 considered chroma locations (Cx=0, Cy=0), (Cx=1, Cy=0), (Cx=1, Cy=1), using conventions of Figure 1. For each class, each sequence is available in 3 versions, corresponding to the 3 considered phases. The fourth possible chroma location corresponds to the default HM configuration and is therefore not tested.

For a given chroma location, the corresponding sequences are encoded using HM4.0 with LM bug fix as reference, and using the proposed solution configured with the right chroma phases.

It can be noticed that taking into account the chroma phases gives noticeable gain for the chroma components, especially for the (Cx=1,Cy=0) and (Cx=1,Cy=1) cases. For these 2 cases, luma BD-rate is also improved by 0.1% in average. BD-rate gains are up to 5-6% in chroma for high resolution high resolution sequences (class A’) and sequences generated from 4:4:4 content (class P). Luma BD-rate can also be improved by up to 0.3%.

The case (Cx=0,Cy=0) is less favorable, probably due to a more aliased interpolated luma signal (only one luma sample RecL is used to generate the interpolated sample RecL’). Low-pass filtering could be applied to the luma samples RecL to limit this drawback, as suggested in JCTVC-F173/F502.

The full results are provided in the attached xls files.

- files ‘JCTVC_G245_xxx_HEVCSeq_yyy.xls’ correspond to the results obtained using the new sequences test set (set1).

- Files ‘JCTVC_G245_xxx_HEVCSeq_yyy.xls’ correspond to the results obtained using the HEVC test set (set2);

### Chroma location (Cx=1, Cy=0)

| Set1 | All Intra HE |  |  | All Intra LC |  |  | Set2 | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| A’ | -0.2% | -5.1% | -2.1% | -0.2% | -4.9% | -2.2% | AP | -0.1% | -1.4% | -0.2% | 0.0% | -1.3% | -0.3% |
| B’ | 0.0% | -1.3% | 0.3% | 0.0% | -1.1% | 0.3% | BP | -0.1% | -1.9% | 0.2% | -0.1% | -1.8% | 0.2% |
| C’ | -0.3% | -1.9% | -2.1% | -0.3% | -1.7% | -1.8% | CP | -0.3% | -2.5% | -2.5% | -0.3% | -2.1% | -2.1% |
| D’ | -0.2% | -1.0% | -1.2% | -0.2% | -1.0% | -1.2% | EP | 0.0% | 0.0% | -0.5% | 0.0% | 0.3% | -0.1% |
| E’ | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.1% | P | -0.1% | -1.9% | -5.6% | -0.1% | -1.8% | -5.4% |
| All | -0.1% | -1.9% | -1.0% | -0.1% | -1.8% | -1.0% | All | -0.1% | -1.6% | -1.7% | -0.1% | -1.5% | -1.5% |
|  | -0.1% | -1.9% | -1.0% | -0.1% | -1.7% | -1.0% |  | -0.1% | -1.6% | -1.7% | -0.1% | -1.4% | -1.5% |
| Enc | 100% |  |  | 99% |  |  | Enc | 100% |  |  | 100% |  |  |
| Dec | 100% |  |  | 100% |  |  | Dec | 100% |  |  | 100% |  |  |

| Set1 | Random Access HE |  |  | Random Access LC |  |  | Set2 | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| A’ | 0.0% | -5.5% | -2.0% | 0.0% | -5.5% | -2.5% | AP | 0.0% | -2.0% | -0.7% | 0.0% | -1.6% | 0.1% |
| B’ | 0.0% | -1.1% | 0.3% | 0.0% | -1.1% | 0.1% | BP | 0.0% | -1.8% | 0.2% | 0.0% | -1.9% | 0.1% |
| C’ | -0.1% | -1.4% | -1.4% | -0.1% | -1.2% | -1.4% | CP | -0.1% | -1.9% | -1.8% | -0.2% | -1.9% | -1.8% |
| D’ | -0.1% | -0.8% | -1.0% | -0.1% | -0.8% | -0.9% | EP |  |  |  |  |  |  |
| E’ |  |  |  |  |  |  | P | 0.0% | -3.0% | -5.6% | -0.1% | -2.3% | -5.6% |
| All | 0.0% | -2.1% | -1.0% | 0.0% | -2.1% | -1.1% | All | 0.0% | -2.1% | -1.9% | -0.1% | -1.9% | -1.7% |
|  | 0.0% | -2.1% | -1.0% | 0.0% | -2.1% | -1.1% |  | 0.0% | -2.1% | -1.9% | -0.1% | -1.9% | -1.7% |
| Enc | 100% |  |  | 100% |  |  | Enc | 100% |  |  | 100% |  |  |
| Dec | 100% |  |  | 100% |  |  | Dec | 99% |  |  | 97% |  |  |

| Set1 | Low delay B HE |  |  | Low delay B LC |  |  | Set2 | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| A’ |  |  |  |  |  |  | AP |  |  |  |  |  |  |
| B’ | 0.0% | -0.2% | -0.2% | 0.0% | -0.4% | -0.2% | BP | 0.0% | -0.4% | -0.4% | 0.0% | -0.9% | -0.3% |
| C’ | 0.0% | -0.4% | -1.2% | 0.0% | -0.7% | -0.8% | CP | 0.0% | -0.9% | -0.6% | -0.1% | -0.8% | -0.7% |
| D’ | -0.1% | -0.2% | -0.8% | 0.0% | -0.7% | -0.2% | EP | 0.0% | -1.0% | -1.8% | 0.0% | -0.8% | -0.2% |
| E’ | 0.1% | -0.1% | -1.5% | -0.1% | 0.6% | -1.0% | P | 0.0% | -1.9% | -2.2% | -0.2% | -0.6% | -2.5% |
| All | 0.0% | -0.2% | -0.8% | 0.0% | -0.3% | -0.5% | All | 0.0% | -1.0% | -1.2% | -0.1% | -0.8% | -0.9% |
|  | 0.0% | -0.4% | -0.8% | 0.0% | -0.4% | -0.5% |  | 0.0% | -1.1% | -1.1% | -0.1% | -0.8% | -1.0% |
| Enc | 101% |  |  | 100% |  |  | Enc | 100% |  |  | 99% |  |  |
| Dec | 100% |  |  | 99% |  |  | Dec | 100% |  |  | 101% |  |  |

### Chroma location (Cx=1, Cy=1)

| Set1 | All Intra HE |  |  | All Intra LC |  |  | Set2 | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| A’ | -0.1% | -2.1% | -1.0% | -0.1% | -2.0% | -1.0% | AP | -0.1% | -2.2% | -0.6% | -0.1% | -2.0% | -0.5% |
| B’ | 0.0% | -0.6% | 0.3% | 0.0% | -0.7% | 0.2% | BP | -0.1% | -1.1% | -0.1% | -0.1% | -1.3% | -0.1% |
| C’ | -0.3% | -1.9% | -1.7% | -0.3% | -1.8% | -1.7% | CP | -0.3% | -2.2% | -2.0% | -0.3% | -2.1% | -2.0% |
| D’ | -0.2% | -1.2% | -1.3% | -0.2% | -1.3% | -1.4% | EP | 0.0% | 0.3% | -0.6% | 0.0% | 0.2% | -0.4% |
| E’ | 0.0% | 0.2% | -0.1% | 0.0% | 0.2% | 0.0% | P | -0.1% | -1.2% | -3.9% | -0.1% | -1.2% | -3.9% |
| All | -0.1% | -1.2% | -0.7% | -0.1% | -1.2% | -0.8% | All | -0.1% | -1.3% | -1.4% | -0.1% | -1.4% | -1.4% |
|  | -0.1% | -1.1% | -0.7% | -0.1% | -1.1% | -0.7% |  | -0.1% | -1.3% | -1.4% | -0.1% | -1.3% | -1.4% |
| Enc | 99% |  |  | 99% |  |  | Enc | 100% |  |  | 99% |  |  |
| Dec |  |  |  |  |  |  | Dec | 99% |  |  | 98% |  |  |

| Set1 | Random Access HE |  |  | Random Access LC |  |  | Set2 | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| A’ | 0.0% | -1.3% | -1.4% | 0.0% | -1.5% | -0.9% | AP | -0.1% | -2.8% | -0.7% | 0.0% | -2.0% | -1.2% |
| B’ | 0.0% | -0.7% | 0.2% | 0.0% | -0.7% | 0.2% | BP | 0.0% | -1.1% | 0.1% | -0.1% | -1.0% | -0.1% |
| C’ | -0.1% | -1.4% | -1.2% | -0.1% | -1.3% | -1.4% | CP | -0.1% | -1.6% | -1.9% | -0.2% | -1.7% | -1.6% |
| D’ | -0.1% | -0.9% | -1.1% | -0.1% | -1.2% | -1.3% | EP |  |  |  |  |  |  |
| E’ |  |  |  |  |  |  | P | 0.1% | -1.5% | -3.6% | 0.0% | -1.6% | -3.1% |
| All | -0.1% | -1.0% | -0.8% | -0.1% | -1.1% | -0.8% | All | 0.0% | -1.7% | -1.4% | -0.1% | -1.5% | -1.4% |
|  | -0.1% | -1.0% | -0.8% | -0.1% | -1.1% | -0.8% |  | -0.1% | -1.6% | -1.4% | -0.1% | -1.5% | -1.4% |
| Enc | 100% |  |  | 100% |  |  | Enc | 100% |  |  | 100% |  |  |
| Dec |  |  |  |  |  |  | Dec | 99% |  |  | 99% |  |  |

| Set1 | Low delay B HE |  |  | Low delay B LC |  |  | Set2 | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| A’ |  |  |  |  |  |  | AP |  |  |  |  |  |  |
| B’ | 0.0% | -0.1% | -0.3% | 0.0% | -0.1% | -0.3% | BP | -0.1% | -0.3% | -1.0% | 0.0% | -0.3% | -0.3% |
| C’ | 0.0% | -0.4% | -0.6% | 0.0% | -0.4% | -0.6% | CP | 0.0% | -0.5% | -0.4% | 0.0% | -0.8% | -0.7% |
| D’ | 0.0% | 0.3% | -0.5% | 0.0% | 0.3% | -0.5% | EP | 0.0% | 0.5% | -0.2% | 0.0% | -1.0% | -1.2% |
| E’ | -0.1% | 0.7% | -0.2% | -0.1% | 0.7% | -0.2% | P | 0.1% | 0.1% | -1.7% | 0.0% | -0.6% | -1.7% |
| All | 0.0% | 0.1% | -0.4% | 0.0% | 0.1% | -0.4% | All | 0.0% | -0.1% | -0.9% | 0.0% | -0.7% | -0.9% |
|  | 0.0% | 0.1% | -0.4% | 0.0% | 0.1% | -0.4% |  | 0.0% | -0.1% | -0.8% | 0.0% | -0.7% | -0.9% |
| Enc | 100% |  |  | 100% |  |  | Enc | 100% |  |  | 100% |  |  |
| Dec |  |  |  |  |  |  | Dec | 99% |  |  | 98% |  |  |

### Chroma location (Cx=0, Cy=0)

| Set1 | All Intra HE |  |  | All Intra LC |  |  | Set2 | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| A’ | -0.1% | -3.9% | -1.4% | -0.1% | -3.7% | -1.6% | AP | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% |
| B’ | 0.0% | -0.6% | 0.2% | 0.0% | -0.4% | 0.2% | BP | 0.0% | -0.8% | 0.2% | 0.0% | -0.6% | 0.3% |
| C’ | 0.0% | -0.5% | -0.7% | 0.0% | -0.4% | -0.5% | CP | 0.0% | -0.8% | -0.9% | 0.0% | -0.4% | -0.4% |
| D’ | 0.1% | 0.1% | -0.1% | 0.0% | 0.2% | 0.1% | EP | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.4% |
| E’ | 0.0% | -0.3% | -0.1% | 0.0% | -0.3% | -0.1% | P | 0.0% | -1.0% | -3.0% | 0.0% | -0.8% | -2.6% |
| All | 0.0% | -1.1% | -0.4% | 0.0% | -0.9% | -0.4% | All | 0.0% | -0.6% | -0.7% | 0.0% | -0.4% | -0.4% |
|  | 0.0% | -1.0% | -0.4% | 0.0% | -0.9% | -0.4% |  | 0.0% | -0.6% | -0.6% | 0.0% | -0.3% | -0.5% |
| Enc | 100% |  |  | 100% |  |  | Enc | 100% |  |  | 99% |  |  |
| Dec |  |  |  |  |  |  | Dec | 100% |  |  | 100% |  |  |

| Set1 | Random Access HE |  |  | Random Access LC |  |  | Set2 | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| A’ | 0.1% | -4.3% | -1.0% | 0.0% | -4.6% | -2.3% | AP | 0.0% | 0.4% | 0.2% | 0.0% | -0.1% | -0.3% |
| B’ | 0.0% | -0.5% | 0.2% | 0.0% | -0.5% | 0.2% | BP | 0.0% | -0.8% | 0.3% | 0.0% | -0.7% | 0.4% |
| C’ | 0.0% | 0.0% | -0.3% | 0.0% | -0.2% | -0.6% | CP | 0.0% | -0.6% | -0.6% | 0.0% | -0.4% | -0.4% |
| D’ | 0.0% | 0.3% | 0.0% | 0.1% | 0.3% | 0.4% | EP |  |  |  |  |  |  |
| E’ |  |  |  |  |  |  | P | 0.0% | -1.5% | -3.1% | 0.0% | -1.0% | -3.1% |
| All | 0.0% | -1.1% | -0.3% | 0.0% | -1.2% | -0.5% | All | 0.0% | -0.6% | -0.7% | 0.0% | -0.5% | -0.8% |
|  | 0.0% | -1.1% | -0.3% | 0.0% | -1.2% | -0.5% |  | 0.0% | -0.7% | -0.7% | 0.0% | -0.5% | -0.8% |
| Enc | 100% |  |  | 100% |  |  | Enc | 100% |  |  | 100% |  |  |
| Dec |  |  |  |  |  |  | Dec | 98% |  |  | 100% |  |  |

| Set1 | Low delay B HE |  |  | Low delay B LC |  |  | Set2 | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  | Y | U | V | Y | U | V |
| A’ |  |  |  |  |  |  | AP |  |  |  |  |  |  |
| B’ | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | BP | 0.1% | -0.1% | 0.3% | 0.0% | -0.3% | 0.4% |
| C’ | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.4% | CP | 0.0% | -0.2% | -0.5% | 0.0% | -0.2% | 0.0% |
| D’ | 0.0% | -0.4% | 0.0% | 0.0% | 0.1% | -0.3% | EP | 0.0% | -1.4% | 0.7% | -0.1% | -0.3% | -0.1% |
| E’ | -0.1% | -0.7% | 0.1% | 0.0% | 0.0% | 0.2% | P | 0.0% | -1.3% | -1.3% | -0.2% | -1.2% | -1.7% |
| All | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | -0.1% | All | 0.0% | -0.7% | -0.2% | -0.1% | -0.5% | -0.3% |
|  | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | -0.1% |  | 0.0% | -0.6% | -0.3% | -0.1% | -0.5% | -0.3% |
| Enc | 100% |  |  | 100% |  |  | Enc | 100% |  |  | 101% |  |  |
| Dec |  |  |  |  |  |  | Dec | 99% |  |  | 100% |  |  |

## Conclusion

In this contribution, only 4:2:0 format is handled. The concept could be generalized to any chroma format, considering also the chroma samples resolution relative to the luma samples resolution. Chroma phases and resolution can also be specific to each color component. Generalization to Bayer sampling can also be considered.

# Proposal 2: luma interpolation process simplification

Proposal 2 also relates to the interpolation process for generating RecL’ luma samples. It is proposed to simplify this process by removing some operations, as described in the next section.

## Modified LM mode

The proposed simplification consists of the following filtering process to generate the luma samples RecL’.

- For generating the left border, vertical filter [1,1] is used

  - RecL’[-1,y] = 	RecL[-1,2y] + RecL[-1,2y+1]

instead of

  - RecL’[-1,y] = 	( RecL[-1,2y] + RecL[-1,2y+1] ) >> 1

- For generating the inner luma samples, vertical filter [1,1] is used

  - RecL’[x,y] = 	RecL[2x,2y] + RecL[2x,2y+1]

instead of

  - RecL’[x,y] = 	( RecL[2x,2y] + RecL[2x,2y+1] ) >> 1

- For generating the top border luma samples, horizontal filter [1,0,1] is used

  - RecL’[x,-1] = 	RecL[2x-1,-1] + RecL[2x+1,-1]

instead of

  - RecL’[x,-1] = 	( RecL[2x-1,-1] + 2 RecL[2x,-1] + RecL[2x+1,-1] + 2 ) >> 2

The computation of the linear model parameters a and b used to predict the chroma signal from the interpolation luma signal RecL’ is not modified.

The final chroma interpolation process is modified as follows:

- Current design: 		RecC[x,y] = ( (a.RecL’[x,y]) >> shift ) + b

- Proposal:		RecC[x,y] = ( (a.RecL’[x,y]) >> (shift+1) ) + b

The table below compares the number of operations involved in current LM mode design and in the proposed modified LM mode.

|  | Upper line samples | Left column samples | Inner samples | total |
| --- | --- | --- | --- | --- |
| HEVC | 3W additions W left shifts W right shifts | H additions H right shifts | W*H additions W*H right shifts | 2W+H+W*H additions W left shifts W+H+W*H right shifts |
| proposed | W additions | H additions | W*H additions | W+H+W*H additions - - |
| difference | 2W additions W left shifts W right shifts | H right shifts | W*H right shifts | 2W additions W left shifts W+H+W*H right shifts |

## Results

Test conditions specified in JCTVC-F900 have been used. HM4.0 with LM bug fix has been used as reference. Full results are provided in attached files (JCTVC-G245_HM4xxx.xls). The table below shows a minor average impact of the proposed modification on BD-rate performance.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.1% | -0.4% | -0.1% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | 0.2% | -0.2% | 0.0% | 0.1% | -0.1% |
| Class D | -0.1% | -0.3% | 0.7% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.8% | 0.5% | 0.1% | 1.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.3% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The modification has also been tested on top of the LM luma samples subsampling, as proposed by TI in JCTVC-F233 (subsampling of 2 for 16x16 PUs). Results are very similar to those obtained when the proposed simplification is not applied. They are also consistent with the results shown in the previous table.

## Generalization with chroma phase-adapted filters

Using the proposed simplification, the following generic filters depending on the input chroma phases (Cx,Cy) could be used:

- For generating the left border, the following filter is used

  - RecL’[-1,y] = 	( RecL[-1,2y] + Cy.RecL[-1,2y+1] ) << Cx

instead of

  - RecL’[-1,y] = 	( RecL[-1,2y] + Cy.RecL[-1,2y+1] ) >> Cy

- For generating the inner luma samples, the following filter is used

  - RecL’[x,y] = 	( RecL[2x,2y] + Cx.RecL[2x+1,2y] +

Cy.RecL[2x,2y+1] + Cx.Cy.RecL[2x+1,2y+1] )

instead of

  - RecL’[x,y] = 	( RecL[2x,2y] + Cx.RecL[2x+1,2y] +

Cy.RecL[2x,2y+1] + Cx.Cy.RecL[2x+1,2y+1] ) >> (Cx+Cy)

- For generating the top border luma samples, the following filter is used

  - RecL’[x,-1] = 	( RecL[2x-1,-1] + Cx.RecL[2x,-1] +

RecL[2x+1,-1] + Cx.RecL[2x+2,-1] ) >> (1-Cy)

instead of

  - RecL’[x,-1] = 	( RecL[2x-1,-1] + (2+Cx).RecL[2x,-1] +

(1+2Cx).RecL[2x+1,-1] ) + Cx.RecL[2x+2,-1] ) >> (2+Cx)

# References

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand., “WD4: Working Draft 4 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F803, 6th Meeting: Torino, Italy, July, 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, Italy, July, 2011.

- Tom McMahon., “Film-Originated Test Sequences”, Joint Video Team (JVT) of ISO/IEC JPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6), JVT-J042, 10th Meeting: Waikoloa, Hawaii, USA, 8-12 December, 2003.

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# ANNEX – Generation of the test sequences with re-phased chroma samples

## A1.	Sequences generated from 4:2:0 content

This first set of sequences corresponds to the JCTVC-F900 [2] test set. Chroma samples are re-phased using DCTIF filter specified in the HM. For instance, for (Cx=1, Cy=0), a displacement of (1/4, -1/4) is applied to the chroma pictures.

Figure 3: illustration of the 4:2:0 chroma re-phasing at same resolution.

## A2.	Sequences generated from 4:4:4 content

The second type of sequences is issued from four 4:4:4 1080p sequences used in the exploration work to define the Fidelity Range Extensions Amendment of H.264/AVC, as described in JVT-J042 [3].

The following sequences have been considered:

- Rolling_Tomatoes,

- Man_In_Restaurant,

- Card_Toss,

- Dinner_Table.

Each sequence is made of 60 frames. Originally, these sequences are in RGB format. They are first converted to YUV using the following transformation:

Y =  0.299R + 0.587G + 0.114B

U = -0.147R - 0.289G + 0.436B

V =  0.615R - 0.515G - 0.100B

For each 4:4:4 sequence, three 4:2:0 sequences have been generated, corresponding to the three chroma locations (Cx=0, Cy=0), (Cx=1, Cy=0), (Cx=1, Cy=1) (see conventions of Figure 1). The fourth possible chroma location corresponds to the default HM configuration.

The chroma downsampling is achieved using a modified version of the downsampling tool of the JSVM (the corresponding filter is given in section A.4).

Figure 4: schematic illustration of the 4:4:4 to 4:2:0 conversion.

The generic chroma signal downsampling equation is as follows:

Cdown[ x, y ] = Cref[ x + Cx/2 , y + Cy/2 ]

with (Cx,Cy) being the horizontal and vertical chroma phases (using conventions of Figure 1).

## A3.	Sequences generated from 4:2:0 content with downsampling

The third type corresponds to sequences generated from the JCTVC test set, as defined in JCTVC-F900. These 4:2:0 sequences are supposed to have a nominal chroma phase (Cx=0, Cy=1). A downsampling is performed on each component, the chroma components being interpolated with the right (input) chroma phases. Again three different input chroma phases are used, resulting in three subsampled sequences per source sequence.

Figure 5: schematic illustration of the 4:2:0 to chroma-rephased 4:2:0 conversion with downsampling.

The generic chroma signal downsampling equation is as follows:

Cdown[ x, y ] = Cref[ x + Cx/2 – Cin_x/4 + 1/4 , y + Cy/2 – Cin_y/4 + 1/4  ]

with (Cx,Cy) being the horizontal and vertical chroma phases in the output frame, (Cin_x,Cin_y) being the horizontal and vertical chroma phases in the input frame. The chroma downsampling is achieved using a modified version of the JSVM downsampling tool (more explanations are given in annex).

## A.4		Downsampling filter used to generate chroma re-phased 4:2:0 sequences

These filters are specified in the downsampling tool of the reference software of the SVC extension of the MPEG-4 AVC/H.264 standard (JSVM). They are use in case of downsampling with a ratio of 2.

const int filter16[16][12] =

{

{   0,  -4,  -7,  11,  38,  52,  38,  11,  -7,  -4,   0,   0 },

{   0,  -4,  -7,   9,  37,  51,  40,  13,  -6,  -7,   2,   0 },

{   0,  -3,  -7,   8,  35,  51,  41,  14,  -5,  -7,   1,   0 },

{   0,  -2,  -8,   6,  33,  51,  42,  16,  -5,  -7,   2,   0 },

{   0,  -2,  -8,   5,  32,  50,  43,  18,  -4,  -8,   2,   0 },

{   0,  -2,  -8,   4,  30,  50,  45,  19,  -3,  -8,   1,   0 },

{   0,  -1,  -8,   2,  28,  49,  46,  21,  -2,  -8,   1,   0 },

{   0,  -1,  -8,   1,  26,  49,  47,  23,  -1,  -8,   0,   0 },

{   0,   0,  -8,   0,  24,  48,  48,  24,   0,  -8,   0,   0 },

{   0,   0,  -8,  -1,  23,  47,  49,  26,   1,  -8,  -1,   0 },

{   0,   1,  -8,  -2,  21,  46,  49,  28,   2,  -8,  -1,   0 },

{   0,   1,  -8,  -3,  19,  45,  50,  30,   4,  -8,  -2,   0 },

{   0,   2,  -8,  -4,  18,  43,  50,  32,   5,  -8,  -2,   0 },

{   0,   2,  -7,  -5,  16,  42,  51,  33,   6,  -8,  -2,   0 },

{   0,   1,  -7,  -5,  14,  41,  51,  35,   8,  -7,  -3,   0 },

{   0,   2,  -7,  -6,  13,  40,  51,  37,   9,  -7,  -4,   0 }

};