Working draft for Experiment I

#### Sample adaptive offset parameter syntax

| sao_offset_param( rx, ry, saoDepth , cIdx ) { | Descriptor |
| --- | --- |
| if( sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] ) { |  |
| sao_offset_param( 2*rx + 0, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 0, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| } else { |  |
| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | ue(v) ¦ ae(v) |
| if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] > 5 && sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] !=12) { |  |
| sao_band_center[ cIdx ][ saoDepth ][ x0 ][ y0 ] + 15 | se(v) ¦ ae(v) |
| } |  |
| if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ) |  |
| sao_offset[ cIdx ][ saoDepth ][ x0 ][ y0 ][ i ] | se(v) ¦ ae(v) |
| } |  |
| } |  |

#### 7.4.3.4 Sample adaptive offset parameter semantics

sample_adaptive_offset_flag specifies whether sample adaptive offset applies or not for the current picture.

sao_flag_cb equal to 1 denotes sample adaptive offset process for Cb shall be applied to the current picture.

sao_flag_cr equal to 1 denotes sample adaptive offset process for Cr shall be applied to the current picture.

sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] specifies whether a region is split into four sub regions with half horizontal and vertical number of LCU for the color component cIdx. The array indices rx and ry specify the region index and saoDepth specifies the split depth of the region. When sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] is not present, it shall be inferred to be equal to 0.

The maximum allowed depth for sample adaptive offset process SaoMaxDepth is derived as follows:

SaoMaxDepth = Min( 4, Min( Floor( Log2( PicWidthInLCUs ) ), Floor( Log2( PicHeightInLCUs ) ) ) )	(7-1)

where

PicWidthInLCUs = Ceil( PicWidthInSamplesL ÷ ( 1 << Log2MaxCUSize ) )	(7-2)
PicHeightInLCUs = Ceil( PicHeightInSamplesL ÷ ( 1 << Log2MaxCUSize ) )	(7-3)

[Ed.: (WJ) PicWidthInLCUs and PicHeightInLCUs could be defined elsewhere]

sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] indicates the offset type for the color component cIdx of the region specified by saoDepth, rx and ry.

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] indicates the offset value of i-th category for the color component cIdx of the region specified by saoDepth, rx and ry.

sao_band_center[ cIdx ][ saoDepth ][ rx ][ ry ] indicates the center of the interval when sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 6, 7, 8, 9, 10, 11, 13, 14.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

The offset value shall be in the range of [ -( 1 << ( SaoBitsRange – 1) ), ( 1<< ( SaoBitsRange – 1) ) – 1 ] where

SaoBitRange = Min( bitDepth, 10 ) – 4		(7-4)

An array SaoOffsetVal is specified as

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 0 ] = 0		(7-5)

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i + 1 ] =

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

with i = 0..NumSaoCategory – 1		(7-6)

The number of categories, NumSaoClass, is specified in Table 7-1.

Table 7-1 – Specification of NumSaoClass

| - sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | - NumSaoCategory | - Edge type (informative) |
| --- | --- | --- |
| - 0 | - 0 | - Not applied |
| - 1 | - 4 | - 1D 0-degree edge |
| - 2 | - 4 | - 1D 90-degree edge |
| - 3 | - 4 | - 1D 135-degree edge |
| - 4 | - 4 | - 1D 45-degree edge |
| - 5 | - 16 | - Side band |
| - 6 | - 2 | - Band: 1/16th of range, 5 most significant bits |
| - 7 | - 4 | - Band: 1/8th of range, 5 most significant bits |
| - 8 | - 4 | - Band: 1/16th of range, 6 most significant bits |
| - 9 | - 8 | - Band: 1/4th of range, 5 most significant bits |
| - 10 | - 8 | - Band: 1/8th of range, 6 most significant bits |
| - 11 | - 8 | - Band: 1/4th of range, 7 most significant bits |
| - 612 | - 16 | - Central band |
| - 13 | - 16 | - Band: 1/4th of range, 6 most significant bits |
| - 14 | - 16 | - Band: 1/8th of range, 7 most significant bits |

##### 7.4.3.4 Modification process for luma and chroma samples

Inputs to this process are:

–	a variable cIdx specifying chroma component index,

–	a sample position ( xC, yC ),

–	a pair of variables ( rx, ry ) specifying the region index,

–	a variable saoDepth specifying the split depth of the region,

–	an array saoValueArray specifying offset values,

–	a variable NumSaoCategory specifying the number of categories,

–	a variable bitDepth specifying the bit deph of current component,

–	an array sao_band_center specifying center of band.

–	a block size nS.

Output of this process is a modified picture buffer for the chroma component cIdx.

Let recSaoPicture represents the processed sample array of the current picture of chroma component cIdx and saoTypeIdx is set equal to sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ].

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC.

Depending on the value of saoTypeIdx, the following applies:

–	If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-2.

- A variable edgeIdx is specified as

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-7)

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ edgeTable[ edgeIdx ] ]	(8-8)

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

–	Otherwise, if saoTypeIdx is equal to one of the values of 5 or 612, the following ordered steps applies:

- A variable bandShift is set equal to BitDepthY – 5 if cIdx is equal to 0, otherwise, set equal to BitDepthC – 5.

- A variable bandType set equal to 0 if saoTypeIdx is equal to 12 otherwise bandType set equal  to 1.

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

saoValueArray[ bandTable[ saoTypeIdx – 5 bandType][ bandIdx ] ]	(8-9)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Table 8-3.

–	Otherwise, if saoTypeIdx is equal to one of the values of 6, 7, 8, 9, 10, 11, 13 or 14, the following ordered steps applies:

- A variable bandShift is set equal to BitDepth – bandShiftTable[ saoTypeIdx – 6 ]. bandShiftTable is specified in Table 8.18.

- A variable SaoBandCenter is set equal to (sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] << ( bandShift – 5 )).

- A variable MinSaoBand is set equal to ( SaoBandCenter ) – (NumSaoClass >> 1 ) + 1

- A variable MaxSaoBand is set equal to ( SaoBandCenter ) + ( NumSaoClass >> 1 )

- A variable bandTable is modified as

bandTable[i] = 0 with i =0..MinSaoBand-1

bandTable[i] = i- MinSaoBand with i = MinSaoBand..MaxSaoBand

bandTable[i] = 0 with i = MaxSaoBand+1..( 1 << bandShift ) - 1

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ bandTable[ bandIdx ] ]	(8-10)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift )

–	Otherwise (sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 0), the following applies:

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] with i = 0..nS-1 and j = 0..nS-1	(8-11)

[Ed. (WJ): copy operation is necessary to use recSaoPicture later.]

Table 8-2 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | 1 |
| vPos[1] | 0 | 1 | 1 | -1 |

Table 8-3 – Specification of array bandTable according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

Table 8-18 – Specification of array bandShiftTable according to saoTypeIdx – 6

| - saoTypeIdx – 6 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - bandShiftTable [saoTypeIdx – 6 ] | - 5 | - 5 | - 6 | - 5 | - 6 | - 7 | - 5 | - 6 | - 7 |