| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G246 |
| --- | --- |

| Title: | On additional SAO Band Offset classifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume LAROCHE Tangi POIRIER Patrice ONNO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution presents a modification of sample adaptive band offsets. The proposed modification consists in adding 8 classifications of band offsets. The additional classifications are based on the first group of sample adaptive band offsets. These additional classifications are the combination of 3 interval sizes and 3 sizes of class. Moreover, the center of the proposed intervals in the intensity range is coded. In the experimental results, two configurations are presented with respectively 6 and 8 additional classes. The proposed modification gives an average BDR of -0.4% for PSNRAV, -0.1% for Luma component, -1.7% for Chroma U and -2.0% for Chroma V with a small impact on the encoding runtime and with complexity reduction of the decoding runtime.

# Introduction

The current HEVC classification uses 2 kinds of classifications: the Edge Offset (EO) and the Band Offset (BO). For Edge Offset, 4 types are defined. For Band Offset, 2 types are defined, corresponding to two groups of pixel values as depicted in Figure 1 (cf. JCTVC-D122 [1]). The intensity range is divided into 32 ranges. Each SAO Band Offset group uses half of the intensity range. The first group is the central range of pixel values containing 16 classes (ranges) and the second group is composed of the 16 classes corresponding to the extremities of the whole intensity interval.

Figure 1 Band Offset classification of current HM4.0.

This classification is useful when the signal of a frame area covers all the intensity of pixel values. But when the signal is compacted to a restricted range of pixel values and when this range is not centered on the middle of the intensity interval, this classification is not adapted.

In this contribution, additional SAO Band Offset types are proposed in order to be more adaptive to the pixel values repartition. These 8 additional types of Band Offsets are added to the current ones. Moreover, in order to well match the pixel values repartition in the intensity range, the center of these new classifications is transmitted into the bitstream.

# Technical description

## Additional group sizes

3 additional intensity subdivision ranges are added to the current HEVC design which considers half the intensity range for each group. Figure 2 depicts these 3 subdivisions in addition to the current first group. The additional subdivisions B0_NB_8, B0_NB_4, B0_NB_2 contain respectively 1/4th, 1/8th and 1/16th of the full intensity range.

| BO_0 |  |
| --- | --- |
| BO_NB_8 |  |
| BO_NB_4 |  |
| BO_NB_2 |  |

Figure 2: Band Offset first group of HEVC and 3 additional proposed sizes.

## Additional class sizes

The current HEVC design uses only one size for classes. For an 8 bits depth, these classes contain 8 pixel values. In this proposal, we suggest 2 additional subdivisions in order to divide the current class size by 2 or 4. These 3 subdivisions (1, 2, and 4) are used for each of the three additional group sizes. Figure 3 represents the 8 additional SAO types added to the current ones.

| BO_NB_8_SUB_1 |  |
| --- | --- |
| BO_NB_8_SUB_2 |  |
| BO_NB_4_SUB_1 |  |
| BO_NB_4_SUB_2 |  |
| BO_NB_4_SUB_4 |  |
| BO_NB_2_SUB_1 |  |
| BO_NB_2_SUB_2 |  |
| BO_NB_2_SUB_4 |  |

Figure 3 Proposed additional SAO band offset classifications.

As depicted in Figure 3, the group size of 1/4th of the intensity range and with a subdivision of 4 is not considered as a possible combination. Indeed, this combination needs 32 offsets and it therefore increases the memory usage dedicated to the offset storage in the worst case. Consequently, the additional proposed classifications need at most 16 offset values.

## Center of additional sizes

For the proposed additional Band Offset groups, the center of each group is transmitted. This center is coded by considering a subdivision of the intensity interval into 32 ranges. The center of an additional group is predicted by the center of the intensity range and this residual is then coded with a unary code. For example, the value of this center is between -15 and +15 for a subdivision of 1/16th of the intensity range (BO_NB_2).

## SAO type coding

The current HM4.0 design codes the SAO type according to Table 1 below:

Table 1 SAO offset type for HM4.0

| Sao_type_idx | offset type |
| --- | --- |
| 1 | EO_1 |
| 2 | EO_2 |
| 3 | EO_3 |
| 4 | EO_4 |
| 5 | BO_0 |
| 6 | BO_1 |

For the proposed modification, the index of Edge Offset types is not changed. The first group of Band Offset 1 BO_1 is inserted at the 5th position (instead of the 6th in HM4.0). The 9 derivations of the first group are added at the end of the list and ordered according to the number of offsets which need to be transmitted.

Table 2 SAO offset type for the proposed scheme

| Sao_type_idx | offset types | Number of offsets |
| --- | --- | --- |
| 1 | EO_1 | 4 |
| 2 | EO_2 | 4 |
| 3 | EO_3 | 4 |
| 4 | EO_4 | 4 |
| 5 | BO_1 | 16 |
| 6 | BO_NB_2_SUB_1 | 2 |
| 7 | BO_NB_4_SUB_1 | 4 |
| 8 | BO_NB_2_SUB_2 | 4 |
| 9 | BO_NB_8_SUB_1 | 8 |
| 10 | BO_NB_4_SUB_2 | 8 |
| 11 | BO_NB_2_SUB_4 | 8 |
| 12 | BO_0 | 16 |
| 13 | BO_NB_8_SUB_2 | 16 |
| 14 | BO_NB_4_SUB_4 | 16 |

## Encoding selection

All these 8 additional classifications are RD-evaluated with the same rate-distortion algorithm as implemented in HM4.0. In the same way, the centers of these additional groups are determined with this algorithm. The RD selection of SAO in HM4.0 is very efficient in terms of complexity because it corresponds to adding distortion and offset rates of each range to determine the RD cost of each center position of a new group. Please note that in the current implementation, the RD cost of each new group is determinate independently. These processes should be combined in order to reduce the number of RD cost evaluation and statistics accumulation. Theoretically, this combination could divide by about 3 the number of operation for the RD selection.

Moreover, we have added an additional RD test for each offset predictor in order to set to zero the offset that would not improve the corresponding class. The current HM4.0 RD selection of SAO offsets does not take into account the fact that an offset for a class does not improve the pixel values of this class in the RD sense. So we have added a RD test to check if the RD cost of the zero offset is lower than the RD cost of the offset determined for the current class. This RD selection has a small impact on the coding efficiency on the HM4.0 anchor. As reported in Excel file Results_HM4.0_New_RDO_for_SAO.xls, in average for the 8 configurations, doesn’t improve the gain for luma component and improve the chroma component U and V of respectively 0.2% and 0.3%. This RD optimization increases the encoding time of 1% for all configurations.

# Comments on complexity

The 2 additional subdivisions of classes have an impact on the complexity of the Band Offset classification. Indeed, the 7 most significant bits need to be checked instead of 5 in the current HM4.0. Yet, these additional 2 bits checking for hardware implementation does not affect the worst case complexity of the SAO loop filtering. Indeed, the worst case complexity is reached with the Edge Offset classification.

In the following section, we provide the results of 2 configurations. In the first one, 7 bits need to be checked at most for Band Offset classification and in the second one, only 6 bits are checked.

# Experimental Results

The experiments were conducted in accordance with the 8 configurations using the common conditions [2]. In this section, we proposed 2 experiments. The first one uses the 8 additional proposed classifications and needs 7 bits checking for the SAO Band Offset classification. The second one uses only 6 additional classifications. In this second experiment the subdivision of 4 of the current classification is removed. Consequently only 6 bits need to be checked for the SAO Band Offset classification.

Table 3 gives the results of experiment I compared to the 8 configurations of the HM4.0 anchors. In average, for Luma component, the proposed scheme offers 0.1% BDR gain and reaches 0.2% for LDLC configuration. In average, for Chroma U and V components, the proposed scheme gives respectively 1.7% and 2.0% BDR gains. The best configuration is obtained for LDLC with respectively 2.3% and 2.7%. For both experiments, the additional classifications are more efficient for Chroma components. Indeed, the pixel values repartitions are more compact for Chroma components than for Luma. So, the smaller range sizes and their subdivisions are more efficient for this kind of repartition.

Moreover, as proposed on the JCTVC reflector, we have computed in column “YUV” the PSNRAV (PSNRAV = (6*PSNR_Y + PSNR_U + PSNR_V) / 8). The proposed scheme demonstrates 0.4% BDR gain in average for the 8 configurations and reaches 0.6% BDR gain on YUV for LDLC configuration. The encoding runtime is only 101% in average. Please note that an encoding time increase is also observed with the proposed RD optimization on the HM4.0 Anchor. The decoding run time of Experiment I  is reduced by 1%. This decoding complexity reduction comes from the reduced number of classes and an increase of the selection of SAO Band Offset groups which decreases the number of SAO Edges Offset selection. Indeed the SAO Band Offset method is less complex than the Edge Offset.

Table 3 Simulation results of Experiment I (8 additional groups) of the proposed scheme compared to the HM4.0 anchors.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.1% | -0.9% | -1.4% | -0.2% | -0.1% | -0.3% | -0.5% | -0.1% |
| Class B | 0.1% | -1.5% | -2.0% | -0.2% | 0.0% | -1.0% | -1.4% | -0.2% |
| Class C | 0.1% | -0.9% | -1.2% | -0.1% | 0.1% | -0.7% | -0.9% | -0.1% |
| Class D | 0.0% | -0.6% | -0.9% | -0.1% | 0.0% | -0.6% | -0.7% | -0.1% |
| Class E | 0.2% | -2.9% | -3.0% | -0.4% | 0.0% | -1.0% | -1.0% | -0.1% |
| Overall | 0.0% | -1.3% | -1.6% | -0.2% | 0.0% | -0.7% | -0.9% | -0.1% |
|  | 0.0% | -1.3% | -1.6% | -0.2% | 0.0% | -0.8% | -0.9% | -0.1% |
| Enc Time[%] | 101% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | 0.0% | -1.9% | -2.2% | -0.3% | 0.0% | -0.8% | -0.9% | -0.1% |
| Class B | 0.0% | -1.9% | -2.2% | -0.4% | -0.1% | -1.8% | -1.9% | -0.4% |
| Class C | 0.0% | -1.1% | -1.1% | -0.2% | -0.1% | -1.4% | -1.4% | -0.3% |
| Class D | 0.0% | -0.9% | -1.1% | -0.2% | -0.1% | -1.2% | -1.6% | -0.3% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -1.5% | -1.7% | -0.3% | -0.1% | -1.4% | -1.5% | -0.3% |
|  | 0.0% | -1.5% | -1.7% | -0.3% | -0.1% | -1.3% | -1.5% | -0.3% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -2.5% | -3.2% | -0.6% | -0.2% | -2.9% | -2.9% | -0.6% |
| Class C | -0.1% | -1.1% | -1.7% | -0.3% | -0.2% | -2.0% | -2.4% | -0.5% |
| Class D | -0.1% | -0.6% | -1.2% | -0.2% | -0.1% | -1.3% | -1.9% | -0.3% |
| Class E | -0.2% | -4.1% | -3.2% | -0.8% | -0.4% | -2.8% | -4.0% | -0.9% |
| Overall | -0.1% | -2.0% | -2.3% | -0.5% | -0.2% | -2.3% | -2.7% | -0.6% |
|  | -0.1% | -2.0% | -2.4% | -0.5% | -0.2% | -2.3% | -2.7% | -0.6% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 97% |  |  |  |
|  | Low delay P HE |  |  |  | Low delay P LC |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -2.2% | -3.0% | -0.5% | -0.1% | -2.7% | -2.8% | -0.5% |
| Class C | 0.0% | -1.2% | -1.7% | -0.3% | -0.1% | -2.0% | -2.0% | -0.5% |
| Class D | 0.0% | -1.2% | -1.9% | -0.3% | 0.0% | -1.4% | -2.2% | -0.3% |
| Class E | -0.2% | -4.6% | -3.7% | -0.9% | -0.3% | -2.7% | -3.7% | -0.8% |
| Overall | -0.1% | -2.1% | -2.5% | -0.5% | -0.1% | -2.2% | -2.6% | -0.5% |
|  | -0.1% | -2.2% | -2.6% | -0.5% | -0.1% | -2.2% | -2.6% | -0.5% |
| Enc Time[%] | 102% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 97% |  |  |  |

Table 4 reports the results of experiment II compared to all configurations of the HM4.0 anchors. In average, the same BDR gain is obtained for Luma component (0.1%). But for U and V Chroma components, experiment II gives respectively 1.4% and 1.5% BDR gains instead of respectively 1.7% and 2.0% for experiment I. Experiment II obtained 0.3% BDR gain on YUV in average for the 8 configurations. The encoding runtime is only 101% in average and the decoding runtime is below 100%. In this experiment II, the selection of the SAO Band Offset increases, but this time, it does not compensate the SAO Edge Offset selection. So the decoding time decrease is not so significant.

Table 4 Simulation results of Experiment II (6 additional groups) of the proposed scheme compared to the HM4.0 anchors.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.1% | -0.7% | -1.0% | -0.2% | -0.1% | -0.3% | -0.3% | -0.1% |
| Class B | 0.1% | -1.2% | -1.6% | -0.2% | 0.0% | -0.7% | -0.9% | -0.1% |
| Class C | 0.1% | -0.8% | -1.1% | -0.1% | 0.0% | -0.6% | -0.7% | -0.1% |
| Class D | 0.0% | -0.4% | -0.6% | -0.1% | 0.0% | -0.4% | -0.5% | -0.1% |
| Class E | 0.1% | -2.1% | -2.3% | -0.3% | 0.0% | -0.5% | -0.6% | -0.1% |
| Overall | 0.0% | -1.0% | -1.3% | -0.2% | 0.0% | -0.5% | -0.6% | -0.1% |
|  | 0.0% | -1.0% | -1.3% | -0.2% | 0.0% | -0.5% | -0.7% | -0.1% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.1% | -1.4% | -1.7% | -0.2% | 0.0% | -0.5% | -0.5% | -0.1% |
| Class B | 0.0% | -1.5% | -1.7% | -0.3% | -0.1% | -1.3% | -1.3% | -0.3% |
| Class C | 0.0% | -1.0% | -1.0% | -0.2% | -0.1% | -1.3% | -1.1% | -0.3% |
| Class D | 0.0% | -0.7% | -0.9% | -0.1% | 0.0% | -0.9% | -1.3% | -0.2% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -1.2% | -1.4% | -0.2% | -0.1% | -1.0% | -1.1% | -0.2% |
|  | 0.0% | -1.2% | -1.4% | -0.2% | -0.1% | -1.0% | -1.1% | -0.2% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -1.8% | -2.3% | -0.4% | -0.2% | -2.2% | -1.9% | -0.5% |
| Class C | -0.1% | -1.3% | -1.5% | -0.3% | -0.2% | -1.9% | -2.2% | -0.5% |
| Class D | -0.1% | -0.5% | -0.9% | -0.2% | -0.1% | -1.1% | -1.9% | -0.3% |
| Class E | -0.2% | -3.7% | -2.4% | -0.7% | -0.3% | -1.5% | -2.7% | -0.6% |
| Overall | -0.1% | -1.7% | -1.8% | -0.4% | -0.2% | -1.7% | -2.1% | -0.5% |
|  | -0.1% | -1.7% | -1.8% | -0.4% | -0.2% | -1.8% | -2.1% | -0.5% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 97% |  |  |  |
|  | Low delay P HE |  |  |  | Low delay P LC |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -1.8% | -2.5% | -0.4% | -0.1% | -1.9% | -2.1% | -0.4% |
| Class C | -0.1% | -1.1% | -1.6% | -0.3% | -0.1% | -2.0% | -1.7% | -0.4% |
| Class D | 0.0% | -1.0% | -1.5% | -0.2% | 0.0% | -1.3% | -1.4% | -0.2% |
| Class E | -0.2% | -3.9% | -3.2% | -0.8% | -0.2% | -2.7% | -2.7% | -0.6% |
| Overall | -0.1% | -1.8% | -2.2% | -0.4% | -0.1% | -1.9% | -1.9% | -0.4% |
|  | -0.1% | -1.8% | -2.2% | -0.4% | -0.1% | -1.9% | -2.0% | -0.4% |
| Enc Time[%] | 102% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 101% |  |  |  |

As a conclusion of these results, experiment I gives the best results with in average for the 8 testing configurations -0.1% BDR gain compared to experiment II. As mentioned in the previous section, the additional bits checking do not affect the worst case complexity of the SAO classification (the Edge Offset classification has a higher complexity than the SAO Band Offset). Consequently, experiment I is the best configuration.

# Conclusion and recommendations

This contribution presents a modification of the Band Offset in SAO. The proposed modification adds 8 Band Offset classifications to the 2 current HM4.0 groups. The additional groups are the combinations of smaller intervals and additional subdivisions of classes. Moreover, the center of the new interval is coded. The aim of these groups is to better match the pixel values repartition especially for Chroma components where the repartition is restricted to a reduced range of values. The average BDR gain computed by using the PSNRAV is 0.4% for the 8 configurations of the HM4.0 anchor. The gain is higher on Chroma components with around 2% gain. In addition, we observed a higher selection of these additional groups of Band Offset which leads to a decrease of the decoding runtime by 1%.

# Reference

- C.-M. Fu, C.-Y. Chen, Y.-W. Huang, and S. Lei, “TE8 Subtest3: Picture Quadtree-based Adaptive Offset”, Joint Collaborative Team on Video Coding, JCTVC-D122, Jan. 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-F900, 6th Meeting: Torino, Italy, July, 2011.

# Working draft

The proposed modifications are reported in JCTVC-G246-WorkingDraft.docx.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).