| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G247 |
| --- | --- |

| Title: | CAVLC counters normalization per LCU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Stephane PAUTET Christophe GISQUET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr christophe.gisquet@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This proposal relates to counter-based adaptation for CAVLC. In current design, a counter sum is used to control the counters normalization. Once this sum reaches 16, right-shifting is applied to each counter and the sum is reset to 0. In this proposal, counter renormalizing only takes place after decoding/encoding one LCU, according to the value of the first counter value. Counter sum is no more used. In addition, counters are stored on 6 bits. The impact on coding efficiency is reportedly negligible for AI, RA and LD configurations.

# Introduction

Counter-based adaptation for CAVLC consists of adding counters to some decoding tables to prevent too frequent swapping of their table entries. In order to prevent counters from overflowing, the occurrence of each codeword is counted. The counters are renormalized (right-shifting applied to each counter) each time a counter sum reaches the value of 16. Hence, for a syntax element, its counters are renormalized each 16 coding/decoding.

This process is depicted in the following figure.

Figure 1: HM4.0 counter updating and renormalizing.

# Proposal description

In this proposal, the renormalization process is applied once per K LCU. The counter sum is no more used. Counter renormalization is only applied when the first counter value reaches a pre-defined threshold . In addition, counters are limited within a range 2N, N being the number of bits used to store these counters. In practical software implementation, this counter bit-depth limitation to N bits is simulated by applying the following counter updating:

cnt = (cnt+1) % 2N

where % is the modulo operation, which can usually be implemented with a logical and operation with 2N-1.

The proposed solution avoids the use of counter sum.

The following figure depicts the proposed counter updating and renormalizing process.

Figure 2: proposed counter updating and renormalizing.

K can be adapted depending on the LCU size (e.g. K=1 for 64x64 LCU, 4 for 32x32 LCU …).

# Results

Test conditions specified in JCTVC-F900 have been used. HM4.0 has been used as reference. Several configurations have been tested, corresponding to different settings for the LCU-based renormalization frequency K, the counter renormalization threshold , the number of bits to store counters N. The results are mainly sensitive to N and K. For N lower than 6, or K greater than 4, noticeable losses can be observed. Parameter  has a small influence.

The following tables summarize the results obtained for different configurations.

Table 1. Results for different K values,  = 15, N=8 bits

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| each K=1 LCU | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| each K=2 LCUs | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| each K=4 LCUs | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% |
| each K=8 LCUs | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.3% | 0.0% |

Table 2. Results for different N values,  = 15, K=1

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Counter bit-depth N=8 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Counter bit-depth N=7 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Counter bit-depth N=6 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Counter bit-depth N=5 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% |
| Counter bit-depth N=4 | 0.5% | 0.3% | 0.3% | Not tested |  |  |  |  |  |

A suggested configuration is defined as follows:

- Renormalization each 1 LCU

- Threshold  to perform normalization

- Number of bits to store counters 6

For this configuration, the following results are obtained (see also attached xls file). Coding impact is negligible in the 3 configurations AI, RA and LD.

Table 3. Results for  = 32, K=1, N=6 bits

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | 0.0% | 0.1% |  |  |  | -0.2% | 0.3% | -0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |

# References

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, Italy, July, 2011.

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).