| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G248 |
| --- | --- |

| Title: | Low Complexity Scalable Extension of HEVC intra pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S.Lasserre, F.Le Léannec, E.Nassor Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876877 {sebastien.lasserre,fabrice.leleannec,eric.nassor}@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution presents a new approach for scalable extension of HEVC INTRA pictures. This scalable INTRA codec design targets coding efficiency together with very low complexity. Spatial random access and a high degree of parallelism are two additional targeted features. The proposed scalable INTRA codec employs only one coding mode, which is inter-layer intra prediction, which provides low complexity. Coding efficiency is obtained through statistical modeling of DCT channels to encode, rate distortion optimal quantifiers that are pre-computed off-line, coupled with a distortion allocation process between DCT channels. Overall, bitrate increase of 12.8% is obtained relative to HEVC single layer coding on tested sequences in dyadic spatial scalability mode. Finally, non-contextual, non-adaptive entropy coding provides the spatial random access feature.

# Introduction

## Low complexity

This contribution proposes a scalable extension of HEVC for INTRA pictures. Some already proposed scalable extensions perform an HEVC-like encoding of enhancement pictures, with the introduction of some inter-layer prediction tools [][]. In these approaches, the enhancement layer coding process involves RDO choice between multiple intra predictions modes, where inter-layer prediction modes are added to modes already present in HEVC. This, in addition to the base layer coding process, results in an encoder complexity that is higher than that of non-scalable HEVC encoding.

On the contrary, the main idea behind this contribution is that a successful scalable standard should specify a scalable codec that provides low complexity together with good coding efficiency. Scalability should be used as a means of reducing HEVC complexity. Such feature is particularly useful for embedded real time UHD video encoders in handset devices, where complexity and memory bandwidth are limited.

Practically, it is desired to design a scalable video coding system, with an overall complexity that is closer to the complexity of the dyadic base layer than that of single-layer encoding. This complexity requirement is illustrated by Figure 1.

Figure 1: Complexity target for the proposed scalable video coding method

To this end, in this contribution, the enhancement layer encoding employs only one INTRA prediction mode. This single coding mode corresponds to inter-layer intra prediction, also known as I_BL in the former scalable extension of H.264/AVC [].

As a consequence, no coding mode decision is required in the enhancement layer, which strongly reduces the encoder complexity, compared to standard video coding schemes, where a number of directions for spatial prediction are being evaluated.

## Parallelism and spatial random access

An additional requirement for the proposed scalable approach is to provide spatial random access feature in the compressed video bit-stream, as well as a high degree of parallelism in the decoding and encoding processes. These two features are obtained through the removal of any dependency between successive spatial entities in the picture. Therefore, in addition to the suppression spatial directional prediction, non-contextual and non-adaptive entropy coding/decoding is required.

The scalable video coding approach introduced in section 2 conforms to the requirements listed above.

# Technical description of the proposal

This section describes the main principles of this contribution for low complexity scalable video coding. First an overview of the enhancement INTRA picture coding and decoding are provided in sections 2.1. Next, more details are given in section 2.2.

## Enhancement INTRA picture codec overview

Figure 2 shows the overall architecture of the proposed INTRA scalable encoder. It consists in the following. The input full resolution original picture is down-sampled to the base layer resolution level, and is encoded with HEVC. Then the base layer picture is reconstructed and up-sampled to the top layer resolution in case of spatial scalability. The difference with the original picture constitutes the spatial residual picture.

For down sampling, H.264/SVC down-sampling filters are used and for up sampling, the DCTIF interpolation filters of quarter-pel motion compensation in HEVC are used.

The resulting residual picture is then encoded according to the new low complexity approach using DCT and quantization described in section 2.2.  The resulting coded enhancement layer consists in coded residual data as well as some parameters used to model DCT channels of the residual picture.

As can be seen, this global architecture corresponds to classical scalable INTRA coding, where the spatial intra prediction and coding mode decision steps have been removed.

Figure 2: overview of the enhancement INTRA picture coding process

Figure 3 depicts the overall architecture of the proposed scalable INTRA video decoder. The input bit-stream to that decoder consists in the HEVC-coded base layer, enhancement residual coded data, and parameters of the DCT channels in the enhancement residual picture. First, the base layer is being decoded, which provides a reconstructed base picture. The reconstructed base picture is up-sampled to the enhancement layer resolution. Then, the enhancement layer is decoded as follows. The residual data decoding process described in section 2.2 is invoked, which provides successive de-quantized DCT blocks. These DCT blocks are then inverse transformed and added to their co-located up-sampled block. The so-reconstructed enhancement picture finally undergoes HEVC low-complexity post-filtering processes, i.e. de-blocking filter and sample adaptive offset.

Figure 3: overview of the enhancement INTRA picture decoding process

## Residual data coding/decoding process

This section explains the main principles of the residual texture coding/decoding performed in the enhancement layer.

The residual coding process is illustrated by Figure 4. The input to the encoder consists in an INTRA residual picture. A DCT transform is applied, which provides a frame of DCT blocks noted X on Figure 4.

The next coding step consists in computing a statistical model of DCT coefficients.  DCT coefficients are thus modeled by a Generalized Gaussian Distribution (GGD). Given its distribution, each DCT channel is assigned a quantifier. This non-uniform scalar quantifier is defined by a set of quantization intervals and associated de-quantized sample values. A pool of such quantifiers is available both on the encoder and on the decoder side. Various quantifiers are pre-computed off-line, through the Chou-Lookabaugh-Gray rate distortion optimization process [].

Figure 4: enhancement INTRA coding

The selection of the rate distortion optimal quantifier for a given DCT channel proceeds as follows. Given input coding parameters, a distortion target for considered DCT channel is determined. To do so, the distortion targets are balanced between various DCT channels.

Once the distortion target is obtained for each DCT channel, the right quantifier to use is chosen. As the rate distortion curve associated to each pre-computed quantifier is known (tabulated), this merely consists in choosing the quantifier that provides minimal bitrate for given distortion target.

Then DCT coefficients are quantized, and entropy coded. The entropy coder used is non-contextual and non-adaptive. It employs, for each DCT channel, a set of fixed probabilities, respectively associated to each pre-computed quantization interval. Therefore, these probabilities are entirely calculated off-line, together with the rate distortion optimal quantifiers. Probability values are never updated during the encoding or decoding processes, and are fixed for the whole picture being processed. In particular, this ensures the spatial random access feature, and also makes the decoding process highly parallelizable.

As a result of the proposed INTRA enhancement coding scheme, the enhancement layer bit-stream is made of the following syntax elements.

- Parameters of each coded DCT channel model. Two parameters are needed to fully specify a generalized Gaussian distribution. Therefore, two parameters are sent for each encoded DCT channel. These are sent only once for each picture.

- Coded residual data results from the entropy coding of quantized DCT coefficients.

Note that the above syntax elements represent the content of coded slice data in the scalable extension of HEVC. The NAL unit container of HEVC can be used to encapsulate a slice that is coded according to the proposed scheme.Figure 5 depicts the INTRA decoding process that corresponds to the coding process of Figure 4. The input to the decoder consists in the coded residual data and the parametric model of DCT channels, for current picture.

First, the same process as in the encoder determines the distortion target of each DCT channel, given the parametric model of each coded DCT channel. Then, the choice of optimal quantifiers for each DCT channel is performed exactly in the same way as on the encoder side. Given the chosen quantifiers, and thus probabilities of all quantized DCT symbols, the entropy decoder is able to decode the input coded residual data. This provides successive quantized DCT blocks, which are then inverse quantized and inverse transformed.

Figure 5: enhancement INTRA decoding

# Experiments

This section describes the testing conditions used to evaluate the proposed INTRA scalable coding method (section 3.1) and then the obtained rate distortion performance (section 3.2).

## Testing conditions

Two test sets were used to run the proposed INTRA scalable codec: one for spatial scalability and one for SNR scalability. Class A 8 bits and class B sequences were tested. Testing conditions were chosen as close as possible to those of []. Original class B sequences were cropped to size 1920x1072 in order to make the base layer a multiple of 8 pixels.

With respect to spatial scalability, 2 dyadic layers were encoded. The JSVM down-sampling process was used to generate the original base layer. The base layer was encoded with HM4.0 in Intra-Loco configuration. Three encoders were compared in the encoding of the enhancement layer.

- HM4.0 single layer. The original sequence was encoded with HM4.0 in non-scalable mode, with QP values equal to 22, 27, 32 and 37.

- HM4.0 simulcast. The HM4.0 was used to encode both base and enhancement layer, without any interaction between them. The obtained bit-rate corresponds to the sum of base and enhancement layer bitrates. For spatial scalability, the base layer QP was set to the top layer QP minus 2. Of course the obtained PSNRs are equal to those of single layer configuration.

- Low complexity INTRA codec. The proposed low complexity INTRA scalable codec was used to encode the enhancement layer, at a PSNR value that is close to the PSNR obtained with simulcast and single-layer configurations. The low complexity INTRA scalable codec is run in full mono-threaded mode. However, note that a parallelized multi-threaded version of it would provide exactly the same rate distortion results.

Regarding SNR scalability, the 3 above coding configurations were used in SNR scalability context. The base layer QP was set equal to the top layer QP plus 3. Next section explains the comparative BD-PSNR results obtained with each coding configuration listed above.

## Obtained performances

Table 1 lists the BD-PSNR results obtained in the conducted experiments for spatial dyadic scalability. Average bitrate increases, respectively relative to HM4.0 simulcast and HM4.0 single layer, are shown in the two right columns. As can be seen, an average bitrate increase of 10.8% and 13.6% is obtained respectively for class A and class B sequences, compared to HM4.0 single layer coding. An average bitrate saving of 27.6% and 20.9% is obtained over simulcast. Moreover, Table 1 also shows the encoding time obtained with the proposed approach. It can be seen that the overall encoding time (base + enhancement) obtained with the proposed scalable approach lies between that of the base layer only and that of HEVC single layer coding. Therefore, as previously illustrated by Figure 1, the proposed scalable approach allows reducing the complexity of HEVC non-scalable coding.

Table 1: obtained rate distortion performances with dyadic spatial scalability

Table 2 depicts the BD-PSNR results obtained in SNR scalability mode. It shows that the average rate distortion cost of scalability compared to single layer coding is 5.8% on class A sequences, and 5.4% on class B sequences.

As a conclusion to this section, the proposed INTRA scalable coding method shows rather good rate distortion performance, despite its very low complexity.

Table 2: obtained rate distortion performances with SNR scalability

# Conclusion and perspectives

As a conclusion to this contribution, it has been proved that good rate distortion performances can be obtained with a very low complexity INTRA scalable coding process.

The main characteristics of this low complexity INTRA scalable codec are a unique available prediction mode and statistical modeling of DCT channel to encode. Very low complexity is obtained through the off-line computing of all heavy processes, which are the design of rate distortion optimal quantifiers, rate distortion models associated to these optimal quantifiers, probabilities used in the entropy coder. All the processing involved on the encoder and the decoder side can easily be performed in real time.

In addition to very low complexity, the proposed scalable codec design provides spatial random access feature, as well as a high degree of parallelism. Moreover, backward compliance with an H.264/AVC base layer is also easily provided, since the scalable coding relies on reconstructed base layer pictures.

The obtained rate distortion performances show that high coding efficiency can be obtained with low complexity. Therefore, we strongly recommend that the JCTVC committee considers very low complexity requirements in the design of the scalable extension of HEVC. In particular, test conditions imposing limited encoder complexity are desired. As an example, scalability testing conditions in all I_BL coding mode should be considered.

# References

[] ITU-T Recommendation H.264, “Advanced Video Coding for Generic Audiovisual Services,” March, 2010.

[] D. Hong, W.Jang, J.Boyce, “Scalability support in HEVC”, JCTVC-F290, Torino, IT, July 2011.

[] H.M. Choi, J.Nam, D.Sim, “Scalable structures and inter-layer prediction for HEVC scalable extension”,  JCTVC-F096, Torino, IT, July 2011.

[] P. Chou, T. Lookabaugh, R. Gray, Entropy Constrained Vector Quantization, IEEE Trans. on acoustics, speech and signal processing, September, vol. 37(1), January 1989.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).