| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G250 |
| --- | --- |

| Title: | CE3: Cross-check of JCTVC-G391 on MC memory band-width reduction (tool8) from Sony and Nokia |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

The purpose of this document is to report cross-checking of CE3 tool8 (tap length reduction for small bi-prediction blocks) proposed by Sony and Nokia in JCTVC-G391. The verification task has been done successfully and the results match those presented in JCTVC-G391. The software implementation has also been checked and the consistency with the CE3 description has been verified. It is reported that the verification tests show a negligible coding efficiency impact of the proposed filters length reduction.

- Code verification

Proposal JCTVC-G391 consists of limiting the interpolation filter length for bi-prediction based on the PU size. Luma interpolation filter is limited to 4-tap in both horizontal and vertical directions for 8x4, 4x8 PUs. For 8x8 PUs, limitation only applies to vertical dimension. Regarding chroma, the horizontal and vertical 2-tap (linear) filter is used for 8x4, 4x8 PUs. For 8x8 PUs, horizontal filter is the usual HM 4-tap filter, and vertical filter is linear.

The source code has been analyzed. It has been verified that this implementation was in line with the CE3 tool8 description. The implementation is fully integrated in the HM4.0 code, without redundant code.

The source code was compiled successfully using windows32, VC2008 and using Linux 2.6.18-194.el5,  gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

# Results

Tests were done according to the tests description provided in [1]. Two tests were initially performed, corresponding to two different encoder settings. In a first test, the filters are changed both in the motion estimation and compensation process. In the second test, the change only applies to motion compensation.

An additional test has been done, with slightly different configuration: shorter tap filters are used only for bi-pred8x4 or bi-pred4x8 PUs.

The results fully match those provided by the proponent in these 3 cases.

The encoding and decoding runtime are slightly impacted, but the difference is within the runtime estimation statistical uncertainty. Test 1 seems to give slightly decrease encoding run time.

# Conclusion

The results presented by Sony and Nokia in JCTVC-G391 are confirmed. The code of the modification is clean and can be easily integrated in the next HM version.

The tap length reduction for small blocks in bi-prediction has a minor impact on coding efficiency, while reducing noticeably the worst case memory bandwidth needs.

# References

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, Italy, July, 2011.