| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G251 |
| --- | --- |

| Title: | CE12 Subtest4: Cross-verification of TI’s proposal JCTVC-G086 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2743 Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross verification results on JCTVC-G086 [1] proposed by TI. In the test, the coding performance and complexity was measured under common test conditions, which is defined in JCTVC-F900 [2]. Proposed tools have been evaluated on the common test condition. The detailed results are summarized in attached the excel sheet. In addition the subjective quality is evaluated on test sequences, QP and coding conditions selected in CE12. The source code was provided by TI to cross verify. It is confirmed that all results perfectly match with ones provided by TI.

# Introduction

TI’s proposal is included in CE12 which is related to improvement for deblocking filter. The BD-rate, the measurement of encoding/decoding time, and subjective evaluation is shown as below.

# Results

## Test condition

The experiment is executed with using HM-4.0 based on JCTVC-F912 [3], and the configuration and common test condition are as JCTVC-F900.

Both the encoding and the decoding time are measured on the same machine as anchor, and the decoding time is measured without output of decoded YUV data.

## BD-rate and encoding/decoding time

The experimental results are summarized in the following tables respectively. The detailed results are shown in the attached excel-file respectively.

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| All | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2-1: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2% | -0.1% | -0.1% | 0.2% | 0.1% | 0.0% |
| Class B | 0.2% | 0.1% | 0.0% | 0.3% | 0.0% | 0.0% |
| Class C | 0.2% | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% |
| Class D | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| All | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table2-2: Results for RandomAccess

|  | Low delay B |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | -0.1% | 0.0% | 0.3% | 0.0% | 0.0% |
| Class C | 0.2% | -0.1% | 0.0% | 0.2% | 0.1% | -0.2% |
| Class D | 0.1% | 0.3% | 0.1% | 0.1% | 0.3% | -0.4% |
| Class E | 0.3% | -0.2% | 1.0% | 0.2% | 0.2% | 0.4% |
| All | 0.2% | 0.0% | 0.2% | 0.2% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table2-3: Results for LowDelay B

## Comment

This proposal contributes to complexity reduction and is preferable for both HW and SW if the subjective quality is not problem.

# Subjective quality evaluation

Subjective quality was evaluated with using below conditions and equipments.

Conditions

- Kimono1 randomaccess QP=37

- Kimono1 lowdelay_loco QP=32

- BQMall randomaccess QP=37

- BQMall lowdelay QP=32

- Vidyo3 lowdelay QP=37

- Vidyo3 lowdelay_loco QP=32

Equipments

- Monitor: EIZO FlexScan SX3031W

- Processor : Intel Core i5 750 BOX

- RAM : DCDDR3-4GB-1333

- Graphic Card : ENGTX285 2DI 1GD3

Visual quality is improved in BQMall (RA_HE, QP=37, around frame=115) as shown in JCTVC-G086.

The differences between this proposal and HM-4.0 in other sequences are visible, but it is difficult to decide which one is better. So, the decision should be done by subjective viewing in Geneva meeting.

# Conclusion

In this contribution, the cross verified results of TI’s proposal regarding deblocking filter are reported. It is confirmed that the results perfectly match with the ones provided by TI.

# References

[1] JCT-VC, “CE12 Subset7.4: Test results on decreasing worst case complexity of deblocking filter”, JCTVC-G086, 7th. JCT-VC Meeting, Geneva, CH, November 2011

[2]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, 6th.JCT-VC Meeting, Torino, IT, July 2011

[3] JCT-VC, “Description of CE12: deblocking filtering”, JCTVC-F912, 6th. JCT-VC Meeting, Torino, IT, July 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).