| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G255 |
| --- | --- |

| Title: | CE12 Subtest2: Parallel deblocking improvement based on JCTVC-F214 tool1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Junichi Tanaka Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2743 Masaru.Ikeda@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes one approach to improve and develop the parallel deblocking filter (JCTVC-E181 [1] and JCTVC-E224 [2]). This approach is both horizontal (H) and vertical (V) filtering process applied to reconstructed samples. This proposal makes the dependency between H and V filter reduced, and then both H and V filtering process can be performed in parallel. In addition it is expected that the necessary memory to store for H filtered samples in AVC is made redundant. Sony has two proposals, and one is basic proposal based on JCTVC-F214 tool1 and the other one is improved one based on the basic proposal. The experimental results show 0.0-0.2% in BD-rate regarding basic proposal, -0.1-0.2 in BD-rate regarding improved one, and the subjective quality is similar to HM-4.0.

# Introduction

Though two proposals regarding parallel deblocking filter is adopted, there are some issues to improve.

- The result of the decision for V filtering has to be stored in both LCU-based and Frame-based process.

- LCU-based process has the constraint that H filtering in #n+1 LCU has to be executed more early than V filtering in #n LCU.

We, Sony, would like to show one approach to improve above with keeping the functionality of the parallel deblocking.

The explanation regarding this proposal is described in Section 2, the results regarding experiment and subjective evaluation are shown in Section 3, the conclusion is provided in Section 4.

# Proposal

## Basic proposal

This proposal makes both H and V filtering process applied to reconstructed samples as shown in Figure1. The output samples for luma are obtained by linear sum of both H and V filter as shown in List1. This description shows that the sample applied by either H or V filter is obtained by the output of the corresponding filter only. The filter coefficients are equal to 1 at all positions from the block boundary.  So this proposal becomes simpler than Sony’s proposal in JCTVC-F214. The remarkable point is that both H and V filtering process use the same filter and decision as HM-4.0 respectively, and all parameters (β, tc and so on) are not changed. So it is possible to make this proposal apply to deblocking filter that is modified as suggested in the other contribution.

In Figure2, for example, sample#0, sample#1, sample#2, sample#3, sample#4, sample#5, sample#6, sample#7 and sample#8 are calculated as shown in List2.

The output samples for chroma are also obtained by linear sum of both H and V filter as shown in List1. But both H and V coefficient (PH and PV) for chroma are always equal to 1 at all positions from the block boundary.

Figure1: Both H and V filtering process

P OUT = PIN + C H*(PH–PIN) + C V*(PV–PIN)

PIN	: Reconstructed sample value

PH	: H filtered sample value (the same filtering process as HM-4.0)

PV	: V filtered sample value (the same filtering process as HM-4.0)

CH	: H filter coefficient

CV	: V filter coefficient

POUT	: Output sample value

List1: description of both H and V filtering process

Figure2: Linear sum of H/V filtering

Sample#0:	P0OUT = P0IN + 1*(P0H-P0IN) + 1*(P0V-P0IN) = P0H+ P0V-P0IN

Sample#1:	P1OUT = P1IN + 1*(P1H-P1IN) + 1*(P1V-P1IN) = P1H+ P1V-P1IN

Sample#2:	P2OUT = P2IN + 1*(P2H-P2IN) + 1*(P2V-P2IN) = P2H+ P2V-P2IN

Sample#3:	P3OUT = P3IN + 1*(P3H-P3IN) + 1*(P3V-P3IN) = P3H+ P3V-P3IN

Sample#4:	P4OUT = P4IN + 1*(P4H-P4IN) + 1*(P4V-P4IN) = P4H+ P4V-P4IN

Sample#5:	P5OUT = P5IN + 1*(P5H-P5IN) + 1*(P5V-P5IN) = P5H+ P5V-P5IN

Sample#6:	P6OUT = P6IN + 1*(P6H-P6IN) + 1*(P6V-P6IN) = P6H+ P6V-P6IN

Sample#7:	P7OUT = P7IN + 1*(P7H-P7IN) + 1*(P7V-P7IN) = P7H+ P7V-P7IN

Sample#8:	P8OUT = P8IN + 1*(P8H-P8IN) + 1*(P8V-P8IN) = P8H+ P8V-P8IN

List2: Example of filter calculation in Figure2

## Improvement of basic proposal

Basic proposal, both H and V filtering applied to reconstructed samples, needs to store 4 lines buffer as reconstructed samples for luma in deblocking filter and HM-4.0 has on/off decision for edge segments of 8 lines. Upper 4 lines in 8 lines on LCU boundary are horizontally filtered in upper LCU and lower 4 lines are horizontally filtered in lower LCU, and so on/off decision buffer for 8 lines requires for horizontal filtering in lower LCU.

Panasonic’s proposal in JCTVC-Gxxx approaches this issue and performs on/off decision for edge segments of 4 lines instead of 8 lines. If Panasonic’s proposal in JCTVC-G590 [3] is added to Sony’s proposal, it is possible to show good harmonization.

# Results

## Experiment

The experiment is executed with using HM-4.0 based on JCTVC-F712 [4], and the configuration and common test condition are as JCTVC-F900[5].

Both the encoding and the decoding time are measured on the same machine as anchor, and the decoding time is measured without output of decoded YUV data.

### Basic proposal

The experimental results are shown in Table 3-1, 3-2 and 3-3.

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
| Class E | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| All | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

Table 3-1: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class C | 0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| All | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 3-2: Results for RandomAccess

|  | Low delay B |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | -0.2% | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | 0.0% | -0.1% | 0.1% | 0.2% | -0.1% |
| Class D | 0.0% | 0.1% | -0.3% | 0.1% | 0.0% | -0.3% |
| Class E | 0.1% | 0.0% | 0.7% | -0.1% | 0.1% | -0.1% |
| All | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 3-3: Results for LowDelay B

The experimental results show 0.0 – 0.2% increases in BD bitrate, no change in encoding time and 0 – 2% increases in decoding time, so the results are similar with HM-4.0.

### Improvement of basic proposal

The experimental results in the harmonization are shown in Table 3-4, 3-5 and 3-6.

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class B | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
| Class E | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| All | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

Table 3-4: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | 0.0% | 0.1% | 0.1% | 0.3% | 0.2% |
| Class B | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| All | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3-5: Results for RandomAccess

|  | Low delay B |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class C | -0.2% | -0.1% | 0.0% | 0.1% | 0.2% | -0.1% |
| Class D | 0.0% | 0.5% | 0.2% | 0.1% | 0.0% | -0.3% |
| Class E | -0.4% | 0.3% | 0.8% | -0.1% | 0.1% | -0.1% |
| All | -0.1% | 0.2% | 0.2% | -0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

Table 3-3: Results for LowDelay B

The experimental results show -0.1 – 0.2% in BD bitrate, -0.2 - 0.0% in encoding time and 0.0 - 0.2% in decoding time, so the results are similar with HM-4.0.

## Subjective quality check

Subjective quality was evaluated with using below conditions and equipments.

Conditions

- Kimono1:	RA_HE/QP=37

- Kimono1:	LD_LC/QP=32

- BQMall:	RA_HE/QP=37

- BQMall:	LD_LC/QP=32

- Vidyo3:	LD_HE/QP=37

- Vidyo3:	LD_LC/QP=32

Equipments

- Monitor: 	EIZO FlexScan SX3031W

- Processor : 	Intel Core i5 750 BOX

- RAM : 	DCDDR3-4GB-1333

- Graphic Card : 	ENGTX285 2DI 1GD3

The difference between Anchor and Sony’s proposal is visible, but it is difficult to say which one is better. So Sony’s proposal has no impact on subjective quality.

# Conclusion

Sony recommends that this proposal, which is both H and V filtering process applied to reconstructed samples, is adopted into HM for the following reasons.

- This proposal is friendly to the parallel deblocking process because the dependency between H and V filtering process decreases.

- It is possible to make this proposal apply to deblocking filter that is modified as suggested in the other contribution.

# References

[1] JCT-VC, “CE12 Subset2: Parallel deblocking filter”, JCTVC-E181,5th. JCT-VC Meeting, Geneva, CH, March 2011

[2] JCT-VC, “CE12: Results for parallel deblocking filter decisions”, JCTVC-E224,5th. JCT-VC Meeting, Geneva, CH, March 2011

[3] JCT-VC, “”, JCTVC-G590, 7th. JCT-VC Meeting, Geneva, CH, November 2011

[4] JCT-VC, “Description of CE12: deblocking filtering”, JCTVC-F912, 6th. JCT-VC Meeting, Torino, IT, July 2011

[5]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900,6th. JCT-VC Meeting, Torino, IT, July 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).