| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G256 |
| --- | --- |

| Title: | CE12 Subtest2: Parallel decision improvement and reduction based on JCTVC-F214 tool2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Junichi Tanaka Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2743 Masaru.Ikeda@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to improve the parallel deblocking filter (JCTVC-E181 [1] and JCTVC-E224 [2]). This approach is decision by using the central non-deblocked samples in block boundary. The decision with the central non-deblocked samples means the decision for reconstructed samples because the decision regarding “on/off decision” and “strong/weak filter selection” is performed by the central non-deblocked samples. Sony has two proposals, and one is basic proposal based on JCTVC-F214 tool2 [3] and the other one is improved one based on the basic proposal. The experimental results show 0.0–0.3% increases in BD-rate and 0–2% decreases in decoding time regarding basic proposal and 0.0–0.2% increases in BD-rate and 0–2% decreases in decoding time regarding improved one. Therefore it is expected that the necessary memory to store for the results of the decision is made redundant with keeping the similar performance to HM-4.0 and that it is possible to save the computation in the decision process, and the subjective quality is similar to HM-4.0.

# Introduction

Though two proposals regarding parallel deblocking filter is adopted, there are some issues to improve. One of some issues is that the result of the decision for vertical (V) filtering has to be stored in both LCU-based and frame-based process. Sony would like to propose to improve above issue with keeping the functionality of the parallel deblocking.

The explanation regarding this proposal is described in Section 2, the results regarding experiment and subjective evaluation are shown in Section 3, the conclusion is provided in Section 4.

# Proposal

## Basic proposal

This proposal modifies deblocking decisions for reconstructed samples by changing the using samples  into the central lines only (line-3 and 4) as shown in Figure1. This decisions show both “on/off decision” and “strong/weak filter selection”.

On/off decision is performed with the central lines only as shown in List-1. The calculation is the same one as HM-4.0.

Strong/weak filter selection for 8 lines is decided by the central lines only. If both line-3 and 4 are decided as strong filter, 8 lines are filtered by strong filter, and if not and either line is decided as weak filter, 8 lines are filtered by weak filter. Though new weak filter decision and filter for pixel p1i and q1i  are adopted at Torino meeting [4], the weak filter decision for pixel p1i and q1i is also performed with the central lines only and the calculation is the same one as HM-4.0.

The clip of tc for difference of filter calculation in strong filter is added to care too much filtering owing to decision with the central samples only as shown in List-2. All parameters (β, tc, iSideThreshold and so on) are not changed.

di = |p2i–2*p1i+p0i|+|q2i–2*p1i+q0i| (i = 0..7)

Figure1: Central samples

|  | HM4.0 | Proposal |
| --- | --- | --- |
| On/off decision | d2 + d5 < β | d3 + d4 < β |
| Strong filter decision | Independent decision for each line | Block decision with line-3 and 4 for 8 lines |
| Weak filter decision | Independent weak filter on/off decision for each line Block decision with line-2 and 5 for pixel p1i and q1i | Block decision with line-3 and 4 for 8 lines Block decision with line-3 and 4 for pixel p1i and q1i |

List-1: Chart of the deblocking decisions in base proposal

p0i’ = Clip (p0i + Clip (-tc)-tc ( ( p2i + 2*p1i + 2*p0i + 2*q0i + q1i + 4 ) >> 3 - p0i ))		(1)

p1i’ = Clip (p1i + Clip (-tc)-tc ( ( p2i + p1i + p0i + q0i + 2 ) >> 2 – p1i ))				(2)

p2i’ = Clip (p2i + Clip (-tc)-tc ( ( 2*p3i + 3*p2i + p1i + p0i + q0i + 4 ) >> 3 – p2i ))			(3)

q0i’ = Clip (q0i + Clip (-tc)-tc ( ( p1i + 2*p0i + 2*q0i + 2*q1i + q2i + 4 ) >> 3 – q0i ))		(4)

q1i’ = Clip (q1i + Clip (-tc)-tc ( ( p0i + q0i + q1i + q2i + 2 ) >> 2 – q1i ))				(5)

q2i’ = Clip (q2i + Clip (-tc)-tc ( ( p0i + q0i + q1i + 3*q2i + 2*q3i + 4 ) >> 3 – q2i ))			(6)

List-2: Expression of strong filter with clip of tc (i=0..7)

## Improvement of basic proposal

The decisions with central lines only are applied to both horizontal and vertical filtering in base proposal. But horizontal decision does not necessarily have to be performed with central lines only (line-3 and 4) for parallel decision because horizontal filter is performed prior to vertical filter, so even if either line in 8 lines is used for horizontal decisions, the parallelization in horizontal decision is kept.

Additionally Sony proposes that horizontal decision is performed with line-2 and 5 only and vertical decision is done with line-3 and 4 only as shown in List-3. This proposal does not have clip of tc for difference of filter calculation in strong filter.

|  | HM4.0 | Proposal |  |
| --- | --- | --- | --- |
| Direction | Horizontal and Vertical | Horizontal | Vertical |
| On/off decision | d2 + d5 < β | d2 + d5 < β | d3 + d4 < β |
| Strong filter decision | Independent decision for each line | Block decision with line-2 and 5 for 8 lines | Block decision with line-3 and 4 for 8 lines |
| Weak filter decision | Independent weak filter on/off decision for each line Block decision with line-2 and 5 for pixel p1i and q1i | Block decision with line-2 and 5 for 8 lines Block decision with line-2 and 5 for pixel p1i and q1i | Block decision with line-3 and 4 for 8 lines Block decision with line-3 and 4 for pixel p1i and q1i |

List-3: Chart of the deblocking decisions in additional proposal

# Results

## Experiment

The experiment is executed with using HM-4.0 based on JCTVC-F712 [5], and the configuration and common test condition are as JCTVC-F900[6].

Both the encoding and the decoding time are measured on the same machine as anchor, and the decoding time is measured without output of decoded YUV data.

### Basic proposal

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| All | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 3-1: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1% | -0.1% | -0.1% | 0.0% | 0.2% | 0.0% |
| Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| All | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table 3-2: Results for RandomAccess

|  | Low delay B |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.2% | -0.1% | 0.2% | 0.0% | 0.2% |
| Class C | 0.1% | -0.1% | -0.1% | 0.1% | 0.0% | -0.4% |
| Class D | 0.1% | 0.2% | 0.4% | 0.1% | 0.1% | -0.1% |
| Class E | 0.8% | 0.4% | 0.6% | 0.3% | 0.4% | 0.0% |
| All | 0.3% | 0.0% | 0.1% | 0.2% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table 3-3: Results for LowDelay B

The experimental results show 0.0 – 0.3% increases in BD bitrate, no change in encoding time and 0 – 2% decreases in decoding time. This proposal provides the similar performance to HM-4.0 and reduction in deblocking decision.

### Improvement of basic proposal

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| All | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 3-4: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | -0.1% | 0.0% |
| Class D | 0.1% | -0.1% | 0.1% | 0.1% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| All | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 3-5: Results for RandomAccess

|  | Low delay B |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.1% | -0.2% | 0.1% | 0.1% | 0.0% |
| Class C | 0.2% | -0.1% | 0.2% | 0.1% | 0.1% | -0.3% |
| Class D | 0.1% | 0.3% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class E | 0.5% | 0.3% | 0.9% | 0.2% | 0.2% | -0.1% |
| All | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 3-6: Results for LowDelay B

The experimental results show 0.0 – 0.2% increases in BD bitrate, no change in encoding time and 0 – 2% decreases in decoding time. This proposal can provide more similar performance to HM-4.0 than basic proposal and reduction in deblocking decision.

## Subjective quality

Subjective quality was evaluated with using below conditions and equipments.

Conditions

- Kimono1:	RA_HE/QP=37

- Kimono1:	LD_LC/QP=32

- BQMall:	RA_HE/QP=37

- BQMall:	LD_LC/QP=32

- Vidyo3:	LD_HE/QP=37

- Vidyo3:	LD_LC/QP=32

Equipments

- Monitor: 	EIZO FlexScan SX3031W

- Processor : 	Intel Core i5 750 BOX

- RAM : 	DCDDR3-4GB-1333

- Graphic Card : 	ENGTX285 2DI 1GD3

The difference between Anchor and Sony’s proposal is visible in both cases, but it is difficult to say which one is better. So Sony’s proposal has no impact on subjective quality.

# Conclusion

Sony recommends that improvement of basic proposal (2.2), which is deblocking decisions with both line-2 and 5 only for horizontal filter and line-3 and 4 for vertical one, is adopted into HM because this proposal can provide parallel decision without extra decision buffers and save computational time in decisions with keeping BD-rate and subjective quality.

# References

[1] JCT-VC, “CE12 Subset2: Parallel deblocking filter”, JCTVC-E181, 5th. JCT-VC Meeting, Geneva, CH, March 2011

[2] JCT-VC, “CE12: Results for parallel deblocking filter decisions”, JCTVC-E224, 5th. JCT-VC Meeting, Geneva, CH, March 2011

[3] JCT-VC, “Parallel deblocking improvement”, JCTVC-F214, 6th. JCT-VC Meeting, Torino, IT, July 2011

[4] JCT-VC, “CE12: Ericsson’s and MediaTek’s deblocking filter”, JCTVC-F118, 6th. JCT-VC Meeting, Torino, IT, July 2011

[4] JCT-VC, “Description of CE12: deblocking filtering”, JCTVC-F912, 6th. JCT-VC Meeting, Torino, IT, July 2011

[5]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900,6th. JCT-VC Meeting, Torino, IT, July 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).