| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G257 |
| --- | --- |

| Title: | CE12 Subtest3: deblocking vertical tap reduction for line buffer based on JCTVC-F215 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2743 Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to reduce the line buffers required in deblocking filter base on JCTVC-F215[1]. A lot of line buffers are required in deblocking filter, SAO (Sample Adaptive Offset) and ALF (Adaptive Loop Filter) in HM-4.0. Especially, deblocking filter is included in both high efficiency and low complexity and it is considered that deblocking filter is used in many cases, and so it is significant that the line buffers in deblocking filter are reduced alone. Sony proposes to reduce one line buffer with keeping BD-Rate and subjective quality by reducing upper one pixel to read in vertical filtering. The experimental results show 0.0-0.1 increases for luma in BD-rate and similar run-time, and the subjective quality is similar to HM-4.0.

# Introduction

HW is usually processed in LCU-based and has line buffers for deblocking filter process for block boundary between vertical LCUs. Vertical filter has the same tap length and shape as horizontal filter in current deblocking filter and maximum tap length is plus or minus 4 taps (total 8 taps) in the case of strong filter for luma as shown in Figure 1 and List1. So it is necessary to have 4 line buffers in HW process. Though it is also necessary to have 4 line buffers in AVC, one line memory for 4K image corresponds to 2 line buffers for 1920 image in AVC. So it is significant even if one line buffer is reduced.

The explanation regarding this proposal is described in Section 2, the results regarding experiment and subjective evaluation are shown in Section 3, the conclusion is provided in Section 4.

Figure1: 8x8 Block boundary

p0 i’ = Clip1Y( ( p2i + 2*p1i + 2*p0i + 2*q0i + q1i + 4 ) >> 3 )		(1)

p1 i’ = Clip1Y( ( p2i + p1i + p0i + q0i + 2 ) >> 2 )				(2)

p2 i’ = Clip1Y( ( 2*p3i + 3*p2i + p1i + p0i + q0i + 4 ) >> 3 )			(3)

q0 i’ = Clip1Y( ( p1i + 2*p0i + 2*q0i + 2*q1i + q2i + 4 ) >> 3 )		(4)

q1 i’ = Clip1Y( ( p0i + q0i + q1i + q2i + 2 ) >> 2 )				(5)

q2 i’ = Clip1Y( ( p0i + q0i + q1i + 3*q2i + 2*q3i + 4 ) >> 3 )			(6)

List1: Formula of strong filter (i=0..7)

# Proposal

Sony proposes to reduce one pixel (p3i in Figure 2) above horizontal edge. Sample p3i is used for calculating sample p2i’, and Sony proposes to use sample p1i as a substitute for sample p3i for all horizontal edges. This make one line buffer (as a red line shown in Figure 2) reduced from 4 to 3 lines.

Figure2: 8x8 Block boundary

p0 i’ = Clip1Y( ( p2i + 2*p1i + 2*p0i + 2*q0i + q1i + 4 ) >> 3 )		(1)

p1 i’ = Clip1Y( ( p2i + p1i + p0i + q0i + 2 ) >> 2 )				(2)

p2 i’ = Clip1Y( ( 2*p1i + 3*p2i + p1i + p0i + q0i + 4 ) >> 3 )			(3)

= Clip1Y( ( 3*p2i + 3*p1i + p0i + q0i + 4 ) >> 3 )

q0 i’ = Clip1Y( ( p1i + 2*p0i + 2*q0i + 2*q1i + q2i + 4 ) >> 3 )		(4)

q1 i’ = Clip1Y( ( p0i + q0i + q1i + q2i + 2 ) >> 2 )				(5)

q2 i’ = Clip1Y( ( p0i + q0i + q1i + 3*q2i + 2*q3i + 4 ) >> 3 )			(6)

List2: Proposed formula of strong filter (i=0..7)

The circuit for both horizontal and vertical filter remains common because sample p1i is used as sample p3i , and the special control for this modification is not required because this modification is applied for all horizontal edges.

# Results

The experiment is executed with using HM-4.0 based on JCTVC-F712 [2], and the configuration and common test condition are as JCTVC-F900[3].

## Experiment

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| All | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 3-1: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| All | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 102% |  |  |

Table 3-2: Results for RandomAccess

|  | Low delay B |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.3% | -0.2% |
| Class D | 0.0% | 0.5% | 0.0% | 0.1% | 0.4% | -0.3% |
| Class E | 0.3% | 0.0% | 0.5% | 0.0% | 0.1% | 0.0% |
| All | 0.1% | 0.1% | 0.1% | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3-3: Results for LowDelay B

Both the BD-rate and the run-time is almost the same one as HM-4.0.

## Subjective quality

Subjective quality was evaluated with using below conditions and equipments.

Conditions

- Kimono1:	RA_HE/QP=37

- Kimono1:	LD_LC/QP=32

- BQMall:	RA_HE/QP=37

- BQMall:	LD_LC/QP=32

- Vidyo3:	LD_HE/QP=37

- Vidyo3:	LD_LC/QP=32

Equipments

- Monitor: 	EIZO FlexScan SX3031W

- Processor : 	Intel Core i5 750 BOX

- RAM : 	DCDDR3-4GB-1333

- Graphic Card : 	ENGTX285 2DI 1GD3

The difference between Anchor and Sony’s proposal is visible, but it is difficult to say which one is better. So Sony’s proposal has no impact on subjective quality.

# Conclusion

Sony recommends that this proposal is adopted into HM for the following reasons.

- Line buffer in HEVC processing 4K image has a large impact on HW processing.

- It is significant that the line buffers in deblocking filter are reduced because deblocking filter (without ALF) is likely to be used in many cases.

- Reducing one pixel in vertical filtering has little impact on BD-Rate and subjective quality.

# References

[1] JCT-VC, “Vertical tap length reduction to reduce line memory in deblocking filter”, JCTVC-F215, 6th. JCT-VC Meeting, Torino, IT, July 2011

[2] JCT-VC, “Description of CE12: deblocking filtering”, JCTVC-F912, 6th. JCT-VC Meeting, Torino, IT, July 2011

[3]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, 6th.JCT-VC Meeting, Torino, IT, July 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).