| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G259_r1 |
| --- | --- |

| Title: | Non-CE3: Modifications of LCU-based adaptive interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shohei Matsuo Seishi Takamura Hirohisa Jozawa 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This document reports two modifications of an LCU-based adaptive interpolation filter described in JCTVC-G258. First modification is that the proposed filter is applied to chrominance components (Cb and Cr) as well as luminance component (Y). Second modification is a prediction method for luminance filter coefficients. The proposal was implemented in HM4.0 software to evaluate its performance. Compared to the HM4.0 anchor, the overall average coding gain for Y, Cb and Cr were about 0.49%, 0.38% and 0.37%, respectively. The maximum coding gain of Y was about 10.2% for the sequence “Vidyo3” in LP-LC case. The maximum coding gains of Cb and Cr were about 7.1% and 5.7% for the sequence “Nebuta” in RA-LC case, respectively. The computational complexity at the encoder and decoder were 101.42% and 102.51% on average, respectively.

# Introduction

Interpolation filter that supports motion estimation with fractional-pel accuracy is one of important coding tools for improving efficiency of inter frame coding. In the 4th JCT-VC meeting, it was decided that HM2.0 adopts the 8-tap DCT-IF as an interpolation filter for both HE and LC cases. In HM3.0 and HM4.0, the same filter is used for the interpolation process. As for chrominance interpolation, 4-tap DCT-IF is adopted. Although DCT-IF reduces effectively residual energy, the coefficient values of the filter are constant regardless of the characteristics of the input video sequences. To improve the performance of such a fixed interpolation, adaptive interpolation filters (AIFs) were proposed, for example, 2D-AIF [1], SAIF [2]. Each AIF derives the optimal filter coefficients for each frame. However, single set of filter coefficients may not be the optimal option, when the image has plural regions each of which has different characteristics (motion, texture, etc.). To deal with the problem, a region-based adaptive approach was introduced into AIF. This approach is called Region-based AIF (RBAIF) [3-6]. RBAIF generates multiple sets of filter coefficients that are optimized for each region. In the 6th JCT-VC meeting, RBAIF had two simple segmentation modes: one is “left and right” segmentation mode, and the other is “top and bottom” segmentation mode. In the contribution [7], more flexible segmentation mode, an LCU-based AIF, is introduced to consider the image locality. In this contribution, the LCU-based AIF is applied to chrominance components as well as luminance component.

# LCU-Based Adaptive Interpolation Filter

## Basic idea

RBAIF designs the optimal multiple sets of filter coefficients for each region in a frame. For example, let us consider a frame which consists of two regions, foreground and background, as shown in Figure 1. In this case, conventional RBAIF generates two sets of filter coefficients. However, the segmentation shape is very simple and the conventional RBAIF does not fully take advantage of image locality.

Figure 1: Conventional RBAIF (left) and LCU-based AIF (right).

Our proposal supports three filter types shown in Table.1. As shown in Table 1, it includes DCT-IF and AIF. The filter type 0, 1 and 2 denote DCT-IF, frame-based AIF and the proposed LCU-based AIF, respectively. In the filter type 2, the input picture is partitioned into multiple regions. The detailed encoding algorithm will be described in Sec. 2.2.

Note that the proposed scheme can support arbitrary number of regions. The proposal is currently an extension of SAIF in terms of filter coefficient coding. Of course, the concept of the proposal can be also coupled with other AIF methods.

Table 1: Definition of filter types.

## Encoding algorithm including chrominance components

The detailed encoding algorithm is summarized as follows.

S1: Derive motion vectors by using DCT-IF and calculate the RD cost of filter type 0.

S2: Derive the filter coefficients of frame-based AIF by using the motion vectors obtained in S1 and calculate the RD cost of filter type 1.

S3: Set n kinds of initial interpolation filters (n denotes the number of regions). The initial interpolation filter is defined by the following equation. ci denotes the filter coefficients and the index i means the position of each filter coefficient. M denotes half the length of interpolation filter.  denotes the phase parameter that controls the filter shape. Each initial interpolation filter is designed by . The default DCT-IF defined in HM4.0 is set when  = 0. In the case of n=2, two kinds of initial interpolation filters are prepared and a filter index is set for each initial interpolation filter, for example, filter index 0 (=0) and filter index 1 (=0.02).

S4: Calculate the SSD for each LCU by using initial interpolation filters and set the filter index which achieves the lowest SSD to each LCU. Note that the same motion vectors are shared in filter type 2 as well as filter type 1.

S5: Categorize LCUs into n kinds of regions based on the filter index in S4, and calculate the filter coefficients for each region. This process means that the input frame is segmented into n kinds of regions. After the filter coefficients for each region are derived, re-categorize LCUs by using the derived filter coefficients. In this step, the SSD of chrominance is added to that of luminance.

S6: Store the filter index of each LCU. This filter index is equivalent to the region index. The region index of all LCUs included in one slice is defined as a segmentation map. Then, calculate the RD cost of filter type 2.

S7: Select the best filter type that minimizes the RD cost.

S8: Encode the filter type and the derived filter coefficients. When filter type 0 (DCT-IF) is selected, only the filter type is encoded and no filter coefficient is encoded. When filter type 1 (frame-based AIF) is selected, the filter type and the three sets of filter coefficients (one set per component) are encoded. When filter type 2 (LCU-based AIF) is selected, the segmentation map is encoded as well as the filter type and 3n sets of filter coefficients.

To reduce the encoding complexity, two methods are applied in this proposal. One is the reuse of motion vectors and the other is the skip of ALF in the RD cost calculation process. Motion vectors derived using DCT-IF is reused as shown in Figure 2. The encoder has to derive the distortion value of a local decoded image. In the HE configuration, the ALF is used on the default setting. Therefore, ALF process was applied four times (DCT-IF, AIF, RBAIF and the decided best filter type) in the contribution [5]. In this contribution, the same skip method described in the previous contribution [6] is used to reduce the calculation for ALF. Three ALF process in filter type selection are skipped. After the best filter type is decided, the ALF process is applied once to the current picture in the same way as HM4.0.

Figure 2: Encoding complexity reduction.

## Modification of filter coefficient prediction

In the contribution [7], the residual filter coefficients are encoded and sent to the decoder. The residual filter coefficients were generated by subtracting the corresponding DCT-IF coefficients from the generated AIF coefficients. In this contribution, a modified prediction method is proposed.

Let us consider the generated filter coefficients for half-pel position and quarter-pel position as follows:

1/2 position: {C1, C2, C3, C4, C4, C3, C2, C1},

1/4 position: {D1, D2, D3, D4, D5, D6, D7, D8}.

Let us consider the predicted filter coefficients for half-pel position and quarter-pel position as follows:

1/2 position: {P1, P2, P3, P4, P4, P3, P2, P1},

1/4 position: {Q1, Q2, Q3, Q4, Q5, Q6, Q7, Q8}.

Regarding half-pel position, firstly, C4 is predicted by a parameter h4: P4 = h4.

Then C3 is predicted by using C4 and parameters h3 and h3: P3 = h3*C4 + h3.

C2 is predicted by using C3 and C4 with parameters h2, 'h2 and h2: P2 = h2*C3 + 'h2*C4 + h2.

In the same manner, the filter coefficients for quarter-pel position are predicted as follows:

Q4 = q4.

Q5 = q5*D4 + q5.

Q3 = q3*D4 + 'q3*D5 + q3.

Q6 = q6*D3 + 'q6*D4 + ''q6*D5 + q6.

Q2 = q2*D3 + 'q2*D4 + ''q2*D5 + '''q2*D6 + q2.

Q7 = q7*D2 + 'q7*D3 + ''q7*D4 + '''q7*D5''''q7*D6 + q7.

Q1 = q1*D2 + 'q1*D3 + ''q1*D4 + '''q1*D5''''q1*D6'''''q1*D7+ q1.

These parameters above are fixed and their values are derived by preliminary experiments.

## LCU-based adaptive interpolation filter parameter syntax

The semantics related to the proposed interpolation filter are as follows.

rbaif_filter_type specifies the type of the interpolation filter that is used in the current picture.

rbaif_num_regions specifies the number of the regions included in the current picture.

rbaif_coding_mode[i] specifies the entropy coding mode for the filter coefficients in the regions included in the current picture. The index i specifies the region number.

rbaif_golomb_mode[i] specifies the shape parameter of the Golomb codes. The index i specifies the region number.

rbaif_vertical_coeff_check_flag_luma[i] specifies whether the values of the generated AIF filter coefficients for vertical interpolation are the same as those of the DCT-IF. If the rbaif_vertical_coeff_check_flag is equal to 1, the residual AIF filter coefficients for the vertical direction are not decoded. If the rbaif_vertical_coeff_check_flag is equal to 0, they are decoded. The index i specifies the region number.

rbaif_horizontal_coeff_check_flag_luma[i] specifies whether the values of the generated AIF filter coefficients for horizontal interpolation are the same as those of the DCT-IF. If the rbaif_horizontal_coeff_check_flag is equal to 1, the residual AIF filter coefficients for the horizontal direction are not decoded. If the rbaif_horizontal_coeff_check_flag is equal to 0, they are decoded. The index i specifies the region number.

rbaif_diff_filter_coefficients_v_quarter_luma[j][i] specifies the residual values between the predicted filter coefficients and the generated AIF filter coefficients for the vertical direction to interpolate the quarter-pel positions of luminance. The index i and j specify the coefficient number and the region number, respectively.

rbaif_diff_filter_coefficients_v_half_luma[j][i] specifies the residual values between the predicted filter coefficients and the generated AIF filter coefficients for the vertical direction to interpolate the half-pel positions of luminance. The index i and j specify the coefficient number and the region number, respectively.

rbaif_diff_filter_coefficients_h_quarter_luma[j][i] specifies the residual values between the predicted filter coefficients and the generated AIF filter coefficients for the horizontal direction to interpolate the quarter-pel positions of luminance. The index i and j specify the coefficient number and the region number, respectively.

rbaif_diff_filter_coefficients_h_half_luma[j][i] specifies the residual values between the predicted filter coefficients and the generated AIF filter coefficients for the horizontal direction to interpolate the half-pel positions of luminance. The index i and j specify the coefficient number and the region number, respectively.

rbaif_vertical_coeff_check_flag_chroma[j][i] specifies whether the values of the generated AIF filter coefficients for vertical interpolation are the same as those of the DCT-IF. If the rbaif_vertical_coeff_check_flag is equal to 1, the residual AIF filter coefficients for the vertical direction are not decoded. If the rbaif_vertical_coeff_check_flag is equal to 0, they are decoded. The index i and j specify the chroma component number and the region number, respectively.

rbaif_horizontal_coeff_check_flag_chroma[j][i] specifies whether the values of the generated AIF filter coefficients for horizontal interpolation are the same as those of the DCT-IF. If the rbaif_horizontal_coeff_check_flag is equal to 1, the residual AIF filter coefficients for the horizontal direction are not decoded. If the rbaif_horizontal_coeff_check_flag is equal to 0, they are decoded. The index i and j specify the chroma component number and the region number, respectively.

rbaif_diff_filter_coefficients_v_one_eighth_chroma[k][j][i] specifies the residual values between the DCT-IF filter coefficients and the generated AIF filter coefficients for the vertical direction to interpolate the one-eighth-pel positions of chrominance. The index i, j and k specify the coefficient number, chroma component number and the region number, respectively.

rbaif_diff_filter_coefficients_v_quarter_chroma[k][j][i] specifies the residual values between the DCT-IF filter coefficients and the generated AIF filter coefficients for the vertical direction to interpolate the quarter-pel positions of chrominance. The index i, j and k specify the coefficient number, chroma component number and the region number, respectively.

rbaif_diff_filter_coefficients_v_three_eighths_chroma[k][j][i] specifies the residual values between the DCT-IF filter coefficients and the generated AIF filter coefficients for the vertical direction to interpolate the three-eighths-pel positions of chrominance. The index i, j and k specify the coefficient number, chroma component number and the region number, respectively.

rbaif_diff_filter_coefficients_v_half_chroma[k][j][i] specifies the residual values between the DCT-IF filter coefficients and the generated AIF filter coefficients for the vertical direction to interpolate the half-pel positions of chrominance. The index i, j and k specify the coefficient number, chroma component number and the region number, respectively.

rbaif_diff_filter_coefficients_h_one_eighth_chroma[k][j][i] specifies the residual values between the DCT-IF filter coefficients and the generated AIF filter coefficients for the horizontal direction to interpolate the one-eighth-pel positions of chrominance. The index i, j and k specify the coefficient number, chroma component number and the region number, respectively.

rbaif_diff_filter_coefficients_h_quarter_chroma[k][j][i] specifies the residual values between the DCT-IF filter coefficients and the generated AIF filter coefficients for the horizontal direction to interpolate the quarter-pel positions of chrominance. The index i, j and k specify the coefficient number, chroma component number and the region number, respectively.

rbaif_diff_filter_coefficients_h_three_eighths_chroma[k][j][i] specifies the residual values between the DCT-IF filter coefficients and the generated AIF filter coefficients for the horizontal direction to interpolate the three-eighths-pel positions of chrominance. The index i, j and k specify the coefficient number, chroma component number and the region number, respectively.

rbaif_diff_filter_coefficients_h_half_chroma[k][j][i] specifies the residual values between the DCT-IF filter coefficients and the generated AIF filter coefficients for the horizontal direction to interpolate the half-pel positions of chrominance. The index i, j and k specify the coefficient number, chroma component number and the region number, respectively.

rbaif_segmentation_map[i] specifies the region index of each LCU included in one slice. The index i specifies the position of LCU. This map is shared between luminance and chrominance.

The syntax specification using above semantics are shown in Table 2.

Table 2: Syntax specification for the proposed technique.

| Slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| rbaif_filter_type | 2 | u(2) |
| if (rbaif_filter_type == 0) { |  |  |
| } |  |  |
| else if (rbaif_filter_type == 1) { |  |  |
| rbaif_coding_mode[0] | 2 | u(2) |
| if (rbaif_coding_mode[0] != 0) { |  |  |
| rbaif_golomb_mode[0] | 2 | u(5) |
| } |  |  |
| rbaif_vertical_coeff_check_flag_luma[0] | 2 | u(1) |
| if (rbaif_vertical_coeff_check_flag_luma[0] == 0) { |  |  |
| for (i = 0; i < RbaifNumFilterCoeffQuarterLuma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_quarter_luma[0][i] | 2 | se(v) |
| } |  |  |
| for (i = 0; i < RbaifNumFilterCoeffHalfLuma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_half_luma[0][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| rbaif_horizontal_coeff_check_flag_luma[0] | 2 | u(1) |
| if (rbaif_horizontal_coeff_check_flag_luma[0] == 0) { |  |  |
| for (i = 0; i < RbaifNumFilterCoeffQuarterLuma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_quarter_luma[0][i] | 2 | se(v) |
| } |  |  |
| for (i = 0; i < RbaifNumFilterCoeffHalfLuma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_half_luma[0][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| for (j=0; j < RbaifChromaComponent; j++) { |  |  |
| rbaif_vertical_coeff_check_flag_chroma[0][j] | 2 | u(1) |
| if (rbaif_vertical_coeff_check_flag_chroma[0][j] == 0) { |  |  |
| for (i=0; i < RbaifNumFilterCoeffOneEighthChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_one_eighth_chroma[0][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffQuarterChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_quarter_chroma[0][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffThreeEighthsChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_three_eighths_chroma[0][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffHalfChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_half_chroma[0][j][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| rbaif_horizontal_coeff_check_flag_chroma[0][j] | 2 | u(1) |
| if (rbaif_horizontal_coeff_check_flag_chroma[0][j] == 0) { |  |  |
| for (i=0; i < RbaifNumFilterCoeffOneEighthChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_half_luma[0][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffQuarterChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_one_eighth_chroma[0][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffThreeEighthsChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_three_eighths_chroma[0][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffHalfChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_half_chroma[0][j][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| else if (rbaif_filter_type == 2) { |  |  |
| rbaif_num_regions | 2 | ue(v) |
| for (i = 0; i < RbaifNumLCUs – 1; i++) { |  |  |
| rbaif_segmentation_map[i] | 2 | ue(v) |
| } |  |  |
| for (k = 0; k < rbaif_num_regions; k ++ ) { |  |  |
| rbaif_coding_mode[k] | 2 | u(2) |
| if (rbaif_coding_mode[k] != 0) { |  |  |
| rbaif_golomb_mode[k] | 2 | u(5) |
| } |  |  |
| rbaif_vertical_coeff_check_flag_luma[k] |  |  |
| if (rbaif_horizontal_coeff_check_flag_luma[k] == 0) { |  |  |
| for (i = 0; i < RbaifNumFilterCoeffQuarterLuma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_quarter_luma[k][i] | 2 | se(v) |
| } |  |  |
| for (i = 0; i < RbaifNumFilterCoeffHalfLuma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_half_luma[k][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| rbaif_horizontal_coeff_check_flag_luma[k] |  |  |
| if (rbaif_horizontal_coeff_check_flag_luma[j] == 0) { |  |  |
| for (i = 0; i < RbaifNumFilterCoeffQuarterLuma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_quarter_luma[k][i] | 2 | se(v) |
| } |  |  |
| for (i = 0; i < RbaifNumFilterCoeffHalfLuma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_half[k][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| for (j=0; j < RbaifChromaComponent; j++) { |  |  |
| rbaif_vertical_coeff_check_flag_chroma[k][j] | 2 | u(1) |
| if (rbaif_vertical_coeff_check_flag_chroma[k][j] == 0) { |  |  |
| for (i=0; i < RbaifNumFilterCoeffOneEighthChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_one_eighth_chroma[k][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffQuarterChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_quarter_chroma[k][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffThreeEighthsChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_three_eighths_chroma[k][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffHalfChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_v_half_chroma[k][j][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| rbaif_horizontal_coeff_check_flag_chroma[k][j] | 2 | u(1) |
| if (rbaif_horizontal_coeff_check_flag_chroma[k][j] == 0) { |  |  |
| for (i=0; i < RbaifNumFilterCoeffOneEighthChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_half_luma[k][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffQuarterChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_one_eighth_chroma[k][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffThreeEighthsChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_three_eighths_chroma[k][j][i] | 2 | se(v) |
| } |  |  |
| for (i=0; i < RbaifNumFilterCoeffHalfChroma – 1; i++) { |  |  |
| rbaif_diff_filter_coefficients_h_half_chroma[k][j][i] | 2 | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

## Decoder description

### Fractional sample interpolation process for luminance

The fractional sample interpolation process for luminance is the same as described in the contribution [7].

Inputs to the decoding process of the proposal are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	a luma reference sample array refPicLXL,

–	a set of DCT-IF filter coefficients for luma,

–	an index rbaif_filter_type specifying the interpolation filter type that is used for the current picture,

–	an index rbaif_coding_mode specifying the entropy coding mode that is used for decoding the residual filter coefficients,

–	an index rbaif_golomb_mode specifying the shape parameter that determines Golomb codes,

–	a set of rbaif_diff_filter_coefficients specifying the difference values between the DCT-IF filter coefficients and the calculated AIF filter coefficients.

Output to this process is a predicted luma sample value predSampleLXL[ xL, yL ].

As described above, the objective is to allow the use of region adaptation. Therefore, the syntax in the slice header is also changed as follows to support the proposal. In the proposed decoder, the rbaif_filter_type is decoded to indicate which interpolation filter is applied to the current picture.

When the rbaif_filter_type is equal to 0, 8-tap DCT-IF adopted in HM4.0 is applied to the current picture and no additional flag is decoded. The picture is interpolated by using the 8-tap DCT-IF filter coefficients. The 8-tap DCT-IF filter coefficients are summarized as follows. The same filter set is used for both horizontal and vertical interpolation as described in the section 8.4.2.2.2.1 of Working Draft [7].

–	1/4 position: {1, 4, 10, 57, 19, 7, 3, 1}

–	1/2 position: {1, 4, 11, 40, 40, 11, 4, 1}

–	3/4 position: {1, 3, 7, 19, 57, 10, 4, 1}

When the rbaif_filter_type is equal to 1, the AIF technique is applied to the current picture. In the current implementation, the numbers of the filter coefficients are the same as those of 8-tap DCT-IF in HM4.0. Therefore, the RbaifNumFilterCoeffQuarterLuma which indicates the number of the filter coefficients for the quarter-pel interpolation is 8 and the RbaifNumFilterCoeffHalfLuma which indicates the number of the filter coefficients for the half-pel interpolation is 4. Since these values are shared between encoder and decoder, these two values are not embedded in the bitstream. Firstly, the rbaif_coding_mode is decoded to indicate which entropy coding is used for decoding the residual filter coefficients. If rbaif_coding_mode is equal to 0, CAVLC is used. Otherwise, the Golomb Codes is used and the rbaif_golomb_mode is also decoded to indicate which shape parameter is used for the Golomb Codes of the residual filter coefficients. Next, four parameters which indicate the residual filter coefficients are decoded:

- rbaif_diff_filter_coefficients_v_quarter_luma (for the vertical interpolation at the quarter-pel samples of luminance),

- rbaif_diff_filter_coefficients_v_half_luma (for the vertical interpolation at the half-pel samples of luminance),

- rbaif_diff_filter_coefficients_h_quarter_luma (for the horizontal interpolation at the quarter-pel samples of luminance),

- rbaif_diff_filter_coefficients_h_half_luma (for the horizontal interpolation at the half-pel samples of luminance).

AIF filter coefficients are reconstructed by adding the residual filter coefficients to the corresponding predicted filter coefficients. In this filter type, the filter coefficients are generated for both horizontal and vertical directions. The AIF filter coefficients are summarized as follows.

–	1/4 position (vertical): {Cvq1, Cvq2, Cvq3, Cvq4, Cvq5, Cvq6, Cvq7, Cvq8}

–	1/2 position (vertical): {Cvh1, Cvh2, Cvh3, Cvh4, Cvh4, Cvh3, Cvh2, Cvh1}

–	3/4 position (vertical): {Cvq8, Cvq7, Cvq6, Cvq5, Cvq4, Cvq3, Cvq2, Cvq1}

–	1/4 position (horizontal): {Chq1, Chq2, Chq3, Chq4, Chq5, Chq6, Chq7, Chq8}

–	1/2 position (horizontal): {Chh1, Chh2, Chh3, Chh4, Chh4, Chh3, Chh2, Chh1}

–	3/4 position (horizontal): {Chq8, Chq7, Chq6, Chq5, Chq4, Chq3, Chq2, Chq1}

As represented in Table 2, in the current implementation, Cvq8, Cvh1, Chq8 and Chh1 are not embedded in the bitstream and not decoded because of constraint conditions as follows:

i=18Cvqi=64,  i=14Cvhi=32, i=18Chqi=64, i=14Chhi=32.

Namely, the total number of luminance filter coefficients to be decoded is 20.

In the following explanation, integer and fractional samples are represented as shown in Figure 3. The positions labelled with upper-case letters Ai,j within shaded blocks represent luma samples at full-sample locations inside the given two-dimensional array refPicLXL of luma samples. These samples may be used for generating the predicted luma sample value predSampleLXL[ xL, yL ]. The locations ( xAi,j, yAi,j ) for each of the corresponding luma samples Ai,j inside the given array refPicLXL of luma samples are derived as follows:

xAi,j = Clip3( 0, PicWidthInSamplesL – 1, xIntL +i )

yAi,j = Clip3( 0, PicHeightInSamplesL – 1, yIntL +j )

Given the luma samples Ai,j at full-sample locations ( xAi,j, yAi,j ), the luma samples ‘a0,0’ to ‘r0,0’ at fractional sample positions are derived by the following rules. The interpolation proceeds in the same manner as described in the rbaif_filter_type equal to 0. The filter coefficients are replaced by the AIF filter coefficients for each interpolation direction. Note that variables shift1 is the same definitions described in Working Draft [7]. The variable shift1 is set equal to BitDepthY – 8, the variable shift2 is set equal to 6, and the variable shift3 is set equal to 14 – BitDepthY. The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the 8-tap filter to the nearest integer position samples as follows:

a0,0 = ( Chq1*A-3,0 + Chq2*A-2,0 + Chq3*A-1,0 + Chq4*A0,0 +	 Chq5*A1,0 + Chq6*A2,0 + Chq7*A3,0 + Chq8*A4,0 ) >> shift1	(1)

b0,0 = ( Chh1*A-3,0 + Chh2*A-2,0 + Chh3*A-1,0 + Chh4*A0,0 +	 Chh4*A1,0 + Chh3*A2,0 + Chh2*A3,0 + Chh1*A4,0 ) >> shift1	(2)

c0,0 = ( Chq8*A-3,0 + Chq7*A-2,0 + Chq6*A-1,0 + Chq5*A0,0 +	 Chq4*A1,0 + Chq3*A2,0 + Chq2*A3,0 + Chq1*A4,0 ) >> shift1	(3)

d0,0 = ( Cvq1*A0,-3 + Cvq2*A0,-2 + Cvq3*A0,-1 + Cvq4*A0,0 +	 Cvq5*A0,1 + Cvq6*A0,2 + Cvq7*A0,3 + Cvq8*A0,4 ) >> shift1	(4)

h0,0 = ( Cvh1*A0,-3 + Cvh2*A0,-2 + Cvh3*A0,-1 + Cvh4*A0,0 +	 Cvh4*A0,1 + Cvh3*A0,2 + Cvh2*A0,3 + Cvh1*A0,4 ) >> shift1	(5)

n0,0 = ( Cvq8*A0,-3 + Cvq7*A0,-2 + Cvq6*A0,-1 + Cvq5*A0,0 +	 Cvq4*A0,1 + Cvq3*A0,2 + Cvq2*A0,3 + Cvq1*A0,4 ) >> shift1	(6)

Figure 3: Integer samples (shaded blocks with upper-case letters) and fractional samples (un-shaded blocks with lower-case letters) for quarter sample luma interpolation.

The samples labelled e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by applying the 8-tap filter to the samples a0,i, b0,i and c0,i where i = −3..4 in vertical direction:

e0,0 = ( Cvq1*a0,-3 + Cvq2*a0,-2 + Cvq3*a0,-1 + Cvq4*a0,0 +	 Cvq5*a0,1 + Cvq6*a0,2 + Cvq7*a0,3 + Cvq8*a0,4 ) >> shift2	(7)

f0,0 = ( Cvq1*b0,-3 + Cvq2*b0,-2 + Cvq3*b0,-1 + Cvq4*b0,0 +	 Cvq5*b0,1 + Cvq6*b0,2 + Cvq7*b0,3 + Cvq8*b0,4 ) >> shift2	(8)

g0,0 = ( Cvq1*c0,-3 + Cvq2*c0,-2 + Cvq3*c0,-1 + Cvq4*c0,0 +	 Cvq5*c0,1 + Cvq6*c0,2 + Cvq7*c0,3 + Cvq8*c0,4 ) >> shift2	(9)

i0,0 = ( Cvh1*a0,-3 + Cvh2*a0,-2 + Cvh3*a0,-1 + Cvh4*a0,0 +	 Cvh4*a0,1 + Cvh3*a0,2 + Cvh2*a0,3 + Cvh1*a0,4 ) >> shift2	(10)

j0,0 = ( Cvh1*b0,-3 + Cvh2*b0,-2 + Cvh3*b0,-1 + Cvh4*b0,0 +	 Cvh4*b0,1 + Cvh3*b0,2 + Cvh2*b0,3 + Cvh1*b0,4 ) >> shift2	(11)

k0,0 = ( Cvh1*c0,-3 + Cvh2*c0,-2 + Cvh3*c0,-1 + Cvh4*c0,0 +	 Cvh4*c0,1 + Cvh3*c0,2 + Cvh2*c0,3 + Cvh1*c0,4 ) >> shift2	(12)

p0,0 = ( Cvq8*a0,-3 + Cvq7*a0,-2 + Cvq6*a0,-1 + Cvq5*a0,0 +	 Cvq4*a0,1 + Cvq3*a0,2 + Cvq2*a0,3 + Cvq1*a0,4 ) >> shift2	(13)

q0,0 = ( Cvq8*b0,-3 + Cvq7*b0,-2 + Cvq6*b0,-1 + Cvq5*b0,0 +	 Cvq4*b0,1 + Cvq3*b0,2 + Cvq2*b0,3 + Cvq1*b0,4 ) >> shift2	(14)

r0,0 = ( Cvq8*c0,-3 + Cvq7*c0,-2 + Cvq6*c0,-1 + Cvq5*c0,0 +	 Cvq4*c0,1 + Cvq3*c0,2 + Cvq2*c0,3 + Cvq1*c0,4 ) >> shift2	(15)

The positions labelled with lower-case letters within un-shaded blocks represent luma samples at quarter-pel sample fractional locations. The luma location offset in fractional-sample units ( xFracL, yFracL ) specifies which of the generated luma samples at full-sample and fractional-sample locations is assigned to the predicted luma sample value predSampleLXL[ xL, yL ]. This assignment is done according to Table 3. The value of predSampleLXL[ xL, yL ] shall be the output. Note that variable shift3 is the same definition described in Working Draft [7].

Table 3: Assignment of the luma prediction sample predSampleLXL[ xL, yL ].

| xFracL | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| predSampleLXL[ xL, yL ] | A << shift3 | d | h | n | a | e | i | p | b | f | j | q | c | G | k | r |

When the rbaif_filter_type is equal to 2, the current picture is segmented into LCU-based multiple regions. The rbaif_num_regions which indicates the number of the regions included in the current picture is decoded. In the current implementation, this filter type has two regions and the number is shared between encoder and decoder. Therefore, the rbaif_num_regions is not embedded in the bitstream and not decoded. The rbaif_golomb_mode and the rbaif_diff_filter_coefficients are also decoded for each region. The rbaif_segmentation_map which indicates the region number of each LCU is decoded. In Table 2, RbaifNumLCUs means the total number of LCUs included in one slice. The interpolation method using the AIF filter coefficients for each region is the same as described in rbaif_filter_type equal to 1.

### Fractional sample interpolation process for chrominance

Inputs to the decoding process of the proposed chrominance interpolation are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in fractional-sample units ( xFracC, yFracC ),

–	a chroma reference sample array refPicLXC,

–	a set of DCT-IF filter coefficients for chroma,

–	an index rbaif_filter_type specifying the interpolation filter type that is used for the current picture,

–	an index rbaif_golomb_mode specifying the shape parameter that determines Golomb codes,

–	a set of rbaif_diff_filter_coefficients specifying the difference values between the DCT-IF chroma filter coefficients and the calculated AIF chroma filter coefficients.

Output of this process is a predicted chroma sample value predSampleLXC[ xC, yC ]

The predicted chroma samples are interpolated in the same manner as the luma interpolation method described in 2.4.1. In the proposed decoder, the rbaif_filter_type is decoded to indicate which interpolation filter is applied to the current picture. The rbaif_filter_type is shared between luminance and chrominance.

When the rbaif_filter_type is equal to 0, 4-tap DCT-IF adopted in HM4.0 is applied to the current picture and no additional flag is decoded. The picture is interpolated by using the 4-tap DCT-IF filter coefficients. The DCT-IF filter coefficients are summarized as follows. The same filter set is used for both horizontal and vertical interpolation as described in the section 8.4.2.2.2.2 of Working Draft [8].

–	1/8 position: {3, 60, , 1}

–	1/4 position: {4, , 16, 2}

–	3/8 position: {5, 46, 27, 4}

–	1/2 position: {4, 36, 36, 4}

–	5/8 position: {4, 27, 46, 5}

–	3/4 position: {2, 16, 54, 4}

–	7/8 position: {1, 8, 60, 3}

When the rbaif_filter_type is equal to 1, the AIF technique is applied to the current picture. In the current implementation, the numbers of the filter coefficients are the same as those of 4-tap DCT-IF in HM4.0. Therefore, the RbaifNumFilterCoeffOneEighthChroma which indicates the number of the filter coefficients for the one-eighth-pel interpolation of chroma is 4. The RbaifNumFilterCoeffQuarterChroma which indicates the number of the filter coefficients for the quarter-pel interpolation of chroma is 4. The RbaifNumFilterCoeffThreeEighthsChroma which indicates the number of the filter coefficients for the three-eighths-pel interpolation of chroma is 4. The RbaifNumFilterCoeffHalfChroma which indicates for the half-pel interpolation of chroma is 2. Since these values are shared between encoder and decoder, these four values are not embedded in the bitstream. Firstly, the rbaif_golomb_mode is decoded to indicate which shape parameter is used for the Golomb Codes of the residual filter coefficients. Next, eight flags which indicate the residual filter coefficients are decoded:

- rbaif_diff_filter_coefficients_v_one_eighth_chroma (for the vertical interpolation at the one-eighth-pel samples of chroma),

- rbaif_diff_filter_coefficients_v_quarter_chroma (for the vertical interpolation at the quarter-pel samples of chroma),

- rbaif_diff_filter_coefficients_v_three_eighths_chroma (for the vertical interpolation at the three-eighths-pel samples of chroma),

- rbaif_diff_filter_coefficients_v_half_chroma (for the vertical interpolation at the half-pel samples of chroma),

- rbaif_diff_filter_coefficients_h_one_eighth_chroma (for the horizontal interpolation at the one-eighth-pel samples of chroma),

- rbaif_diff_filter_coefficients_h_quarter_chroma (for the horizontal interpolation at the quarter-pel samples of chroma),

- rbaif_diff_filter_coefficients_h_three_eighths_chroma (for the horizontal interpolation at the three-eighths-pel samples of chroma),

- rbaif_diff_filter_coefficients_h_half_chroma (for the horizontal interpolation at the half-pel samples of chroma).

AIF filter coefficients are reconstructed by adding the residual filter coefficients to the corresponding DCT-IF filter coefficients. In this filter type, the filter coefficients are generated for both horizontal and vertical directions. The AIF filter coefficients are summarized as follows.

–	1/8 position: {Cvoe1, Cvoe2, Cvoe3, Cvoe4} (vertical), 	{Choe1, Choe2, Choe3, Choe4} (horizontal)

–	1/4 position: {Cvq1, Cvq2, Cvq3, Cvq4} (vertical), 	{Chq1, Chq2, Chq3, Chq4} (horizontal)

–	3/8 position: {Cvte1, Cvte2, Cvte3, Cvte4} (vertical),	{Chte1, Chte2, Chte3, Chte4} (horizontal)

–	1/2 position: {Cvh1, Cvh2, Cvh2, Cvh1} (vertical), 	{Chh1, Chh2, Chh2, Chh1} (horizontal)

–	5/8 position: {Cvte4, Cvte3, Cvte2, Cvte1} (vertical), 	{Chte4, Chte3, Chte2, Chte1} (horizontal)

–	3/4 position: {Cvq4, Cvq3, Cvq2, Cvq1} (vertical), 	{Chq4, Chq3, Chq2, Chq1} (horizontal)

–	7/8 position: {Cvoe4, Cvoe3, Cvoe2, Cvoe1} (vertical), 	{Choe4, Choe3, Choe2, Choe1} (horizontal)

In the current implementation, Cvoe4, Choe4, Cvq4, Chq4, Cvte4, Chte4, Cvh1 and Chh1 are not embedded in the bitstream and not decoded because of constraint conditions as follows:

i=18Cvoei=i=18Cvqi=i=18Cvtei=i=18Choei=i=18Chqi=i=18Chtei=64,   i=14Cvhi=i=14Chhi=32.

Namely, the total number of chrominance filter coefficients to be decoded is 40 (20 for Cb and 20 for Cr).

In the following explanation, integer and fractional samples are represented as shown in Figure 4. In Figure 4, the positions labelled with upper-case letters Bi, j within shaded blocks represent chroma samples at full-sample locations inside the given two-dimensional array refPicLXC of chroma samples. These samples may be used for generating the predicted chroma sample value predSampleLXC[ xC, yC ]. The locations ( xBi, j, yBi, j ) for each of the corresponding chroma samples Bi, j inside the given array refPicLXC of chroma samples are derived as follows:

xBi, j = Clip3( 0, PicWidthInSamplesC – 1, xIntC +i )		
yBi, j = Clip3( 0, PicHeightInSamplesC – 1, yIntC +j )

Figure 4: Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation.

Given the chroma samples Bi, j at full-sample locations ( xBi, j, yBi, j ), the chroma samples ‘ab0,0’ to ‘hh0,0’ at fractional sample positions are derived by the following rules. The interpolation proceeds in the same manner as described in the rbaif_filter_type equal to 0. The filter coefficients are replaced by the AIF filter coefficients for each interpolation direction. Note that variables shift1 is the same definitions described in Working Draft [8]. Thus, the variable shift1 is set equal to BitDepthY – 8, the variable shift2 is set equal to 6, and the variable shift3 is set equal to 14 – BitDepthY. The samples labelled ab0,0, ac0,0, ad0,0, ae0,0, af0,0, ag0,0, and ah0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ab0,0 = ( Choe1*B−1,0 + Choe2*B0,0 + Choe3*B1,0 + Choe4*B2,0 ) >> shift1	(16)

ac0,0 = ( Chq1*B−1,0 + Chq2*B0,0 + Chq3*B1,0 + Chq4*B2,0 ) >> shift1	(17)

ad0,0 = ( Chte1*B−1,0 + Chte2*B0,0 + Chte3*B1,0 + Chte4*B2,0 ) >> shift1	(18)

ae0,0 = ( Chh1*B−1,0 + Chh2*B0,0 + Chh2*B1,0 + Chh1*B2,0 ) >> shift1	(19)

af0,0 = ( Chte4*B−1,0 + Chte3*B0,0 + Chte2*B1,0 + Chte1*B2,0 ) >> shift1	(20)

ag0,0 = ( Chq4*B−1,0 + Chq3*B0,0 + Chq2*B1,0 + Chq1*B2,0 ) >> shift1	(21)

ah0,0 = ( Choe4*B−1,0 + Choe3*B0,0 + Choe2*B1,0 + Choe1*B2,0 ) >> shift1	(22)

The samples labelled ba0,0, ca0,0, da0,0, ea0,0, fa0,0, ga0,0, and ha0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ba0,0 = ( Cvoe1*B0,−1 + Cvoe2*B0,0 + Cvoe3*B0,1 + Cvoe4*B0,2 ) >> shift1	(23)

ca0,0 = ( Cvq1*B0,−1 + Cvq2*B0,0 + Cvq3*B0,1 + Cvq4*B0,2 ) >> shift1	(24)

da0,0 = ( Cvte1*B0,−1 + Cvte2*B0,0 + Cvte3*B0,1 + Cvte4*B0,2 ) >> shift1	(25)

ea0,0 = ( Cvh1*B0,−1 + Cvh2*B0,0 + Cvh2*B0,1 + Cvh1*B0,2 ) >> shift1	(26)

fa0,0 = ( Cvte4*B0,−1 + Cvte3*B0,0 + Cvte2*B0,1 + Cvte1*B0,2 ) >> shift1	(27)

ga0,0 = ( Cvq4*B0,−1 + Cvq3*B0,0 + Cvq2*B0,1 + Cvq1*B0,2 ) >> shift1	(28)

ha0,0 = ( Cvoe4*B0,−1 + Cvoe3*B0,0 + Cvoe2*B0,1 + Cvoe1*B0,2 ) >> shift1	(29)

The samples labelled bX0,0, cX0,0, dX0,0, eX0,0, fX0,0, gX0,0 and hX0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by applying the 4-tap filter to the intermediate values aX0,i where i = −1..2 in vertical direction:

bX0,0 = ( Cvoe1*aX0,−1 + Cvoe2*aX0,0 + Cvoe3*aX0,1 + Cvoe4*aX0,2 ) >> shift2	(30)

cX0,0 = ( Cvq1*aX0,−1 + Cvq2*aX0,0 + Cvq3*aX0,1 + Cvq4*aX0,2 ) >> shift2	(31)

dX0,0 = ( Cvte1*aX0,−1 + Cvte2*aX0,0 + Cvte3*aX0,1 + Cvte4*aX0,2 ) >> shift2	(32)

eX0,0 = ( Cvh1*aX0,−1 + Cvh2*aX0,0 + Cvh2*aX0,1 + Cvh1*aX0,2 ) >> shift2	(33)

fX0,0 = ( Cvte4*aX0,−1 + Cvte3*aX0,0 + Cvte2*aX0,1 + Cvte1*aX0,2 ) >> shift2	(34)

gX0,0 = ( Cvq4*aX0,−1 + Cvq3*aX0,0 + Cvq2*aX0,1 + Cvq1*aX0,2 ) >> shift2	(35)

hX0,0 = ( Cvoe4*aX0,−1 + Cvoe3*aX0,0 + Cvoe2*aX0,1 + Cvoe1*aX0,2 ) >> shift2	(36)

The positions labelled with lower-case letters within un-shaded blocks represent chroma samples at eighth-pel sample fractional locations. The chroma location offset in fractional-sample units ( xFracC, yFracC ) specifies which of the generated chroma samples at full-sample and fractional-sample locations is assigned to the predicted chroma sample value predSampleLXC[ xC, yC ]. This assignment is done according to Error: Reference source not found. The value of predSampleLXC[ xC, yC ] shall be the output.

Table 4: Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively

| xFracC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | B << shift3 | ba | ca | da | ea | fa | ga | ha | aY | bY | cY | dY | eY | fY | gY | hY |

When the rbaif_filter_type is equal to 2, the current picture is segmented into LCU-based multiple regions. As described in Sec. 2.4.1, the decoded rbaif_golomb_mode is used for chrominance as well as luminance. The chroma filter coefficients such as rbaif_diff_filter_coefficients_v_one_eighth_chroma are also decoded for each region. The rbaif_segmentation_map is shared between luminance and chrominance. The interpolation method using the AIF filter coefficients for each region is the same as described in rbaif_filter_type equal to 1.

# Experimental results

The proposal has been implemented in HM 4.0. It is noted that the proposed method is applied to chrominance as well as luminance. The configurations of RBAIF are subject to the following conditions:

- Filter characteristics: separable filter

- Tap length: 8 taps for luminance and 4 taps for chrominance (Both are 6-bit representation.)

- Algorithm to generate filter coefficients of each region: that of SAIF [2]

- RBAIF settings are as follows.

  - Number of regions: n=2

  - Initial interpolation filter:

    - Half-pel: =0 (index 0), =0.08 (index 1)

    - Quarter-pel: =0 (index 0), =0.04 (index 1)

  - Filter coefficient prediction setting: (only luminance, magnified 256 times)

    - Half-pel: {h4=10031}, {h3= 297, h3=9097}, {h2=234, 'h2=301 and h2=7128}

    - Quarter-pel: {q4=14369}, {q5=152, q5=13259}, {q3=232, 'q3=150, q3=13663},

{q6=67, 'q6=139, ''q6=196, q6=9225},

{q2=244, 'q2=128, ''q2=105, '''q2=34, q2=7645},

{q7=39, 'q7=40, ''q7=86, '''q7=108,''''q7=220, q7=5662},

{q1=170, 'q1=101, ''q1=91, '''q1=82,''''q1=70,'''''q1=35, q1=5726}

The other configuration is subject to the common test condition [9].

The objective performance is shown in Table 4 and Table 5. The gains (BD-rate [10]) against the anchor of HM 4.0 were calculated. In the HE configuration, the average gains were about 0.1 %, 0.01% and 0.1 % for RA, LB and LP, respectively. In the LC configuration, those were 0.3 %, 0.5% and 2.0 % for RA, LB and LP, respectively. The maximum gain was about 10.2 % for the sequence “Vidyo3” in LP-LC case. The maximum coding gains of Cb and Cr were about 7.1% and 5.7% for the sequence “Nebuta” in RA-LC case, respectively. It is confirmed that the proposal is effective for high resolutions such as Class A, B and E, especially in the LC configuration.

Regarding the computational complexity, the encoding and decoding time were about 101.42% and 102.51% against the anchor on average, respectively. The number of addition and multiplication of the proposal were almost the same as those of HM4.0. Memory bandwidth of 1D and 2D for the worst case were increased by about 4% on average compared to those of HM4.0.

To confirm the performance of the modified prediction method for the luma filter coefficients, the simulation was done without the chroma interpolation. In this case, the syntax and the decoder description are based on the contribution [7]. The objective performance without the chroma interpolation is shown in Table 6 and Table 7. The gains against the anchor of HM 4.0 were calculated. In the HE configuration, the average gains were about 0.05 %, 0.03% and 0.1 % for RA, LB and LP, respectively. In the LC configuration, those were 0.4 %, 0.6% and 2.2 % for RA, LB and LP, respectively. The maximum gain was about 10.3 % for the sequence “Vidyo3” in LP-LC case.

Regarding the computational complexity without the chroma interpolation, the encoding and decoding time were about 101.02% and 102.14% against the anchor on average, respectively. The number of addition and multiplication of the proposal were almost the same as those of HM4.0. Memory bandwidth of 1D and 2D for the worst case were increased by about 5% on average compared to those of HM4.0.

Table 4: Simulation results for each GOP structure (Chroma on).

Table 5: Summary of objective performance (Chroma on).

Table 6: Summary of objective performance (Chroma off).

Table 7: Simulation results for each GOP structure (Chroma off).

# Conclusion

In this contribution, an LCU-based AIF technique was extended to chrominance, and the modification of the prediction method for filter coefficients was introduced. Overall average coding gain for luminance signal (Y), chrominance signals (Cb and Cr) were about 0.49%, 0.38% and 0.37%, respectively. The average encoding and decoding run-time increment against the anchor were about 1.42% and 2.51%, respectively. When the chroma interpolation was off, overall average coding gain for luminance signal was about 0.56%. The average encoding and decoding run-time increment against the anchor were about 1.02% and 2.14%, respectively. It is confirmed that the proposed interpolation filter was effective for chrominance as well as luminance.

It is recommended to incorporate the proposal into HEVC as interpolation method.

# References

[1] Y. Vatis, B. Edler, D. T. Nguyen and J. Osterman, “Two-dimensional non-separable adaptive wiener interpolation filter for H.264/AVC,” ITU-T SG16 Q.6 Document, VCEG-Z17r1, 26th VCEG Meeting, Busan, Korea, April 2005.

[2] S. Wittmann and T. Wedi, “Separable adaptive interpolation filter for video coding,” in Proc. International Conference on Image Processing (ICIP) 2008, San Diego, California, USA, October 2008.

[3] S. Matsuo, Y. Bandoh, S. Takamura and H. Jozawa, “Region-based adaptive interpolation filter,” JCTVC-B051, 2nd. JCT-VC Meeting, Geneva, CH, 21-28 July, 2010.

[4] S. Matsuo, Y. Bandoh, S. Takamura and H. Jozawa, “Response to CE3: Region-based adaptive interpolation filter,” JCTVC-D150, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[5] S. Matsuo, Y. Bandoh, T. Ito, S. Takamura and H. Jozawa, “CE3: Region-based adaptive interpolation filter,” JCTVC-E078, 5th. JCT-VC Meeting, Geneva, CH, 16-23 March, 2011.

[6] S. Matsuo, Y. Bandoh, T. Ito, S. Takamura and H. Jozawa, “CE3: Modifications of region-based adaptive interpolation filter,” JCTVC-F048, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

[7] S. Matsuo, S. Takamura and H. Jozawa, “CE3: LCU-based adaptive interpolation filter,” JCTVC-G258, 7th. JCT-VC Meeting, Geneva, CH, 21-30 November, 2011.

[8] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803_d5, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

[9] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

[10] G. Bjontegaard, “Calculation of average PSNR differences between RD-curves,” ITU-T SG16 Q.6 Document, VCEG-M33, 13th VCEG Meeting, Austin, Texas, USA, April 2001.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).