| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G263 |
| --- | --- |

| Title: | Hardware complexity analysis of spatial transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiang Tian, Altera Corporation Russel James, Altera Corporation | Tel: Email: | xtian@altera.com rjames@altera.com |
| Source: | Altera Corporation |  |  |

_____________________________

# Abstract

In this report, a hardware complexity analysis of two HEVC spatial transform algorithms is described, including HM4 and F251. All transform size (4pt up to 32pt), and both forward and inverse transform have been implemented using an Altera Stratix V FPGA. Results are reported with logic utilization, Fmax, and the number of pipeline stages.

# Introduction

Two spatial transform methods are chosen to do hardware complexity analysis, including HM4.0 method [1] and F251 proposal [2].

# Test methods and conditions

Below are the methods and conditions used in the test.

- Partial butterfly is chosen for HM4 method, and full factorization is chosen for F251.

- RTL are generated automatically using C-RTL tool.

- C codes are from source code in HM4.0 and F251 proposal package.

- The tool can do optimization of the C input, such as unroll.

- The RTL of 32pt forward and inverse transforms in HM4 and F251 have been tested using large amount of random sequences, and the output are identical to the results from C codes. Smaller transforms results have been tested with a couple of data sets, and all are correct.

- The RTL have been synthesized using Quartus II and Stratix V. The resources consumption is reported using logic utilization count (one logic utilization is a six input LUT plus two registers). The clock frequency is reported as Fmax, under different temperature conditions.

- No hardware multiplier (i.e DSP block) is used during synthesis, so only the logic count can reflect the real resources consumption.

- The numbers of pipeline stages are decided by the tool. The tool inserts as many as possible pipeline stages to achieve the maximal performance.

# Results

Table 1. Synthesis results of forward transform

| DCT Algorithm | Size | Logic Utilization | Fmax (MHz) 85C/0C | Latency (pipeline stage) |
| --- | --- | --- | --- | --- |
| HM4 | 32 | 31917 | 229/261 | 22 |
|  | 16 | 9411 | 273/309 | 13 |
|  | 8 | 2474 | 351/401 | 9 |
|  | 4 | 687 | 494/571 | 6 |
| F251 | 32 | 13394 | 264/293 | 15 |
|  | 16 | 4746 | 302/343 | 13 |
|  | 8 | 1715 | 356/406 | 10 |
|  | 4 | 702 | 438/503 | 7 |

Table 2. Synthesis results of inverse transform

| IDCT Algorithm | Size | Logic Utilization | Fmax (MHz) 85C/0C | Latency (pipeline stage) |
| --- | --- | --- | --- | --- |
| HM4 | 32 | 26327 | 223/252 | 22 |
|  | 16 | 7236 | 277/314 | 12 |
|  | 8 | 2137 | 370/422 | 8 |
|  | 4 | 775 | 489/556 | 7 |
| F251 | 32 | 12877 | 276/314 | 15 |
|  | 16 | 4572 | 322/364 | 12 |
|  | 8 | 1674 | 352/398 | 9 |
|  | 4 | 549 | 521/590 | 7 |

# References

- A. Fuldseth, G. Bjøntegaard, M. Sadafale, and M. Budagavi, “Transform design for HEVC with 16 bit intermediate data representation,” JCTVC-E243, Geneva, March, 2011.

- E.Alshina, A.Alshin, Il-Koo Kim, and Pankaj Topiwala “CE10: Full-factorized core transform proposal by Samsung/FastVDO.,” JCTVC-F251, Torino, July, 2011.

# Patent rights declaration(s)

Altera does not have any current or pending patent rights relating to the technology described in this contribution.