| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G264 |
| --- | --- |

| Title: | AHG18: Adaptive Resolution Coding (ARC) |  |  |
| --- | --- | --- | --- |
| Status: | JCTVC Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Davies 14 Waterside Drive Langley, SL3 6EZ, UK P. Topiwala 5840 Banneker Rd., #230 Columbia, MD 21044 USA | Email: | thdavies@cisco.com pankajtva@gmail.com |
| Source: | Cisco Systems, FastVDO LLC |  |  |

_____________________________

# Abstract

This contribution reports the results of further investigations into resolution adaption. Adaptive Resolution Coding (ARC) is described, in which resolution is selected dynamically by means of a threshold test, and the compression performance investigated in a rapid bit-rate reduction/high QP scenario against simply increasing QP with and without pre-filtering. Mean luma BD-Rate gains range from 6.9-14.3% in Class A and 3.5-11.5% in Class B but performance depends greatly on picture content and configuration. Individual luma BD-rate gains can range from negligible up to 30%. Typically chroma has lower objective quality by a fixed penalty of around 0.5dB but this is asserted to have little subjective impact. The contribution includes modified Working Draft text.

# Introduction

This contribution provides the results of further investigations of the utility of adapting resolution within a single stream without introducing an IDR frame. This concept was originally part of H.263 Annex P [1] , and was introduced in the context of HEVC by JCTVC-F158 [2]. This contribution is intended to answer the mandates of AHG 18 on Resolution Adaption [3]. In particular, the concept of Adaptive Resolution Change (ARC) – adapting resolution dynamically in response to circumstance – is presented, and the compression performance investigated in a rapid bit-rate reduction scenario. We hypothesize a dramatic channel capacity drop (of 70%), with the intent of providing continuous streaming. Such a significant data rate variation is unfortunately quite common in network video applications, whether over Internet or on cellular networks. Such conditions predicate significant adaptation to continue streaming.

An important part of our contribution is a specific test architecture for dealing with the above scenario, including utilizing a lower resolution native resolution for coding and streaming. As part of this contribution a pair of co-designed down- and up-sampling filters are proposed for optimal performance.

ARC is compared against other methods of reducing bit rate, specifically increasing QP with or without prefiltering. In this context it is found that results depend on picture characteristics, but for some pictures significant objective gains (up to 25% in luma BD-Rate) can be found. It is also claimed that subjectively, although softer, pictures have lower levels of artifacts in many cases. Chroma performance can be reduced by a fixed penalty of around 0.5dB, but this does not appear particularly noticeable subjectively given the reduced luma resolution.

This contribution also includes modified WD [4] text.

# Background and applications

As the reference to H.263 implies, the concept of changing resolutions mid-stream is not new. There are a number of reasons to do this:

- to provide a significant reduction in complexity (for example if a decoder is overloaded);

- to adapt the resolution seamlessly as a display window is re-sized in a decoder application;

- to provide a fast start in low-delay applications; or

- to provide dramatic adjustments in bit rate, for example when a network becomes congested.

It is possible to change resolution within the context of AVC coding by inserting a new SPS and IDR frame. However in many applications, but particularly video communications where delay must be minimized, an IDR causes problems. [2] investigated the impact of inserting an IDR, especially if its quality is constrained to reduce the impact on delay, compared with predicting across the resolution change. It reported significant impacts on quality, with average gains of 6.9-7.4% for cross-resolution prediction versus IDR insertion. Subjective impacts of an IDR in a low-delay scenario are very great, since it must be coded with very low quality to maintain a comparable size to other frames. This causes a noticeable “blip” in quality.

There are clear benefits in terms of managing complexity in changing resolution dynamically, and it is also clear from [2] that if resolution is to be changed then a predictive change provides gains.

However, the focus of this contribution is on point 4 above i.e. whether there are also potential coding gains from switching resolution to reduce bit rate significantly. After all, it is to be expected that HEVC offers a wider coding range for a given picture size than previous standards due to its large range of block sizes.

One aspect of coding at low resolution, which influences judgements of respective quality, is that “super-resolution” coding techniques, using adaptive upsampling filters can be used. This is particularly pertinent in video conferencing infrastructure where scaling and transcoding are routine to match resolutions and bit rate: it is very that video with go through a resampling chain before viewing.

# Approach

Typically, it is found that the Rate-Distortion curves of a low-resolution and a high resolution version of the same sequence will cross at some point. Where this point is depends greatly on the content, and to a lesser extent on the prediction structure.

The approach taken is therefore as follows:

- Determine cross-over points for R-D curves for low-resolution coding versus high resolution anchors

- Devise an approximate rule for selecting low-resolution coding to implement ARC

- Compare ARC with this rule versus the anchors

The purpose of this proposal is not to propose an encoder algorithm for switching resolution, but to demonstrate that ARC can provide gains given a suitable method. Evidently, simultaneous coding of both resolutions would provide the best results, but would forfeit complexity gains. Full spreadsheets for both ARC and the static switch used to determine A) are provided.

Comparing quality between resolutions is notoriously difficult. It is likely that switch points chosen on a subjective basis would be different, and depend on the likely viewing conditions and audience also.

# Implementation

Simulations have been performed using a modified version of HM.4.0, in which pictures may have both low and high-resolution copies.

## Filters

A pair of linear up- and down-sampling filters from [5,9] are used as the re-sampling filters. This pair of filters is co-designed to work best in concert, providing improved energy compaction and reconstruction quality.

Filter h11 from [5] is used for reference-frame down-sampling, with taps:

[1  0  -3  0  10  16  10  0  -3  0  1] / 32

The matching up-sampling filter is f7 from [5,9] and has taps:

[-1     0     5     8     5     0    -1] / 8

The frequency responses and design details can be found in [5,9] and [8].

In ARC the filtering process applies normatively only to generating reference frames for prediction when the reference resolution is different from the current picture resolution. As proposed here, it does not normatively apply to generating high-resolution from low-resolution for display or low-resolution from high resolution for coding. Nevertheless, in some applications the built-in filters may very likely be used for these purposes for simplicity. It is very important, therefore, that up- and down-sampling processes are co-designed for best performance.

## Resolution selection method

A simple method for resolution selection is used. Prior to the switch point, video is encoded at full resolution. PSNR[0] is the measured PSNR at full resolution. PSNR[1] is the measured PSNR of the video at lower resolution i.e. comparing coded and down-sampled video with the down-sampled original. When these values are close, it indicates that low-resolution coding may be beneficial. Hence a threshold T is set so that if:

PSNR[1] – PSNR[0] < T

for a continuous set of frames (at least 4) just before the switch point, then low-resolution coding is used. Otherwise QP increase is used. The threshold T is dependent on resolution:

T=0.7 + picture_height / 900

Therefore resolution switch is less likely at lower resolution (as T is smaller) and more likely at high resolution.(In fact low resolution coding is not selected by this method for classes C and D even though it would sometimes  be beneficial).

## Non-linear adaptive up-sampling

Decoding a sub-sampled resolution of the original resolution is substantially less complex (e.g., about 3X faster 2-to-1 subsampling). Therefore it is also possible that optimized “super-resolution” up-sampling technologies can be used to provide coding efficiency benefits – especially subjectively – whilst still having less complexity than decoding the full resolution. Results with one such filter, a much-simplified version of [7], are therefore also presented.

The non-linear up-sampler is intended to be illustrative of what is possible in decoder applications. It interpolates samples in two stages. High resolution pixels in positions (2x, 2y) are copied from low-resolution pixels in positions (x,y). In the first phase, pixels values in positions (2x+1, 2y+1) are estimated in the two diagonal directions (Figure 1) with 4-tap filters.

Figure 1: First interpolation stage

The filters in each direction vary according to a quantised feature vector computed from the differences d0, d1, d2 computed along the diagonal. The differences are quantized to 3 bits on a log scale, producing a 9 bit value (reduceable to 8 bits by symmetry).

The two estimates are combined according to another 6-bit feature vector, calculated from the two diagonal differences.

In the second stage, pixel values in positions (2x, 2y+1) and (2x+1, 2y) are interpolated in a very similar way, but using vertical and horizontal filters and differences (Figure 2). The same filters are used in both cases by symmetry.

Figure 2: Second interpolation stage

The filter taps were tuned with short sequences of uncoded frames taken from classes A, B and E.

# Experiments

## Test scenario

In each test a network crisis is simulated as in [2], in which bit rate must be substantially reduced: a number of frames (5) are deemed lost and are zeroed, and QP is either increased (with or without prefiltering) or resolution is changed. In practice, rather more frames might be lost, or, if a bit rate reduction is planned, none.

## Configurations

All simulations use modified Common Conditions [7], excluding All-Intra. In each case the base QPs are set as: 32, 35, 39, 42. This QP range places bit rates in a range more typical for streaming or video communications applications, especially in stressed circumstances.

For low-resolution coding, these QPs are used throughout the sequence.

There are two anchors.

Anchor 1 consists of coding full resolution video up to the switch frame, from which QP is increased by 6. Anchor 2 is the same except the video is half-band prefiltered whilst having the same QP increase.

The switch frame in all cases is Frame 21 in coding order.

## Half-band anchor prefilter

The prefilter used is the half-band 11-tap Lanczos filter:

[1  0  -5  0  20  32  20  0  -5  0  1] / 64

## Adaptive Resolution Coding tests

At the switch point a low-resolution coding (using the same QP) or high resolution coding (using QP+6) is adaptively selected. The following four simulation sets are presented:

|  | Anchor | Tested |
| --- | --- | --- |
| Test A | HM4.0 + QP increase of 6 at switch point | ARC + 1:2 linear up-sampling |
| Test B | HM4.0 + QP increase of 6 at switch point | ARC + 1:2 adaptive up-sampling |
| Test C | HM4.0 + QP increase of 6 and half-band prefiltering at switch point | ARC + 1:2 linear up-sampling |
| Test D | HM4.0 + QP increase of 6 and half-band prefiltering at switch point | ARC + 1:2 adaptive up-sampling |

# Results and performance analysis

The following sub-sections show summary comparisons for the four test cases. Full R-D data are in subdirectory ARC simulations. BD-Rate and timing data have been gathered only for the portion after the switch point, so that effects are not diluted.

Mean results show approximately 7%-14% gain for class A and 3.5%-11% gain for class B against the QP-increase anchor. These average results disguise very large variations between individual sequences: SteamLocomotive in RA_LC configuration shows a 29.9% luma BD-Rate gain. Generally LC configurations show higher gain.

Note that chroma figures are not at all reliable. In some cases the chroma curves do not overlap as the curves are nearly flat and these is a constant fractional dB offset. However, it is clear that there are very large luma gains for some sequences (as much as 25%). Chroma losses appear not to be visible subjectively.

The objective gains against a pre-filtered anchor are very large: 10-14% over all classes. This is because the prefiltered video R-D curve almost never lies above the non-prefiltered anchor: prefiltering provides no objective Rate-Distortion benefit.Hence the fact that ARC is switching between QP-increase and low-resolution contributes to the objective gain..

## Crossover points

Additional data showing the comparison of a static resolution switch against the two anchors are included in subdirectory Resolution-R-D-cross-over for information. This demonstrates the content dependencies of coding at different resolutions. It also shows that cross-over points are higher against the prefiltered anchor, and therefore that some contribution to the additional gain is coming from low-resolution coding.

For example, low-resolution coding of Kimono shows significant and consistent luma BD-Rate gains:

Figure 3: Kimono, RA_HE configuration, low resolution, linear upsampling (red) versus full resolution (black)

By contrast, Traffic shows a cross-over:

Figure 4: Traffic, RA_HE configuration, low resolution, linear upsampling (red) versus full resolution (black)

BQTerrace in RA_HE configuration does not have a cross-over within the range of testing:

Figure 5: BQTerrace, RA_HE configuration, low resolution, linear upsampling (red) versus full resolution (black)

## HM4.0+QP increase versus ARC

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -7.0% | 74.1% | 67.8% | -14.3% | 76.5% | 82.9% |
| Class B | -4.3% | 23.5% | 26.5% | -11.4% | 13.4% | 18.5% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -2.9% | 24.3% | 23.8% | -6.7% | 21.9% | 24.9% |
|  | -2.9% | 78.5% | 61.3% | -6.7% | 74.4% | 62.5% |
| Enc Time[%] | 69% |  |  | 69% |  |  |
| Dec Time[%] | 74% |  |  | 73% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.5% | 36.5% | 35.4% | -11.0% | 25.2% | 21.9% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.5% | 11.4% | 9.4% | -6.8% | 6.9% | 3.8% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.2% | 13.5% | 12.8% | -4.7% | 9.2% | 7.5% |
|  | -1.2% | 13.8% | 12.8% | -4.7% | 9.3% | 7.2% |
| Enc Time[%] | 79% |  |  | 84% |  |  |
| Dec Time[%] | 83% |  |  | 83% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.5% | 36.5% | 28.9% | -11.5% | 23.8% | 20.8% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.8% | 10.7% | 7.8% | -6.9% | 6.9% | 3.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.3% | 13.4% | 10.5% | -4.9% | 8.7% | 7.2% |
|  | -1.2% | 13.7% | 9.9% | -4.9% | 8.3% | 6.8% |
| Enc Time[%] | 86% |  |  | 84% |  |  |
| Dec Time[%] | 83% |  |  | 82% |  |  |

## HM4.0+QP increase versus ARC+adaptive up-sampling

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -6.9% | 76.6% | 69.8% | -13.9% | 81.7% | 89.2% |
| Class B | -4.5% | 25.1% | 27.2% | -11.5% | 15.6% | 20.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -2.9% | 25.4% | 24.4% | -6.7% | 23.8% | 26.9% |
|  | -2.9% | 81.4% | 63.7% | -6.6% | 80.9% | 67.7% |
| Enc Time[%] | 77% |  |  | 78% |  |  |
| Dec Time[%] | 74% |  |  | 73% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.6% | 37.7% | 36.4% | -11.0% | 27.3% | 23.7% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -1.7% | 11.8% | 9.9% | -7.6% | 7.7% | 4.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.5% | 14.0% | 13.2% | -4.9% | 10.0% | 8.3% |
|  | -1.4% | 14.3% | 13.3% | -4.9% | 10.2% | 8.0% |
| Enc Time[%] | 87% |  |  | 79% |  |  |
| Dec Time[%] | 83% |  |  | 82% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.6% | 37.6% | 29.8% | -11.6% | 25.9% | 22.5% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -2.1% | 11.0% | 8.3% | -7.8% | 7.7% | 4.6% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.5% | 13.8% | 10.9% | -5.1% | 9.5% | 7.9% |
|  | -1.5% | 14.2% | 10.2% | -5.1% | 9.1% | 7.5% |
| Enc Time[%] | 80% |  |  | 79% |  |  |
| Dec Time[%] | 83% |  |  | 82% |  |  |

## HM4.0+half-band prefiltering and QP increase versus ARC

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -8.6% | 189.9% | 153.3% | -14.5% | 72.7% | 159.3% |
| Class B | -12.7% | 26.2% | 28.4% | -12.9% | 20.7% | 27.6% |
| Class C | -13.4% | -2.6% | 2.2% | -11.7% | -3.5% | 1.5% |
| Class D | -12.6% | 8.1% | 5.8% | -10.7% | 5.7% | 1.1% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -11.9% | 53.7% | 46.3% | -12.5% | 23.7% | 46.2% |
|  | -11.9% | 53.9% | 47.3% | -12.4% | 56.8% | 47.2% |
| Enc Time[%] | 69% |  |  | 68% |  |  |
| Dec Time[%] | 73% |  |  | 74% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -12.4% | 36.8% | 31.8% | -11.4% | 35.0% | 29.3% |
| Class C | -15.4% | -5.7% | -3.5% | -12.1% | -3.4% | -1.9% |
| Class D | -10.0% | 0.7% | -0.3% | -7.1% | 0.7% | -0.3% |
| Class E | -9.8% | 13.5% | 13.3% | -11.9% | 10.1% | 5.9% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -12.0% | 12.8% | 11.5% | -10.6% | 12.1% | 9.7% |
|  | -12.0% | 12.5% | 11.2% | -10.6% | 12.0% | 9.2% |
| Enc Time[%] | 78% |  |  | 84% |  |  |
| Dec Time[%] | 82% |  |  | 84% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -12.8% | 36.2% | 32.3% | -11.7% | 33.3% | 30.1% |
| Class C | -16.0% | -5.0% | -4.0% | -12.0% | -3.5% | -1.9% |
| Class D | -12.5% | 5.9% | 2.0% | -8.7% | -2.8% | -2.5% |
| Class E | -11.1% | 14.1% | 10.0% | -11.6% | 9.5% | 6.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -13.2% | 14.2% | 11.4% | -11.0% | 10.6% | 9.4% |
|  | -13.1% | 14.6% | 11.2% | -11.0% | 9.8% | 9.2% |
| Enc Time[%] | 86% |  |  | 84% |  |  |
| Dec Time[%] | 81% |  |  | 81% |  |  |

## HM4.0+prefiltering+QP increase versus ARC+adaptive up-sampling

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -8.6% | 197.7% | 159.7% | -14.2% | 77.9% | 172.1% |
| Class B | -12.9% | 27.9% | 29.1% | -13.0% | 23.5% | 29.5% |
| Class C | -13.4% | -2.6% | 2.2% | -11.7% | -3.5% | 1.5% |
| Class D | -12.6% | 8.1% | 5.8% | -10.7% | 5.7% | 1.1% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -11.9% | 56.0% | 48.0% | -12.4% | 25.8% | 49.8% |
|  | -11.9% | 56.2% | 49.2% | -12.4% | 62.0% | 51.2% |
| Enc Time[%] | 77% |  |  | 78% |  |  |
| Dec Time[%] | 73% |  |  | 73% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -12.5% | 38.2% | 32.9% | -11.5% | 37.6% | 31.7% |
| Class C | -15.4% | -5.7% | -3.5% | -12.1% | -3.4% | -1.9% |
| Class D | -10.0% | 0.7% | -0.3% | -7.1% | 0.7% | -0.3% |
| Class E | -11.2% | 13.8% | 13.9% | -12.8% | 10.9% | 6.9% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -12.3% | 13.3% | 11.9% | -10.8% | 13.1% | 10.6% |
|  | -12.3% | 13.0% | 11.7% | -10.7% | 13.1% | 10.1% |
| Enc Time[%] | 86% |  |  | 79% |  |  |
| Dec Time[%] | 83% |  |  | 83% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -12.9% | 37.4% | 33.3% | -11.8% | 35.9% | 32.3% |
| Class C | -16.0% | -5.0% | -4.0% | -12.0% | -3.5% | -1.9% |
| Class D | -12.5% | 5.9% | 2.0% | -8.7% | -2.8% | -2.5% |
| Class E | -12.5% | 14.3% | 10.5% | -12.5% | 10.4% | 7.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -13.5% | 14.6% | 11.9% | -11.2% | 11.6% | 10.3% |
|  | -13.4% | 15.1% | 11.6% | -11.2% | 10.9% | 10.1% |
| Enc Time[%] | 80% |  |  | 79% |  |  |
| Dec Time[%] | 82% |  |  | 81% |  |  |

# Working draft text

A modified copy of the latest Working Draft [4] accompanies this contribution. Features include:

- The SPS is modified to provide values of sequence_pic_width_in_luma_samples and sequence_pic_height_in_luma_samples, and a flag indicating whether resolution can be switched

- The PPS contains pic_size_idx, and based on this value different values of pic_width_in_luma_samples and  pic_height_in_luma_samples are computed. These are used in any slice referencing that PPS

- Co-located MV prediction shall be disabled when the resolution of the current picture and the reference picture differ.

- For any reference picture, both high and low-resolution copies may be simultaneously available. Upsampled and down-sampled versions are considered as separate pictures for the purposes of DPB management

# Conclusions

It is recommended that Adaptive Resolution Coding is adopted into the Working Draft and the HM.

# References

[1] ITU-T H.263 (01/2005): Video coding for low bit rate communication, Annex O “Temporal, SNR and Spatial Scalability mode” and Annex P “Reference Picture Resampling”

[2] JCTVC-F158, “Resolution switching for coding efficiency and resilience”, T. Davies (Cisco), JCTVC 6th Meeting, Turin, IT, 14-22 July 2011

[3] JCTVC-G018, “JCTVC AHG report: Resolution adaption (AHG18), T. Davies (Cisco), P. Topiwala (FastVDO) and P. Wu (ZTE), JCT-VC 7th Meeting, Geneva, CH,  21-30 November, 2011

[4] JCTVC-F803, “WD4: Working Draft 4 of High-Efficiency Video Coding”, K. McCann, S. Sekiguci, B. Bross, W.-J. Han (Eds.), JCTVC 6th Meeting, Turin, IT, 14-22 July 2011

[5] JCTVC-F618, “Advanced Resampling Filters for Spatial Scalability and Chroma Sampling Applications in HEVC”, W. Dai, M. Krihnan, P. Topiwala (FastVDO LLC), JCTVC 6th Meeting, Turin, IT, 14-22 July 2011

[6] "Context-based adaptive image resolution upconversion", Shi et al, Journal of Electronic Imaging 19(1), 013008 (Jan-Mar 2010)

[7] JCTVC-900, “Common test conditions and software reference configurations”, F. Bossen, JCT-VC 6th Meeting, Turin, Italy, 14-22 July 2011

[8] JVT-V030, “Advanced Extended Spatial Re-sampling Filters For SVC”, Trac D. Tran, L. Liu, P. Topiwala (FastVDO), JVT 22nd Meeting: Marrakech, Morocco, 13–19 January, 2007

[9] JCTVC-G862, “Advanced Resampling Filters for HEVC Applications”, W. Dai, M.Krihnan, P. Topiwal (FastVDO), JCT-VC 7th Meeting, Geneva, CH,  21-30 November, 2011

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

FastVDO LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).