| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G265 |
| --- | --- |

| Title: | Core Transform Property for Practical Throughput Hardware Design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Mehul Tikekar Chao-Tsung Huang Chiraag Juvekar Anantha Chandrakasan Massachusetts Institute of Technology | Email: | mtikekar@mit.edu |
| Source: | Massachusetts Institute of Technology |  |  |

_____________________________

# Abstract

The number of arithmetic operations (multiplications, additions and shifts) per inverse transform operation does not completely reflect the complexity of a real hardware design. Specifically, the number of unique coefficients in the transform matrices also impacts hardware complexity.  A study of the HM-4.0 core transform identifies this as a useful property that enables 25% area reduction for hardware designs with practical throughput. This contribution suggests that this property be considered when comparing complexity of core transform proposals for HEVC.

# Introduction

The conventional complexity metric of number of operations per inverse transform used in [1] and [2] to compare complexities of various transforms does not accurately predict the area of a typical hardware implementation. For example, a complete 2D inverse transform of a 32x32 Transform Unit requires 21888 multiplications for the partial butterfly structure in HM-4.0. This number represents the computational cost of processing 1024 pixels. Thus, a hardware design with a 1024 pixels/cycle throughput requires 21888 constant multiplier blocks. However, practical hardware designs need to process far fewer pixels per cycle. For example, even for high video rates at typical clock speeds - 4Kx2K @ 30fps running at 200MHz – the inverse transform throughput is only 2 pixels/cycle. The hardware area for a practical implementation does not directly scale from the complexity of processing 1024 pixels. A useful property of the HM-4.0 core transform enables significant area reduction at reduced throughputs through sharing of hardware over multiple cycles.

A more appropriate metric for hardware complexity is the number of unique arithmetic operations. The rationale is that if the same operation is being performed multiple times, the same hardware can be reused for every time. Hence, multiple instances of the same operation do not add to the hardware cost.

# Hardware Cost of Core Transforms

Figure 1: HM-4.0 Partial Butterfly Structure

Fig. 1 shows the partial butterfly structure for the 32-point DCT from HM-4.0. The property of unique operations is demonstrated for the EEO computation below though it applies to the EO and O parts as well. The EEO computation is essentially a 4x4 matrix multiplication:

While this matrix multiplication requires 16 multiplies, there are only 4 unique multiplication constants – 89, 75, 50 and 18. Hence, a hardware that performs one row operation of this matrix multiplication every cycle needs only 4 multiplier modules – one for each constant. The inputs xi (i = 4, 12, 20, 28) must be multiplexed into these multipliers according to the output eeon (n = 0, 1, 2, 3) being computed.

These four multipliers can be implemented using shift-and-adds as one of the multiplicands is a constant. If all the 16 coefficients were unique, 4 generic multipliers and 4 look-up tables would be needed.

Figure 2: 4x4 EEO Matrix Multiplication with constant multipliers and with generic multipliers and LUTs

This approach can be extended to the 8x8 EO and 16x16 O matrices as they too possess this property of few unique operations. The 8x8 matrix has only 8 unique factors and the 16x16 matrix has 16 unique factors.

This property was exploited to design a 1-D inverse transform module that processes 4 pixels per cycle. The area of this transform design is 77.4 kgates, while a design that does not exploit this property of unique operations has an area of 104 kgates. This area represents a fully compliant, complete HM-4.0 inverse transform implementation i.e. the logic for 1-D 4-pt, 8-pt, 16-pt and 32-pt inverse DCT and a 4-pt inverse DST, thus supporting all TU sizes (square and non-square) and control logic to switch between them. When the same module is used for row and column transforms, the throughput for a 2-D transform is 2 pixels per cycle. Thus, this approach yields a 25% reduction in the area for a design with 2 pixels/cycle throughput.

| N | HM-4.0 [3] |
| --- | --- |
| 4 | 3 |
| 8 | 7 |
| 16 | 15 |
| 32 | 29 |

Table 1: Number of unique coefficients in transform matrices of HM-4.0

# Conclusions

For practical implementations, the number of unique coefficients in the transform matrices impacts hardware complexity and should be considered when comparing complexity of core transform proposals for HEVC.

# References

[1] R. Cohen, C. Yeo, R. Joshi, F. Fernandes, “CE7: Summary report of Core Experiment on additional transforms,” JCTVC-F027, July 2011

[2] P. Topiwala, W. Dai, M. Krishnan, I. Kim, “CE 10: FastVDO/Samsung Core Transform Proposal,” JCTVC-F363, July 2011

[3] HM-4.0 High efficiency test model (HM) version 4.0 URL: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0/

# Patent rights declaration(s)

Massachusetts Institute of Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).