| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G267 |
| --- | --- |

| Title: | CE6.c Report on SDIP chroma extension scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Song (Huawei) Changcai Lai (Hisilicon) Haitao Yang (Huawei) Haoping Yu (Huawei) | Tel: Email: | +86 10 60614081 andy.songjin@huawei.com laichangcai@huawei.com haitao.yang@huawei.com haoping.yu@huawei.com |
| Source: | Huawei & Hisilicon |  |  |

_____________________________

# Abstract

This document reports the experimental results of CE6.c subset for SDIP (short distance intra prediction) chroma extension scheme in the common test condition. It is reported to provide Y/U/V component BD-rate of -0.1%/-1.2%/-1.3% and -0.1%/-1.8%/-1.6% under High Efficiency all intra and Low Complexity all intra conditions with Class F based on CE6.c SDIP subset case1 and case2.

# Introduction

A chroma coding scheme for intra CUs with non-square PUs was proposed in last meeting. In this scheme, the chroma intra prediction mode set contains 6 modes. According to the mode distribution of the non-square luma PUs within a CU, the 6 modes in the mode set is selected from 9 candidate modes, i.e. LM, DM1, DM2, SDM, Planar, Vertical, Horizontal, DC and Vertical+8. DM1, DM2 and SDM are new modes for SDIP, and other modes are the same as those in HM4.0.

DM1 and DM2 are very similar with DM mode in HM4.0. The number of different prediction modes of the four SDIP luma partitions in one CU is counted. If the number of any prediction mode is larger than 1, DM1 gets the mode type. If any other mode still matches condition, DM2 gets the mode type. An example of deciding DM1 and DM2 is shown in Table 1, where A, B, C, or D represent the mode of a luma PU.

Table 1 An example of deciding DM1, DM2 from four SDIP luma PUs
(A, B, C, or D represents the mode of a luma PU)

| four SDIP luma partitions | DM1 | DM2 |
| --- | --- | --- |
| AAAA | A | N/A |
| AAAB | A | N/A |
| AABB | A | B |
| AABC | A | N/A |
| ABCD | N/A | N/A |

With the proposed SDM mode, chroma component shares the same PU partition and prediction modes of luma component. That means in a SDIP CU, when luma component is split into 2NxhN or hNx2N PU, so is chroma component. And the chroma predictions in four “chroma PU” will be the same as luma prediction in their related “luma PU”. In other word, it is the same way that implementing luma SDIP mode and chroma SDM mode. It is applied to SDIP CUs larger than or equal to 16x16.

Figure 1 Illustration of SDM mode

The first six available modes from the nine candidate modes listed above, in the described order, composes the mode set for chroma intra prediction. And codewords are assigned to them from short to long, which is the same as chroma prediction mode coding in HM4.0.

Table 2 Mode Numbers and Codewords for intra chroma prediction mode in SDIP partitions

| Intra chroma prediction mode (the nine candidate modes) | Mode number | Codewords (the first six available modes) |
| --- | --- | --- |
| LM | 35 | 0 |
| DM1 | 37 | 10 |
| DM2 | 38 | 110 |
| SDM | 36 | 1110 |
| PLANAR | 0 | 11110 |
| Vertical | 1 | 11111 |
| Horizontal | 2 |  |
| DC | 3 |  |
| Vertical + 8 | 7 |  |

# Simulations & Results

The proposed scheme is implemented on top of HM4.0-sdip software which is released in CE6 group. Simulations are conducted based on the JCT-VC common test condition defined in Error: Reference source not found. This proposal has been cross-verified by Samsung. Table 3 to Table 6 show the performance of the proposal based on CE6.c SDIP subset case 1 and case 2, with Class F and without Class F.

Table 3 The performance of the proposal based on CE6.c SDIP subset case 1 with Class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.8% | -0.3% | 0.0% | -1.9% | -1.4% |
| Class B | 0.0% | -1.4% | -1.2% | 0.0% | -2.4% | -2.3% |
| Class C | -0.1% | -1.0% | -1.1% | -0.1% | -1.1% | -1.1% |
| Class D | 0.0% | -1.3% | -1.3% | -0.1% | -1.2% | -1.0% |
| Class E | 0.0% | -1.8% | -1.9% | 0.0% | -2.1% | -2.0% |
| Class F | -0.3% | -1.2% | -1.5% | -0.2% | -1.8% | -1.9% |
| Overall | -0.1% | -1.3% | -1.2% | -0.1% | -1.8% | -1.6% |
|  | -0.1% | -1.3% | -1.2% | -0.1% | -1.8% | -1.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 4 The performance of the proposal based on CE6.c SDIP subset case 1 without Class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.8% | -0.3% | 0.0% | -1.9% | -1.4% |
| Class B | 0.0% | -1.4% | -1.2% | 0.0% | -2.4% | -2.3% |
| Class C | -0.1% | -1.0% | -1.1% | -0.1% | -1.1% | -1.1% |
| Class D | 0.0% | -1.3% | -1.3% | -0.1% | -1.2% | -1.0% |
| Class E | 0.0% | -1.8% | -1.9% | 0.0% | -2.1% | -2.0% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -1.3% | -1.1% | 0.0% | -1.8% | -1.6% |
|  | 0.0% | -1.3% | -1.2% | 0.0% | -1.8% | -1.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 5 The performance of the proposal based on CE6.c SDIP subset case 2 with Class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.7% | -0.4% | 0.0% | -1.4% | -1.1% |
| Class B | 0.0% | -1.3% | -1.2% | 0.0% | -2.0% | -1.9% |
| Class C | 0.0% | -0.9% | -0.9% | -0.1% | -0.8% | -0.9% |
| Class D | 0.0% | -1.2% | -1.1% | 0.0% | -1.0% | -0.8% |
| Class E | 0.0% | -1.5% | -1.6% | 0.0% | -1.7% | -1.6% |
| Class F | -0.1% | -1.0% | -1.2% | -0.2% | -1.6% | -1.6% |
| Overall | 0.0% | -1.1% | -1.0% | 0.0% | -1.4% | -1.3% |
|  | 0.0% | -1.1% | -1.1% | 0.0% | -1.4% | -1.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 6 The performance of the proposal based on CE6.c SDIP subset case 2 without Class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.7% | -0.4% | 0.0% | -1.4% | -1.1% |
| Class B | 0.0% | -1.3% | -1.2% | 0.0% | -2.0% | -1.9% |
| Class C | 0.0% | -0.9% | -0.9% | -0.1% | -0.8% | -0.9% |
| Class D | 0.0% | -1.2% | -1.1% | 0.0% | -1.0% | -0.8% |
| Class E | 0.0% | -1.5% | -1.6% | 0.0% | -1.7% | -1.6% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -1.1% | -1.0% | 0.0% | -1.4% | -1.3% |
|  | 0.0% | -1.1% | -1.0% | 0.0% | -1.4% | -1.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

# Conclusion

SDIP chroma extension scheme is proposed in this contribution. The coding performance of SDIP is improved and no complexity is introduced by the scheme. We recommend adopting this method in next version of HEVC Test Model.

# Reference

- F. Bossen, "Common test conditions and software reference configurations," JCT-VC document JCTVC-Gxxx, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, CH, 21-30 November, 2011

- J. Song, et al, “A chroma coding scheme for SDIP mode”, Doc. JCTVC-F505, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, IT, 14-22 July, 2011

# Proposed change to WD text

8.3.2	Derivation process for chroma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–  a variable log2TrafoSize specifying the size of the current coding unit.

Output of this process is the variable IntraPredModeC.

Depending on PartMode, the chroma intra prediction mode IntraPredModeC is derived as follows:

–  If PartMode is equal to PART_2Nx2N or PART_NxN, the chroma intra prediction mode IntraPredModeC is derived as specifed in Table 8-1 or Table 8-2 with intra_chroma_pred_mode, IntraPredMode[ xB ][ yB ] and chroma_pred_from_luma_enabled_flag as inputs. IntraPredModeC is further modified to be equal to Intra_Planar when IntraPredModeC is equal to Intra_DC and planar_flag_chroma is equal to 1.

Table 8-1 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | X ( 3 <= X < 34 ) |
| 0 | 34 | 34 | 34 | 34 |
| 1 | n/a | 0 | 0 | 0 |
| 2 | 1 | n/a | 1 | 1 |
| 3 | 2 | 2 | n/a | 2 |
| 4 | 0 | 1 | 2 | X |

Table 8-2 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 0

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | X ( 3 <= X < 34 ) |
| 0 | n/a | 0 | 0 | 0 |
| 1 | 1 | n/a | 1 | 1 |
| 2 | 2 | 2 | n/a | 2 |
| 3 | 0 | 1 | 2 | X |

–  Otherwise (PartMode is equal to PART_2NxhN or PART_hNx2N), IntraPredModeC is derived by following procedures with intra_chroma_pred_mode and the four intra prediction modes of the current coding unit as inputs.

- The variables xOff, yOff are set as follows:

if(PartMode == PART_2NxhN)

{

xOff = 0;

yOff = (1<< log2TrafoSize)>>2;

}

else if(PartMode == PART_ hNx2N)

{

xOff = (1<< log2TrafoSize)>>2;

yOff = 0;

}

- mapIntraPredModeC[0] is derived as follows:

j = 0;

mapIntraPredModeC[j++] = 35;

- The table IntraPredModeCnt with a size of 34 recording the four intra prediction modes of the current coding unit is given by

IntraPredModeCnt[34] = {0};

for(blkIdx = 0; blkIdx < 4; blkIdx++)

IntraPredModeCnt[ IntraPredMode[ xB+xOff*blkIdx][yB+yOff*blkIdx] ]++;

- mapping table mapIntraPredModeC with a size of 6 is derived as follows:

for( n = 0; n < 34; n++ )

if( IntraPredModeCnt[n] >1)

{

mapIntraPredModeC[j++] = n;

}

If(log2TrafoSize > 3)

mapIntraPredModeC[j++] = 34;

for( n = 0; n < 4; n++ )

if(IntraPredModeCnt[n] < 2  && j < 5)

mapIntraPredModeC[j++] = n;

if(j < 5)

mapIntraPredModeC[j] = 7;

- IntraPredModeC is then derived by

IntraPredModeC = mapIntraPredModeC[ intra_chroma_pred_mode ].

# Patent rights declaration(s)

Huawei and Hisilicon may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).