| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G269 |
| --- | --- |

| Title: | CE11 Report on Prediction Unit Dependent Coefficient Scanning For Inter Frame |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Song (Huawei) Xiaozhen Zheng (Hisilicon) Haitao Yang (Huawei) Haoping Yu (Huawei) | Tel: Email: | +86 10 60614081 andy.songjin@huawei.com zhengxiaozhen@huawei.com haitao.yang@huawei.com haoping.yu@huawei.com |
| Source: | Huawei & Hisilicon |  |  |

_____________________________

# Abstract

This document reports the experimental results of CE11 prediction unit dependent coefficient scanning for inter frame scheme in the common test condition. It is reported to provide Y/U/V component BD-rate of -0.01%/-0.21%/-0.08% and -0.10%/-0.35%/-0.39% under Low Complexity Random access and Low delay conditions without Class F.

# Introduction

A prediction unit dependent coefficient scanning order selection scheme (PDCS) for inter coding is proposed in the last meeting. The best scanning order for transform coefficients is selected among zigzag, horizontal, and vertical scans based on the prediction unit partition. The relationship between PU partition and scanning pattern is shown in Table 1.

Table 1 The relationship between PU partition and scanning pattern

| PU partition | Scanning pattern |
| --- | --- |
| SIZE_2NxN | Vertical scan |
| SIZE_Nx2N | Horizontal scan |
| SIZE_2Nx2N | Zig-zag scan |
| SIZE_NxN | Zig-zag scan |

Because NSQT has been adopt into HM4.0 software, PDCS has to combine with NSQT. The relationship among PU partition, TU partition and scanning pattern is shown in Table 2.

Table 2 The relationship between PU partition, TU partition and scanning pattern

|  | 2NxN PU | 2NxnU PU | 2NxnD PU | Nx2N PU | nLx2N PU | nRx2N PU | 2Nx2N PU | NxN PU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-square TU | Vertical scan (TU is horizontal stripe) |  |  | Horizontal scan (TU is vertical stripe) |  |  | N/A |  |
| Square TU | Zig-zag scan |  |  |  |  |  |  |  |

Figure 1 Horizontal Scan

Figure 2 Vertical Scan

# Simulations & Results

The proposed scheme is implemented on top of HM4.0 software. Simulations are conducted based on the JCT-VC common test condition defined in Error: Reference source not found. This proposal has been cross-verified by Samsung.

Table 3 shows the performance of the proposal under Low Complexity Random access and Low delay conditions with Class F and Table 4 shows the performance without Class F.

Table 3 the performance of the proposal under LC_RA and LC_LD conditions with Class F

|  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.2% |  |  |  |
| Class B | -0.1% | -0.5% | -0.3% | -0.1% | -0.7% | -0.5% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | -0.3% | -0.5% |
| Class D | 0.0% | -0.2% | -0.1% | -0.1% | -0.2% | -0.3% |
| Class E |  |  |  | -0.3% | -0.1% | -0.2% |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | -0.3% | -0.1% |
| Overall | 0.0% | -0.2% | -0.1% | -0.1% | -0.3% | -0.3% |
|  | 0.0% | -0.2% | -0.1% | -0.1% | -0.4% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 4 the performance of the proposal under LC_RA and LC_LD conditions without Class F

|  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.2% |  |  |  |
| Class B | -0.1% | -0.5% | -0.3% | -0.1% | -0.7% | -0.5% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | -0.3% | -0.5% |
| Class D | 0.0% | -0.2% | -0.1% | -0.1% | -0.2% | -0.3% |
| Class E |  |  |  | -0.3% | -0.1% | -0.2% |
| Overall | 0.0% | -0.2% | -0.1% | -0.1% | -0.4% | -0.4% |
|  | 0.0% | -0.2% | -0.1% | -0.1% | -0.4% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusion

PU dependent coefficient scanning method is proposed in this contribution. It gets extra gain on NSQT and no complexity is introduced. We recommend adopting this method in next version of HEVC Test Model.

# Reference

- F. Bossen, "Common test conditions and software reference configurations," JCT-VC document JCTVC-Gxxx, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, CH, 21-30 November, 2011

- J. Song, et al, “Prediction Unit Dependent Coefficient Scanning For Inter Frame”, Doc. JCTVC-F501, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, IT, 14-22 July, 2011

# Proposed change to WD text

### Inverse scanning process for transform coefficients

Inputs to this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit.

–	a list of (nW)x(nH) values transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ].

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is a variable c containing a two-dimensional array of (nW)x(nH) values.

The variable nS is set equal to Sqrt(nW*nH).

The variable scanIdx, indicating wich scan is used to scan the transform coefficients (zigzag, horizontal or vertical), is derived as follows.

- If PredMode is equal to MODE_INTRA, the following applies.

  - If cIdx is equal to 0, scanIdx is set to ScanType[ Log2( nS >> 2 ) ][ IntraPredMode ].

  - Otherwise (cIdx is equal to 1 or 2), scanIdx is set to ScanType[ Log2( nS >> 2 ) ][ IntraPredModeC ]

Otherwise (PredMode is not equal to MODE_INTRA), when nW is equal to nH, scanIdx is set to 0,when nW is larger than nH, scanIdx is set to 2,else scanIdx is set to 1;

# Patent rights declaration(s)

Huawei and Hisilicon may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).