| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G270 |
| --- | --- |

| Title: | Non-CE6a: Cross-checking of JCTVC-G244 - Luma-based chroma prediction - Model correction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | P.Bordes, P.Salmon | Tel: Email: | +33 2 99 27 32 42 philippe.bordes@technicolor.com philippe.salmon@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This document is a cross check report for the Model correction of the Luma-based chroma prediction proposed in JCTVC-G244 [1].

In the current HM4.0 implementation, the luma-based chroma intra prediction uses the reconstructed luma samples and 2 parameters (,) to derive the chroma prediction samples. However, it is reported the method to compute (,) may be untrustworthy in some cases identified in the proposal. In these cases, it is proposed to use pre-determined fixed values for ( is derived from  using the model parameter).

The corresponding performance results are reported. The simulation results reportedly match with the results provided by the proponents.

# Modified Luma-based chroma prediction

In the current HM4.0 implementation, the luma-based chroma intra prediction mode uses samples from the causal borders of the block to estimate through OLS algorithm the linear parameters (,) linking luma and chroma samples.

The chroma samples (PredC) are predicted from the reconstructed down-sampled luma samples (RecL) using the parameters (,) (1):

PredC(i) = RecL(i) + 	(1)

However, the OLS algorithm may lead to unreliable values in some cases listed in the proposal. For each of these cases, it is proposed to use a pre-determined value transmitted in the bitstream.

- Code verification and test conditions

Canon provided Technicolor with the source code to be tested.

The code implements the bug fix proposed by TI in [2] (ALPHA_BITS = 7).

The tested configuration corresponds to the following settings:

#define LM_CHROMA                             1	// JCTVC-E266: Chroma intra prediction based

// on luma signal

#if LM_CHROMA

#  define LM_CHROMA_SIMPLIFICATION            1   	// JCTVC-F760: LM simplification: One line

// luma reference pixel buffer, alpha 7

// bits.

#  define LM_CHROMA_ALT                       1	// Use of an alternate alpha when high

// probabilities of errors

#  define LM_CHROMA_ALT_UPDATE                1	// Update the alternate alpha on a frame

// basis

#  define LM_CHROMA_ALT_DECIMATION            0	// Decimate the borders used in the OLS

// computation

#endif

The source code have been successfully compiled and tested on a cluster grid Linux (RedHat 5.5) with the following characteristics:

- CPU: 2 x Intel(R) Xeon(R) E5680 @ 3.33GHz, 6 cores.

- RAM: 96 Go per 6 cores.

# Simulation Results

We present the simulation results obtained for A,B,C,D,E classes (Figure 1) and also for class F (Figure 2), in Intra only High-Efficiency and Intra only Low-Complexity.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -14,0% | -16,8% | -0,1% | -14,3% | -16,0% |
| Class B | 0,0% | -1,8% | -1,0% | 0,0% | -1,9% | -1,0% |
| Class C | -0,1% | -1,3% | -1,7% | -0,1% | -1,6% | -1,5% |
| Class D | -0,1% | -1,4% | -1,4% | -0,1% | -1,5% | -1,4% |
| Class E | 0,0% | -2,6% | -1,7% | 0,0% | -2,9% | -1,7% |
| Overall | 0,0% | -4,2% | -4,5% | -0,1% | -4,4% | -4,3% |
|  | 0,0% | -4,1% | -4,4% | -0,1% | -4,3% | -4,2% |
| Enc Time[%] | 100,0% |  |  | 100,6% |  |  |
| Dec Time[%] | 99,9% |  |  | 96,8% |  |  |

Figure 1: results obtained for A,B,C,D,E classes.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -14,1% | -16,8% | -0,1% | -14,2% | -16,0% |
| Class B | 0,0% | -1,8% | -1,0% | 0,0% | -2,0% | -1,0% |
| Class C | -0,1% | -1,3% | -1,7% | -0,1% | -1,6% | -1,5% |
| Class D | -0,1% | -1,4% | -1,4% | -0,1% | -1,5% | -1,5% |
| Class E | 0,0% | -2,6% | -1,6% | -0,1% | -2,9% | -1,7% |
| Class F | -0,1% | -1,1% | -1,2% | -0,1% | -0,8% | -0,9% |
| Overall | 0,0% | -3,7% | -3,9% | -0,1% | -3,8% | -3,7% |
|  | 0.0% | -3.6% | -3.9% | -0,1% | -3,8% | -3,7% |
| Enc Time[%] | 100,1% |  |  | 100,7% |  |  |
| Dec Time[%] | 99,6% |  |  | 97,2% |  |  |

Figure 2: results obtained for A,B,C,D,E,F classes.

# Conclusion

The results presented by Canon in JCTVC-G244 are confirmed.

# References

- C.Gisquet & E.François, “Non-CE6a: Luma-based chroma prediction - Model correction,” Doc. JCTVC-G244, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, November 2011.

- Jianle Chen, “BoG report on simplification of intra_chromaFromLuma mode prediction,” Doc. JCTVC-F760, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, Italy, July 2011.