| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 Nov, 2011 | Document: JCTVC-G274 WG11 Number: m21836 |
| --- | --- |

| Title: | Non-CE9: cross-check of contribution JCTVC-G082 on modified H TMVP position |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Jung Orange Labs 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

The document reports cross-check results for the two experiments proposed in JCTVC-G082. A modification of the H TMVP positions is proposed to avoid co-located motion data required for TMVP derivation of the current LCU going beyond the current LCU row. For that purpose, the H TVMP positions around the bottom boundary of the current LCU are replaced with closest ones inside the current LCU row (configuration 1), or with C3 TMVP positions (configuration 2).

It has been verified that common conditions are followed; results provided by the other tester (TI, JCTVC-G082) are complete and match with the results reported in this document. The source code has also been checked, the software has been compiled and no issue is reported.

# Modified H TMVP position

HM4.0 uses the H temporal MV predictor, or the centered C0 position if H is not available. The main drawback of the current HM design is that the collocated motion data fetch is not aligned with the current LCU. Two configurations are proposed to maintain the TMVP predictor inside the LCU row.

| Description | Average loss | Comment |
| --- | --- | --- |
| HM4.0: - Use H if available - Use C0 otherwise |  | TMVP can be outside the LCU row |
| Configuration 1: - Use H if available and inside the LCU row - Use C0 if H not available - Use BR if H outside the LCU row | 0.2% | TMVP always inside the LCU row. |
| Configuration 2: - Use H if available and inside the LCU row - Use C3 otherwise | 0.2% | TMVP always inside the LCU row. |

The software has been verified and compiled. It does not raise any issue.

The cross check of the results have been done for all classes and Inter configurations. Results obtained by TI and Orange Labs are reported in the following table. A perfect match is achieved.

Configuration 1:

| Exp. | RA-HE |  |  | RA-LC |  |  | LD-HE |  |  | LD-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time |
| Tester1 | 0.1 | 102 | 103 | 0.1 | 100 | 101 | 0.2 | 97 | 93 | 0.2 | 96 | 94 |
| Orange Labs | 0.1 | - | - | 0.1 | - | - | 0.2 | - | - | 0.2 | - | - |

Configuration 2:

| Exp. | RA-HE |  |  | RA-LC |  |  | LD-HE |  |  | LD-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time | BD-Rate | Enc. Time | Dec. Time |
| Tester1 | 0.2 | 102 | 102 | 0.1 | 103 | 110 | 0.3 | 100 | 102 | 0.3 | 100 | 106 |
| Orange Labs | 0.2 |  |  | 0.1 |  |  | 0.3 |  |  | 0.3 |  |  |

It is of interest to notice that the average loss of both configurations is 0.2% (0.18% and 0.23% when accurate precisions are used for computation. This difference is of course marginal.

|  | RA-HE | RA-LC | LD-HE | LD-LC | AVG |
| --- | --- | --- | --- | --- | --- |
| Config1 | 0.15 | 0.13 | 0.24 | 0.22 | 0.18 |
| Config2 | 0.17 | 0.15 | 0.29 | 0.26 | 0.23 |

# Conclusion

This verification contribution confirms the results obtained by TI in JCTVC-G082. According to the results obtained, it is recommended to adopt configuration 2, which provides similar results as configuration 1, allowing however cleaner WD text and software.