| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G276 |
| --- | --- |

| Title: | CE6: Cross-Check report for CE6b subtests of Intra Mode Coding (F062, F459, F091, F106, F269) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Guichun Li (Santa Clara University), Lingzhi Liu (Hisilicon Technologies Co. Ltd.) , Nam Ling (Santa Clara University), Jianhua Zheng (Hisilicon Technologies Co. Ltd.), Philipp Zhang (Hisilicon Technologies Co. Ltd.), | Tel: Email: | +1-408-554-4794 g1li@scu.edu lingzhi.liu@huawei.com nling@scu.edu zhengjianhua@huawei.com pzhang@huawei.com |
| Source: | Santa Clara University & Hisilicon Technologies Co. Ltd. |  |  |

_____________________________

# Abstract

Four of CE6 intra mode coding subtests, F062 & F459 combined, F091, F106, and F269, in CE6b are cross-verified and the performance results are reported. The simulation results reportedly match with the results provided by the proponents.

# Introduction

In Core Experiment 6 subtest CE6b, various MPMs (Most Probable Modes) derivation and remaining mode coding proposals are tested. The two MPMs derivation proposals, F062 and F459 were combined together as a single test. Three remaining mode coding methods, F091, F106, and F269 were tested separately.

# MPMs Derivation and Remaining Mode Coding Tests

## The combination of Luma Intra Prediction Mode Coding (F062) and Parsing friendly intra mode coding (F459)

In this test, the original proposals of F062 and F459 are harmonized and combined. In the proposal, three additional most probable remaining modes (MPRMs) are added for intra luma prediction mode coding. Two neighboring tables are used for deriving the additional MPMs, one for 4x4 PUs, the other for PUs with size 8x8, 16x16, or 32x32. In intra chroma mode coding, the original vertical, horizontal, and DC modes were replaced by two intra prediction modes adjacent ot the coded intra luma prediction mode and one mode perpendicular to the coded intra luma mode.

The provided software is studied. The proposed changes were correctly implemented.

## Remaining mode coding proposal from Sony (F091)

In the proposal of F091, it was claimed that the same methods of binarization and adaptation of remaining mode coding are used for both High Efficiency (HE) and Low Complexity (LC) cases.

In the implementation, two arrays are used for coded intra mode’s ranking. The one of size 17 is used for 4x4 PUs, the other one with size of 34 is used for 8x8, 16x16, and 32x32 PUs. Similarly, two sets of code words are used, with generalized version of Exponencial-Golomb codes.

The code provided by the proponents was studied, and it’s found to reflect the proposed methods correctly.

## Remaining mode coding proposal for CAVLC from LG (F106)

In proposal F106, the orders of index mapping process and remaining mode derivation process are switched to reflect the dependency of remaining modes on the value of MPM candidates. The intra luma prediction mode is directly converted in to table index by inverse index mapping table. For 4x4 PUs, one index mapping table and on inverse index mapping table with 17 entries are used; for 8x8, 16x16, 32x32 PUs, one index mapping table and one inverse index mapping table with 34 entries are used. The method only applies to CAVLC, no modification was made for CABAC. All the proposed methods were found in the code provided by the proponents.

## Remaining mode coding proposal from Canon (F269)

In proposal F269, the statistics of coded intra modes are updated in real time. And the statistical results are used to code intra modes when it’s not equal to any of the neighboring coded intra modes. It’s similar to F106, but applies to both CABAC and CAVLC cases.

The software is studied and the implementation was found to reflect the proposal correctly.

# Simulation Results

Simulation of the proposals were conducted on the same hardware platforms, using common test conditions and configurations [7]. HM-4.0 software was used as Anchor.

## Performance of the combination of F062 and F459

Table – 1 Combination of F062 and F459 compared with HM-4.0 Anchor

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.35% | -0.01% | -0.05% | -0.29% | -0.28% | -0.25% |
| Class B | -0.34% | -0.38% | -0.46% | -0.37% | -0.69% | -0.69% |
| Class C | -0.38% | -0.66% | -0.73% | -0.47% | -0.97% | -0.92% |
| Class D | -0.34% | -0.57% | -0.64% | -0.41% | -0.65% | -0.78% |
| Class E | -0.63% | -0.30% | -0.46% | -0.60% | -0.79% | -0.83% |
| Overall | -0.39% | -0.39% | -0.47% | -0.42% | -0.67% | -0.69% |
|  | -0.39% | -0.39% | -0.48% | -0.42% | -0.67% | -0.69% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

The simulation results reportedly show that the coding gain brought by the joint software of F062 and F459 is 0.39% in HE case, and 0.42% in LC case. It is the same as the results reported by the proponents.

## Performance of F091

Table – 2 F091 compared with HM-4.0 Anchor

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.39% | -0.11% | -0.07% | -0.22% | -0.12% | -0.17% |
| Class B | -0.29% | -0.06% | 0.00% | -0.26% | -0.22% | -0.19% |
| Class C | -0.42% | -0.14% | -0.18% | -0.35% | -0.26% | -0.26% |
| Class D | -0.31% | -0.10% | -0.17% | -0.25% | -0.14% | -0.18% |
| Class E | -0.47% | -0.14% | -0.06% | -0.35% | -0.27% | -0.17% |
| Overall | -0.37% | -0.10% | -0.10% | -0.28% | -0.20% | -0.20% |
|  | -0.37% | -0.11% | -0.11% | -0.28% | -0.20% | -0.20% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

The simulation results reportedly show that the coding gain brought by F091 is 0.37% in HE case, and 0.28% in LC case. It is the same as the results reported by the proponents.

## Performance of F106

Table – 3 F106 compared with HM-4.0 Anchor

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | -0.19% | -0.05% | -0.12% |
| Class B | 0.00% | 0.00% | 0.00% | -0.24% | -0.19% | -0.18% |
| Class C | 0.00% | 0.00% | 0.00% | -0.21% | -0.20% | -0.17% |
| Class D | 0.00% | 0.00% | 0.00% | -0.06% | 0.02% | -0.03% |
| Class E | 0.00% | 0.00% | 0.00% | -0.13% | 0.02% | -0.03% |
| Overall | 0.00% | 0.00% | 0.00% | -0.17% | -0.09% | -0.11% |
|  | 0.00% | 0.00% | 0.00% | -0.17% | -0.09% | -0.11% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |

The simulation results reportedly show that the coding gain brought by F106 is 0.17% in LC case. It modified CAVLC only, so no performance change in HE case. It is the same as the results reported by the proponents.

## Performance of F269

Table – 4 F269 compared with HM-4.0 Anchor

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.42% | -0.01% | -0.01% | -0.29% | -0.09% | -0.11% |
| Class B | -0.38% | -0.06% | -0.05% | -0.32% | -0.22% | -0.14% |
| Class C | -0.36% | -0.14% | -0.18% | -0.36% | -0.27% | -0.24% |
| Class D | -0.25% | -0.06% | -0.08% | -0.20% | -0.05% | -0.09% |
| Class E | -0.39% | 0.06% | 0.10% | -0.29% | 0.01% | 0.14% |
| Overall | -0.36% | -0.05% | -0.05% | -0.29% | -0.14% | -0.10% |
|  | -0.36% | -0.05% | -0.06% | -0.29% | -0.14% | -0.11% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

The simulation results reportedly show that the coding gain brought by F269 is 0.36% in HE case, and 0.29% in LC case. It is the same as the results reported by the proponents.

# Conclusion

The implementation of each proposal was studied. Each software works as proposed.

All the tests results are the same as the results provided by the proponents.

The benefits from each proposal can be further combined to get more overall coding gain. And it was reported that the combination was made by the CE6 participants. The performance of the combined software is not included in this report.

# References

- A. Tabatabai, et. Al., “CE6: Intra Coding Improvements”, JCTVC- F906, Torino, July 2011.

- T.-D. Chuang, C.-Y. Chen, M. Guo, X. Guo, Y.-W. Huang, S. Lei, “Luma Intra Prediction Mode Coding”, JCTVC- F062, Torino, July 2011.

- E. Maani, A. Tabatabai, “Unifying binarizations of Intra modes in HE and LC”, JCTVC- F091, Torino, July 2011.

- J. Park, B. Jeon, “CAVLC coding for Intra Pred mode”, JCTVC- F106, Torino, July 2011.

- E. Francois, S. Pautet, C. Gisquet, “Modified Intra Mode Coding”, JCTVC- F269, Torino, July 2011.

- W.-J. Chien, X. Wang, M. Karczewicz, “Parsing friendly intra mode coding”, JCTVC- F459, Torino, July 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.