| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 Nov., 2011 | Document: JCTVC-G280 |
| --- | --- |

| Title: | CE6 Subset d: Intra Prediction with Secondary Boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Mei Guo, 1Xun Guo, 1Xin Zhao, 1Shawmin Lei, 2Jani Lainema, 2Kemal Ugur, 3Kazuo Sugimoto, 3Shunichi Sekiguchi, and 3Akira Minezawa 4Jinho Lee, 4Sung-Chang Lim, 4Hui Yong Kim, 4Jin Soo Choi. 1No. 2, Kexueyuan South Rd., Raycom Infotech Park, Beijing, China 100190 2Visiokatu 1, Tampere, Finland 3Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN 4ETRI, 218 Gajeongno, Yuseong-gu Daejeon, South Korea | Tel: Email: | +86-10-82800818 {mei.guo, xun.guo, x.zhao, shawmin.lei}@ mediatek.com {Jani.Lainema, Kemal.Ugur}@nokia.com Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp, Sekiguchi.Shunichi@eb. MitsubishiElectric.co.jp, Minezawa.Akira@ds. MitsubishiElectric.co.jp {jinosoul, sclim, hykim5, jschoi}@etri.re.kr |
| Source: | 1MediaTek Inc., 2Nokia, 3Mitsubishi, 4ETRI |  |  |

_____________________________

# Abstract

This contribution presents the experimental results of the intra prediction with secondary boundary for Core Experiment 6 (CE6) Subset d. The intra prediction with secondary boundary has been proposed to increase the coding efficiency of intra prediction in the contributions of JCTVC-F122, JCTVC-F172, JCTVC-F358, and JCTVC-F456 at Torino meeting. The predictors of vertical, horizontal, planar, and several angular modes are improved with the reference samples at the secondary boundary. It is reported that the proposed method can achieve an average bit-rate reduction of 0.7% for both high efficiency all-intra (HE-AI) and low complexity all-intra (LC-AI). The running time is reported to be similar to HM4.0.

# Introduction

In HM4.0, the intra prediction mode of a prediction unit (PU) is selected from up to 33 angular modes, DC mode and planar mode by encoder and signaled in the bitstream. Figure 1 presents the intra prediction modes with different angles for PU sizes of 8x8~32x32. In the figure, v and h denote vertical and horizontal intra prediction modes, respectively, and other prediction modes are represented as v±i (1≤i≤8) and h±i (1≤i≤8) with reference to the vertical and horizontal modes accordingly. Besides DC and Planar, only 16 and 2 angular modes are allowed for 4x4 and 64x64 PU, respectively. The angular intra prediction derives the predictor from one line of neighboring reference samples (the above row or the left column of the current block). Specifically, for a certain angle, the predictor is generated only using the neighboring samples in one end of the direction. At Torino meeting, four contributions F122 [1], F172 [2], F358 [3] and F456 [4] proposed to utilize more neighboring reference samples to improve the prediction efficiency, which have been included in CE6 subtest d. In this contribution, the methods in those four contributions are harmonized, which is described in Section 2 in details. The results are shown in Section 3.

Figure 1. 33 angular intra prediction modes (v: vertical; h: horizontal).

# Proposed Method

The proposed method has three techniques: the gradient based prediction, the bi-boundary based prediction, and the filtering for planar prediction. In the first technique, the predictor is compensated with the gradient along the direction of intra prediction, which is calculated with the reference samples from the secondary boundary. As illustrated in Figure 2, this technique is applied to VH set of modes (from v to v+2 and from h to h+2), which is highlighted with blue color. In the second technique, the proposed method generates the predictor with a weighted sum of two reference samples at both ends of the direction. This technique is applied to the DGL set of modes (from v+4 to v+8, from h+4 to h+8, from v-4 to v-8, and from h-4 to h-8), which is highlighted with green color in Figure 2. In the third technique, a filter is applied to the boundary of planar prediction. These three techniques are elaborated in the following sub-sections.

Figure 2. Two sets of intra prediction modes (blue: VH set; green: DGL set).

## Gradient based prediction

As shown in Figure 3, Pij denotes the predicted values at (i,j) using angular intra prediction in HM4.0. AL represents the reference sample at the left-above corner of the current block. Li represents the reference sample in the left column of the current block, and Aj represents the reference sample in the above row.

Figure 3. Intra prediction with gradient along the vertical direction.

Figure 4. Intra Prediction with gradient along the direction of v+1 or v+2.

As illustrated in Figure 3, the gradient-based predictor of vertical mode, P’ij, is calculated as

P'ij=Pij+((Li−AL)+(1<<j))>>(j+1).                  (1)

Analogous to the vertical mode, the predictor P’ij of horizontal mode is calculated as

P'ij=Pij+((Aj−AL)+(1<<i))>>(i+1).                  (2)

Due to the weak correlation, the predictor of vertical (horizontal) mode with j (i) larger than 1 will just stick to the angular predictor Pij.

As for the angular modes v+1~v+2 and h+1~h+2, Li or Aj first obtains its corresponding reference sample RLi or RAj along the direction of intra prediction. When RLi or RAj is not located at the position of integer samples, they are produced by interpolation of integer samples in the above row or the left column of the current block. As shown in Figure 4, the predictor P’ij of modes v+1~v+2 is calculated as

P'ij=Pij+((Li−RLi)+2)>>2,                          (3)

and the predictor P’ij of modes h+1~h+2 is calculated as

P'ij=Pij+((Aj−RAj)+2)>>2.                          (4)

In this case, the proposed method is only applied to the column with j=0 for modes v+1~v+2 and the row with i=0 for modes h+1~h+2, while other predictors just use the angular predictor Pij.

## Bi-boundary based prediction

As shown in Figure 5, for the modes in DGL set, a weighted sum of the angular predictor Pij and the reference sample across the neighboring boundary Fij is utilized as the final predictor P’ij. Table 1 gives the selection of Fij for different modes.

Table 1. The selection of Fij

| Intra Modes | Fij |
| --- | --- |
| v+8 | Li+1 |
| h+8 | Aj+1 |
| v+4~v+7 | Li+1 |
| h+4~h+7 | Aj+1 |
| v-4~v-8 | Li-1 |
| h-4~h-8 | Aj-1 |

The predictor P’ij for modes v+8 and h+8 is calculated as:

Pij'=((16−α)⋅Pij+α⋅Fij+8)>>4,                  (5)

where α=1<<(3-j) for mode v+8, and α=1<<(3-i) for mode h+8. Therefore, up to four pixel lines are filtered for these two modes. The predictor of mode v+8/h+8 with j/i larger than 3 still uses the angular predictor Pij.

For other modes in DGL set, only one pixel line is filtered, and the predictor P’ij is calculated as

Pij'=(6⋅Pij+2⋅Fij+4)>>3,                         (6)

Figure 5. Bi-boundary based prediction (v+4~v+8 modes)

## Filtering for planar prediction

A filter is applied to the boundary of planar prediction as same as filtering of DC prediction in HM4.0.

Figure 6. Filter coefficients for planar predicted samples

The process can be described as follows:If cIdx is equal to 0 and nS is less than 64, the following applies.

predSamples[ 0, 0 ] = ( 1*p[ -1, 0 ] + 2*predPlanar[0, 0] + 1*p[ 0, -1 ] + 2 ) >> 2
	predSamples[ x, 0 ] = ( 1*p[ x, -1 ] + 3*predPlanar[x, 0] + 2 ) >> 2, with x = 1..nS-1
	predSamples[ 0, y ] = ( 1*p[ -1, y ] + 3*predPlanar[0, y] + 2 ) >> 2, with y = 1..nS-1
	predSamples[ x, y ] = predPlanar[x, y], with x, y = 1..nS-1

Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = predPlanar[x, y], with x, y = 0..nS-1

# Simulation Results

Simulations are based on HM4.0 and the recommended test conditions in JCTVC-F900 [5]. Table 2 gives the coding performance and running time of the proposed method. An average BD-Rate reduction of 0.7% is achieved. Moreover, there is only minor running time increase.

Table 2. Coding performance of the proposed method

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.93% | -0.82% | -0.86% | -0.83% | -0.98% | -1.12% |
| Class B | -0.63% | -0.60% | -0.59% | -0.57% | -0.78% | -0.78% |
| Class C | -0.64% | -0.65% | -0.66% | -0.68% | -0.77% | -0.73% |
| Class D | -0.66% | -0.64% | -0.65% | -0.70% | -0.69% | -0.73% |
| Class E | -0.80% | -0.70% | -0.87% | -0.75% | -0.88% | -0.92% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.72% | -0.68% | -0.71% | -0.70% | -0.82% | -0.85% |
|  | -0.73% | -0.69% | -0.71% | -0.70% | -0.82% | -0.85% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Conclusion

The methods in CE6 subset d are harmonized. An average BD-Rate reduction of 0.7% for high efficiency all-intra (HE-AI) and low complexity all-intra (LC-AI) is achieved with minor increase of the running time. We recommend to adopt the proposed method as a part of HEVC standard.

# References

- M. Guo, X. Zhao, X. Guo, and S. Lei, “Direction based angular intra prediction”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F122, 6th Meeting: Torino, IT, 14-22 July, 2011.

- A. Minezawa, K. Sugimoto, and S. Sekiguchi, “An improved intra vertical and horizontal prediction”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F172, 6th Meeting: Torino, IT, 14-22 July, 2011.

- J. Lee, S.-C. Lim, H. Y. Kim, and J. S. Choi, “Mode dependent filtering for intra predicted sample”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F358, 6th Meeting: Torino, IT, 14-22 July, 2011.

- J. Lainema, and K. Ugur, “Directional intra prediction smoothing”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F456, 6th Meeting: Torino, IT, 14-22 July, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mitsubishi may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI (Electronics and Telecommunications Research Institute) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).