| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH., 21-30, Nov., 2011 | Document: JCTVC-G281 |
| --- | --- |

| Title: | Non-CE7: Boundary-Dependent Transform for Inter-Predicted Residue |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | *Jicheng An, *Xin Zhao, *Xun Guo, #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | {jicheng.an, x.zhao, xun.guo, shawmin.lei}@mediatek.com |  |
| Source: | MediaTek Inc. |  |  |  |

_____________________________

# Abstract

This contribution proposes a boundary-dependent transform for inter prediction, in which different transforms (DST-VII/DCT-IV or DCT-II) are selected depending on whether the Transform Unit (TU) boundary is a Prediction Unit (PU) boundary. It is reported that the proposed method can achieve around 0.2% BD-rate reduction for RA-HE and RA-LC configurations, and 0.5% BD-rate reduction for LD-HE and LD-LC configurations. The run time is reportedly similar to that of HM4.0. It is also reported that there is no new transforms imported for the proposed method.

# Introduction

The Discrete Cosine Transform type II (DCT-II) is often used in signal processing, especially for the lossy data compression, because it has a strong "energy compaction" property [1]. Most of the signal information tends to be concentrated in a few low-frequency components of the DCT-II which approximates the Karhunen-Loève Transform (KLT, which is optimal in the decorrelation sense) for signals based on certain limits of Markov processes. In [2], the KLT for a class of random processes is proven to be a set of periodic sine functions.

The N-point DCT-II of the signal f[n] is defined as (1).

(1)

For intra-predicted residue, there are other transforms to be more efficient than DCT-II. Mode-Dependent Directional Transform (MDDT) was previously introduced to improve transform coding of intra-predicted blocks [3]. The theory of DCT/DST was presented for the first time for intra-predicted residue in [4]. In JCTVC-B024[5], JCTVC-C108[6], JCTVC-E125[7], the theory of DCT/DST was presented in JCTVC for oblique intra modes.

For inter-predicted residue, DCT-II is the only transform used in current HEVC [8]. However, the DCT-II is not the optimal transform for all cases. In this contribution, the Discrete Sine Transform type VII (DST-VII) and Discrete Cosine Transform type IV (DCT-IV) [9] are proposed to replace DCT-II in some cases.

# Motivation

When using inter prediction for a PU, generally the prediction error are larger near the PU boundaries than in the middle of the PU. The numbers in Fig. 1 show an example of the square error distribution over a 4x4 PU.

Fig. 1. The square error distribution within a 4x4 PU

Similarly, when a PU is split into multiple TUs as shown in Fig. 2, the prediction error are larger near PU boundaries than near interior TU (non-PU) boundaries. The numbers in Fig. 2 show the square error distribution in TU0 at the top-left corner of the PU. It can be seen that the error decreases basically from top to bottom and from left to right.

Fig. 2 A split PU and the error distribution within the top-left TU

The reason of this effect may be due to the different motion vectors (MV) between two neighboring PUs. In order to handle this uneven error distribution, alternative transforms such as DST-VII and DCT-IV can be used. Equation (2) and (3) show the N-point DST-VII and DCT-IV of signal f[n], respectively. Please refer to Appendix A and B to get more information for DST-VII and DCT-IV.

(2)

(3)

# Proposed boundary-dependent transform

## Mapping from boundary type to transform

Based on the above observation, it is proposed to use DST-VII or DCT-IV instead of DCT-II if one and only one of the two TU boundaries is a PU boundary. Table 1 shows the mapping from boundary type to transform by using the DST-VII. Table 2 shows the mapping from boundary type to transform by using the DCT-IV. The proposed DST-VII and DCT-IV are not used for 32-point (pt) transform for little gain and high complexity.

Table 1. Mapping from boundary type to transform by using the DST-VII (4-pt)

| TU Boundary |  | Horizontal Trans. | TU Boundary |  | Vertical Trans. |
| --- | --- | --- | --- | --- | --- |
| Left | Right |  | Top | Bottom |  |
| non-PU | PU | DST-VII | non-PU | PU | DST-VII |
| PU | non-PU | F(DST-VII) | PU | non-PU | F(DST-VII) |
| PU | PU | DCT-II | PU | PU | DCT-II |
| non-PU | non-PU | DCT-II | non-PU | non-PU | DCT-II |

Table 2. Mapping from boundary type to transform by using the DCT-IV (8-pt and 16-pt)

| TU Boundary |  | Horizontal Trans. | TU Boundary |  | Vertical Trans. |
| --- | --- | --- | --- | --- | --- |
| Left | Right |  | Top | Bottom |  |
| non-PU | PU | F(DCT-IV) | non-PU | PU | F(DCT-IV) |
| PU | non-PU | DCT-IV | PU | non-PU | DCT-IV |
| PU | PU | DCT-II | PU | PU | DCT-II |
| non-PU | non-PU | DCT-II | non-PU | non-PU | DCT-II |

The F(DST-VII) in Table 1 means flipping the DST matrix from left to right. The cases of using F(DST-VII) can also be implemented as flipping the input data first and then using DST-VII. It is also the case for F(DCT-IV). In Table 1 and Table 2, the items “non-PU” and “PU” mean non-PU boundary and PU boundary, respectively.

According to the Table 1, the four TUs in Fig. 3 should take transforms as shown in Table 3.

Table 3． Transform used for the TUs in Fig. 3

| TU | Horizontal Trans. | Vertical Trans. |
| --- | --- | --- |
| TU0 | F(DST-VII) | F(DST-VII) |
| TU1 | DST-VII | F(DST-VII) |
| TU2 | F(DST-VII) | DST-VII |
| TU3 | DST-VII | DST-VII |

## Transform used for different TU sizes

In order not to import new transform, it is proposed to use DST-VII for 4-pt transform, and DCT-IV for 8-pt and 16-point transforms. The 4-pt DST-VII has been used in HEVC for intra coding. The 8-pt and 16-point DCT-IV can be performed by reusing the circuits of larger DCT-II transform, which is already in HEVC.

The feasible analysis of this circuits sharing is as follows.

Regrouping the terms f[n] and f[N-1-n] of a DCT-II in (1) yields the following equations：

(4)

(5)

Fig. 3 N-point DCT-II

The even index coefficients of the DCT-II are thus equal to the DCT-II of N/2-pt signal f[n]+f[N-1-n]. The odd index coefficients of the DCT-II are equal to the DCT-IV of N/2-pt signal f[n]-f[N-1-n]. Fig. 3 shows the flow chart of N-pt DCT-II. That is also the partial butterfly realization structure in current HEVC Test Model version 4.0 (HM4.0) [10]. In current CE10 (Core Transform), the DCT-IV can be further realized by full-factorized fast algorithms [11][12]. Therefore, the 8-pt and 16-pt DCT-IV can share the partial logic of 32-pt DCT-II so that the implementation cost is very low. The similar analysis was also presented in JCTVC-D235[], which showed that DST-IV (a flipped version of DCT-IV) can be realized using DCT-II.

Therefore, the proposed solution is to use the Table 1 for 4-pt transform and Table 2 for 8-pt and 16-pt transforms.

# Experimental Results

The proposed boundary-dependent transform has been implemented on top of the HM4.0. The experiments are conducted according to the common test condition JCTVC-F900 [13]. Table 3 shows the BD-rate performance (using Table 1 for 4-pt transform and Table 2 for 8-pt and 16-pt transforms).

Table 3 BD-rate reduction of using DST-VII (Table 1) for 4-pt transform and DCT-IV (Table 2) for 8-pt and 16-pt transforms

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | -0.1% | -0.3% | 0.1% | 0.1% |
| Class B | -0.2% | 0.2% | 0.3% | -0.2% | 0.1% | 0.1% |
| Class C | -0.2% | 0.0% | 0.1% | -0.2% | 0.1% | 0.1% |
| Class D | -0.2% | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.0% | 0.1% | -0.2% | 0.1% | 0.1% |
|  | -0.2% | 0.0% | 0.1% | -0.2% | 0.1% | 0.1% |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | 0.1% | 0.1% | -0.5% | 0.2% | 0.0% |
| Class C | -0.5% | -0.2% | 0.0% | -0.5% | 0.2% | -0.2% |
| Class D | -0.4% | 0.5% | 0.1% | -0.4% | 0.2% | 0.2% |
| Class E | -0.5% | 0.2% | 0.3% | -0.6% | 0.6% | -0.2% |
| Overall | -0.5% | 0.1% | 0.1% | -0.5% | 0.3% | 0.0% |
|  | -0.5% | 0.2% | 0.1% | -0.5% | 0.2% | 0.0% |

Table 4~7 show the supplementary experimental results by applying the DST-VII or DCT-IV to different transform sizes.

Table 4 shows the performance of using the DST-VII for 4-pt, 8-pt, and 16-pt transforms. It can be seen that around 0.4% BD-rate reduction with similar runtime can be achieved.

Table 5~7 show the performance of using the DST-VII for different transform sizes. It can be seen from Table 4 and Table 7 that the DST-VII has slightly better BD-rate performance than DCT-IV and the proposed solution is a good tradeoff between complexity and BD-rate gain.

Table 4. BD-rate increase of using DST-VII (Table 1) for 4-pt, 8-pt, and 16-pt transforms

|  | Random Acess HE |  |  | Random Acess LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.1% | -0.2% | -0.4% | -0.1% | -0.2% |
| Class B | -0.2% | 0.1% | 0.1% | -0.3% | 0.1% | 0.2% |
| Class C | -0.2% | 0.0% | 0.0% | -0.2% | 0.1% | 0.0% |
| Class D | -0.2% | 0.1% | 0.0% | -0.3% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
|  | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | -0.1% | -0.2% | -0.6% | 0.1% | 0.1% |
| Class C | -0.5% | 0.0% | -0.2% | -0.5% | 0.3% | 0.0% |
| Class D | -0.4% | 0.1% | 0.0% | -0.4% | 0.3% | -0.1% |
| Class E | -0.6% | 0.3% | 0.8% | -0.6% | 0.8% | 0.2% |
| Overall | -0.5% | 0.0% | 0.0% | -0.5% | 0.3% | 0.0% |
|  | -0.5% | 0.0% | 0.0% | -0.5% | 0.3% | 0.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 5. BD-rate increase of using DST-VII (Table 1) for 4-pt transform

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.3% | -0.2% | 0.1% | 0.0% |
| Class B | -0.1% | 0.1% | 0.0% | -0.2% | 0.1% | 0.2% |
| Class C | -0.2% | -0.1% | 0.0% | -0.2% | 0.1% | 0.0% |
| Class D | -0.2% | 0.0% | 0.0% | -0.3% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | 0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | 0.1% |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | 0.2% | -0.3% | 0.1% | 0.2% |
| Class C | -0.2% | 0.1% | -0.1% | -0.3% | 0.1% | -0.1% |
| Class D | -0.3% | 0.4% | -0.1% | -0.3% | 0.5% | 0.2% |
| Class E | -0.2% | -0.1% | 0.7% | -0.4% | 0.3% | 0.4% |
| Overall | -0.2% | 0.1% | 0.2% | -0.3% | 0.3% | 0.1% |
|  | -0.2% | 0.1% | 0.1% | -0.3% | 0.3% | 0.1% |

Table 6. BD-rate increase of using DST-VII (Table 1) only for 4-pt and 8-pt transforms

|  | Random Acess HE |  |  | Random Acess LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.3% | -0.3% | -0.1% | -0.2% |
| Class B | -0.2% | 0.1% | 0.1% | -0.3% | 0.1% | 0.2% |
| Class C | -0.2% | 0.0% | 0.1% | -0.2% | 0.1% | -0.1% |
| Class D | -0.2% | 0.1% | 0.0% | -0.3% | -0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
|  | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.1% | -0.1% | -0.5% | 0.1% | 0.0% |
| Class C | -0.4% | -0.2% | -0.1% | -0.4% | 0.2% | 0.0% |
| Class D | -0.3% | 0.0% | 0.1% | -0.4% | 0.5% | 0.4% |
| Class E | -0.4% | -0.2% | 0.4% | -0.7% | 0.6% | 0.2% |
| Overall | -0.4% | 0.0% | 0.1% | -0.5% | 0.3% | 0.2% |
|  | -0.4% | -0.1% | 0.1% | -0.5% | 0.3% | 0.2% |

Table 7. BD-rate increase of using DCT-IV (Table 2) for 4-pt, 8-pt, and 16-pt transforms

|  | Random Acess HE |  |  | Random Acess LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | 0.1% | -0.2% | 0.3% | 0.0% |
| Class B | -0.2% | 0.3% | 0.3% | -0.2% | 0.2% | 0.2% |
| Class C | -0.2% | 0.0% | 0.0% | -0.2% | 0.1% | 0.0% |
| Class D | -0.2% | 0.1% | 0.1% | -0.2% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.1% | 0.1% | -0.2% | 0.1% | 0.1% |
|  | -0.2% | 0.1% | 0.1% | -0.2% | 0.2% | 0.1% |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | 0.1% | 0.3% | -0.5% | 0.0% | 0.1% |
| Class C | -0.4% | -0.1% | -0.2% | -0.4% | 0.2% | -0.1% |
| Class D | -0.4% | 0.5% | -0.1% | -0.4% | 0.1% | -0.1% |
| Class E | -0.7% | -0.2% | 0.4% | -0.7% | 0.5% | 0.1% |
| Overall | -0.5% | 0.1% | 0.1% | -0.5% | 0.2% | 0.0% |
|  | -0.5% | 0.1% | 0.1% | -0.5% | 0.1% | 0.0% |

# Conclusion

In this proposal, a boundary-dependent transform is proposed to select the transform depending on whether the TU boundary is a PU boundary or not, which can adapt the inter-predicted residual distribution better than the only-DCT-II method. It is proposed to use 4-pt DST-VII and 8-pt and 16-pt DCT-IV depending on the TU boundary type. Since the 4-pt DST-VII has been in current HM4.0, and 8-pt and 16-pt DCT-IV can be realized as part of larger size DCT-II, the increased complexity of the proposed method would be very low.

The proposed method can achieve 0.2% BD-rate gain for RAHE and RALC configurations and 0.5% BD-rate gain for LDHE and LDLC confugurations with similar runtime compared to HM4.0. It is suggested to include this boundary-dependent transform into HEVC.

# References

- K. R. Rao and P. Yip, “Discrete Cosine Transform: Algorithms, Advantages, Applications”, Academic Press, Boston, 1990.

- A. K. JAIN, “A Fast Karhunen-Loeve Transform for a Class of Random Processes”, IEEE Trans. Comm., 1023-1029, 1976.

- Y. Ye and M. Karczewicz, “Improved Intra Coding,” ITU-T Q.6/SG11 VCEG, VCEG-AG11, Shenzhen, China, Oct 2007.

- J. Han, A. Saxena, and K.Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding”, IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

- C. Yeo, Y. Tan, Z. Li, and S. Rahardja, “Mode-Dependent Fast Separable KLT for Block-based Intra Coding”, JCTVC-B024, July, 2010.

- A. Saxena, and F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform”, JCTVC-C108, Oct., 2010.

- A. Saxena, and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4x4 full matrix multiplication for intra prediction”, JCTVC-E125, Mar., 2011.

- T. Wiegand, W.J. Han, B. Bross, J.-R. Ohm, and G. J. Sullivan, “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, Torino, IT, July, 2011.

- S. A. Martucci, "Symmetric convolution and the discrete sine and cosine transforms," IEEE Trans. Sig. Processing SP-42, 1038-1051, 1994.

- K. McCann, B. Bross, S. Sekiguchi, W.-J. Han, “HM4: High Efficiency Video Coding (HEVC) Test Model 4 Encoder Description”, JCTVC-F802, Torino, IT, July, 2011.

- E.Alshina, A. Alshin, Il-Koo Kim, and P. Topiwala, “CE10: Full-factorized core transform proposal by Samsung/FastVDO”, JCTVC-F251, Torino, IT, July, 2011.

- R. Joshi, Y. Reznik, J. Sole, and M. Karczewicz, “CE10: Scaled orthogonal integer transforms supporting recursive factorization structure”, JCTVC-F352, Torino, IT, July, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, IT, July, 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A

The norm-corrected DST-VII matrixes implemented on top of HM4.0 are as follows:

4-point DST-VII matrix:

8-point DST-VII matrix:

16-point DST-VII matrix:

# Appendix B

The norm-corrected DCT-IV matrixes implemented on top of HM4.0 are as follows:

4-point DCT-IV matrix:

8-point DCT-IV matrix:

16-point DCT-IV matrix: