| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G282 |
| --- | --- |

| Title: | Non-CE7: Mode-dependent DCT/DST for Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *#Xin Zhao, *Xun Guo, *Mei. Guo, *Shawmin Lei, #Siwei Ma, #Wen Gao *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Email: | {x.zhao, xun.guo, mei.guo, shawmin.lei}@mediatek.com {swma, wgao}@pku.edu.cn |
| Source: | *MediaTek Inc., #Peking University |  |  |

_____________________________

# Abstract

In JCTVC-F553, the existing mode-dependent DCT/DST method was proposed to be extended to 4x4 chroma blocks for horizontal and vertical intra prediction modes. In this contribution, besides horizontal and vertical modes, the mode-dependent DCT/DST is also applied for other chroma intra prediction modes, including the planar and right-down diagonal mode. It is reported that for AI-HE configuration, the proposed method can achieve an average BD-Rate reductions of 0.0%, 0.6%, and 0.7% for Y, U and V components, respectively. And for AI-LC configuration, the average BD Rate reductions are 0.0%, 1.1%, and 1.2% for Y, U and V components, respectively. It is also reported that there is no run time increase for both encoder and decoder.

# Introduction

In HM4.0, a mode-dependent DCT/DST method [1] is used for 4x4 luma intra prediction, in which one of DCT and DST is applied according to the intra prediction modes. A mapping table between intra modes and transform is used for this purpose. This method is not used for chroma component.

There are 6 intra chroma modes including DM, LM, planar, horizontal, vertical and DC modes. If one of planar, horizontal, vertical and DC modes is identical to DM mode, it is replaced by right-down diagonal mode. In JCTVC-F553 [2], the mode-dependent DCT/DST was proposed to be also applied on chroma components. In specific, the method was used for horizontal and vertical modes of 4x4 blocks when they are not equal to DM mode. In this contribution, besides horizontal and vertical modes, we propose to also use this method for Planar and right-down diagonal mode.

# Proposed method

We use the proposed method only for chroma 4x4 blocks so that no new DST transform would be imported. Selection of horizontal and vertical transform for chroma intra prediction modes is shown in Table 1. For the modes other than DM mode, the usage of mode-dependent DCT/DST is the same with that of luma intra prediction modes. For DM mode, DST is used only when it is equal to Planar mode.

Table 1 Selection of horizontal/vertical transform for 4x4 intra chroma blocks

| Chroma mode | Horizontal Transform | Vertical Transform |
| --- | --- | --- |
| DM= Planar | DST | DST |
| DM≠Planar | DCT | DCT |
| LM | DCT | DCT |
| Planar | DST | DST |
| Horizontal | DST | DCT |
| Vertical | DCT | DST |
| DC | DCT | DCT |
| right-down diagonal mode | DCT | DST |

# Simulations and results

The proposed method is implemented on top of HM 4.0. Simulations are performed under all intra configurations defined in [3]. Four tests are performed as below:

T1: DCT/DST is applied for “Horizontal” and “Vertical” only.

T2: DCT/DST is applied for “Planar” and “DM=Planar”

T3: DCT/DST is applied for “Planar” and “DM=Planar” and “Right-down diagonal”

T4: DCT/DST is applied for “DM=Planar”, “Planar”, “Horizontal”, “Vertical” and “Right-down diagonal”.

The BD-rate reductions for the above tests are reported in Table 2 ~ Table 5, respectively.

Table 2 Experimental results of T1

|  | Intra, HE |  |  | Intra, Loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.4% |
| Class B | 0.0% | -0.3% | -0.3% | 0.0% | -0.5% | -0.5% |
| Class C | 0.0% | -0.4% | -0.6% | 0.0% | -0.8% | -0.9% |
| Class D | 0.0% | -0.7% | -0.8% | 0.0% | -0.9% | -1.1% |
| Class E | 0.0% | -0.1% | -0.2% | 0.0% | -0.3% | -0.5% |
| All Avg | 0.0% | -0.4% | -0.4% | 0.0% | -0.6% | -0.7% |

Table 3 Experimental results of T2

|  | Intra, HE |  |  | Intra, Loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.2% | 0.0% | -0.6% | -0.5% |
| Class B | 0.0% | -0.5% | -0.5% | 0.0% | -0.9% | -0.8% |
| Class C | -0.1% | -0.7% | -0.8% | -0.1% | -1.1% | -1.2% |
| Class D | -0.1% | -0.9% | -1.0% | -0.1% | -1.3% | -1.4% |
| Class E | 0.0% | 0.0% | -0.3% | 0.0% | -0.5% | -0.9% |
| All Avg | 0.0% | -0.5% | -0.6% | -0.1% | -0.9% | -1.0% |

Table 4 Experimental results of T3

|  | Intra, HE |  |  | Intra, Loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.3% | 0.0% | -0.6% | -0.5% |
| Class B | 0.0% | -0.6% | -0.5% | 0.0% | -0.9% | -0.8% |
| Class C | -0.1% | -0.7% | -0.9% | -0.1% | -1.1% | -1.3% |
| Class D | -0.1% | -1.0% | -1.1% | -0.1% | -1.3% | -1.5% |
| Class E | 0.0% | -0.1% | -0.3% | 0.0% | -0.5% | -0.9% |
| All Avg | 0.0% | -0.6% | -0.6% | -0.1% | -0.9% | -1.0% |

Table 5 Experimental results of T4

|  | Intra, HE |  |  | Intra, Loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.3% | 0.0% | -0.7% | -0.7% |
| Class B | 0.0% | -0.6% | -0.6% | 0.0% | -1.1% | -1.0% |
| Class C | -0.1% | -0.8% | -1.1% | -0.1% | -1.4% | -1.5% |
| Class D | -0.1% | -1.2% | -1.3% | -0.1% | -1.6% | -1.8% |
| Class E | 0.0% | -0.1% | -0.3% | 0.0% | -0.5% | -1.1% |
| All Avg | 0.0% | -0.6% | -0.7% | 0.0% | -1.1% | -1.2% |
| Enc Time (%) | 99% |  |  | 100% |  |  |
| Dec Time (%) | 97% |  |  | 97% |  |  |
|  | RA, HE |  |  | RA, Loco |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.6% | -0.7% | 0.0% | -0.7% | -0.9% |
| Class B | 0.0% | -1.0% | -1.1% | 0.0% | -1.0% | -0.9% |
| Class C | 0.0% | -1.0% | -1.0% | 0.0% | -1.2% | -1.2% |
| Class D | 0.0% | -0.9% | -1.6% | 0.0% | -1.2% | -0.9% |
| Class E |  |  |  |  |  |  |
| All Avg | 0.0% | -0.9% | -1.1% | 0.0% | -1.0% | -1.0% |
| Enc Time (%) | 100% |  |  | 100% |  |  |
| Dec Time (%) | 100% |  |  | 101% |  |  |
|  | LB, HE |  |  | LB, Loco |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.6% | 0.0% | -0.2% | -0.3% |
| Class C | 0.0% | -0.3% | -0.4% | 0.0% | -0.2% | -0.7% |
| Class D | 0.0% | -0.1% | -0.2% | 0.0% | -0.4% | -0.4% |
| Class E | 0.0% | -0.3% | 0.1% | -0.1% | -0.9% | -0.9% |
| All Avg | 0.0% | -0.2% | -0.3% | 0.0% | -0.4% | -0.5% |
| Enc Time (%) | 100% |  |  | 100% |  |  |
| Dec Time (%) | 98% |  |  | 100% |  |  |

# Conclusion

In this contribution, the mode dependent DCT/DST method is extended to 4x4 chroma blocks for planar, horizontal, vertical and Right-down diagonal modes. Average BD Rates of -0.0%, -0.6% and -0.7% are reported for Y, U and V in AI-HE case, while average -0.0%, -1.1% and -1.2% are reported in AI-LC case. Moreover, the run time is almost the same with that of HM4.0 for both encoder and decoder.

# References

[1] A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction," JCTVC-E125, Geneva, Switzerland, March 2011

[2] A. Saxena, F. Fernandes, E. Alshina and J. Chen, “Mode-Dependent DCT/DST for 4x4 Chroma Blocks”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F553, 6th Meeting: Torino, IT, 14-22 July, 2011.

[3] F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Patent rights declaration

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

The proposed text changes to the working draft are provided as below,

### Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit,

–	a (nW)x(nH) array d of scaled transform coefficients with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

Output of this process is residual samples as a (nW)x(nH) array r with elements rij.

The variable trafoPrecisionExt is derived as follows:

–	If nW is greater than 4 or nH is greater than 4,

- If cIdx is equal to 0 and bit_depth_luma_minus8 is equal to 0, trafoPrecisionExt is set equal to 4.

- Otherwise, if cIdx is not equal to 0 and bit_depth_chroma_minus8 is equal to 0, trafoPrecisionExt is set equal to 4.

- Otherwise, trafoPrecisionExt is set equal to 0.

–	Otherwise, trafoPrecisionExt is set equal to 0.

Depending on PredMode and IntraPredMode, the following applies:

–	If the current block is luma block, PredMode is equal to MODE_INTRA, log2(nW*nH) is equal to 4, and cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Error: Reference source not found with IntraPredMode as input.

–	Otherwise, if the current block is chroma block, PredMode is equal to MODE_INTRA, log2(nW*nH) is equal to 4, and cIdx is equal to 0,

–	If the IntraPredMode is equal to 36, if the IntraPredMode of the top-left luma block of the current block is equal to 0, the variables horizTrType and vertTrType are set equal to 0.

–	Otherwise, if IntraPredMode is not equal to 35, the variables horizTrType and vertTrType are specified as Error: Reference source not found with IntraPredMode as input.

–	Otherwise, both the variables horizTrType and vertTrType are equal to 0.

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.