| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G283 |
| --- | --- |

| Title: | Residue Quad Tree Depth for Chroma in Intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *#Xin Zhao, *Xun Guo, *Xiang Li, *Shawmin Lei, #Siwei Ma, #Wen Gao, *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Email: | {x.zhao, xun.guo, x.li, shawmin.lei}@mediatek.com {swma, wgao}@pku.edu.cn |
| Source: | *MediaTek Inc., #Peking University |  |  |

_____________________________

# Abstract

In HM4.0, luma and chroma components share the same maximum Residue Quad Tree (RQT) depth. This contribution proposes to use separate maximum RQT depth for luma and chroma components in intra coding. In specific, a user-defined parameter is added in the sequence parameter set (SPS) to allow independent RQT depth setting for chroma. It is reported that with setting the parameter as 0, average BD-Rate reductions of 0.04%, 1.67% and 1.70% are achieved for Y, U and V in AI-HE, respectively, and 0.20%, 2.43% and 2.53% are achieved for Y, U and V in AI-LC, respectively. It is also reported that the decoding time is slightly decreased.

# Introduction

In HEVC, RQT is applied to allow flexible transform unit (TU) sizes inside each coding unit (CU), and the optimal RQT depth for luma component is decided using rate-distortion optimization (RDO) and signaled in the bitstream. For chroma component, the transform depth of corresponding luma component is simply reused, i.e., luma and chroma components share the same transform depth. However, in JCTVC-D060 [1], it is reported that the reuse of luma RQT depth may results in coding loss for chroma. JCTVC-E377 [2] proposed to limit the maximum chroma transform depth relative to luma, so that different transform depth can be used for chroma. Meanwhile, 32x32 transform is introduced for chroma component, which is not included in the original design of HEVC. This contribution proposes to allow independent maximum RQT depth for chroma, while no additional transform is imported.

# Proposed method

In this contribution, a user-defined parameter max_transform_depth_intra_chroma is proposed, which defines the maximum RQT depth for chroma in intra coding. As shown in Table 1, the proposed parameter is coded after max_transform_hierarchy_depth_intra, which specifies the maximum RQT depth for luma in intra coding. The default value for max_transform_hierarchy_depth_intra_chroma is 1, which means RQT depth is 0 for chroma. However, for 64x64 CU, RQT depth 1 is still used to avoid the usage of 32x32 transform for chroma. The RQT depth setting of chroma for this default case is illustrated in Table 2.

Table 1 Proposed user-defined parameters in the SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | … |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| max_transform_ depth_intra_chroma | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| … | … |
| } |  |

Table 2 RQT depth for chroma when max_transform_hierarchy_depth_intra_chroma is default

| CU size | 64x64 | 32x32 | 16x16 | 8x8 |
| --- | --- | --- | --- | --- |
| RQT depth | 1 | 0 | 0 | 0 |
| Chroma transform size | 16x16 | 16x16 | 8x8 | 4x4 |

# Simulations and results

The proposed method is implemented on top of HM 4.0. Simulations are performed under all intra configurations defined in [4]. The BD-rate reductions are reported in Table 3. According to the results, average BD-Rate reductions of 1.67% and 1.70% are achieved for U and V respectively in AI-HE. And average BD-Rate reductions of 2.43% and 2.53% are achieved for U and V respectively in AI-LC. Moreover, a BD-Rate reduction of 0.2% is also observed for luma in AI-LC.

Table 3 Experimental results of proposed method

|  | AI-HE |  |  | AI-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -1.5% | -1.4% | -0.1% | -2.0% | -2.3% |
| Class B | -0.1% | -1.8% | -1.8% | -0.3% | -2.4% | -3.0% |
| Class C | 0.0% | -1.3% | -1.5% | -0.2% | -1.5% | -1.9% |
| Class D | 0.0% | -1.5% | -1.7% | -0.1% | -1.1% | -1.2% |
| Class E | -0.1% | -2.3% | -2.2% | -0.2% | -5.9% | -4.7% |
| All Avg | -0.04% | -1.67% | -1.70% | -0.20% | -2.43% | -2.53% |
| Enc Time (%) | 99% |  |  | 99% |  |  |
| Dec Time (%) | 96% |  |  | 96% |  |  |

# Conclusion

In this contribution, a user-defined parameter is proposed to be added in SPS to allow separate RQT depth setting for luma and chroma in Intra coding. For AI-HE, the proposed default setting achieves average BD Rates -0.04%, -1.67% and -1.70% for Y, U and V, respectively. For AL-LC, the average BD Rates are -0.10%, -1.87% and -2.00% for Y, U and V, respectively.

# References

[1] M. Zhou and A. Tabatabai, “Evaluation Results on Residual Quad Tree (RQT)”, JCTVC-D060, JCT-VC Meeting, Daegu, KR, Jan. 2011.

[2] L. Guo, X. Wang, P. Chen, Y. Chen, M. Karczewicz, “Limiting Chroma Transform Depth in Residue Quad Tree (RQT)”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E377, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[3] F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Patent rights declaration

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

The proposed text changes to the working draft are provided as below,

7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | … |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| max_transform_depth_intra_chroma | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| … | … |
| } |  |

7.4.2.1 Sequence parameter set RBSP semantics

…

max_transform_hierarchy_depth_intra specifies the maximum hierarchy depth for transform blocks of coding blocks coded in intra prediction mode.

max_transform_hierarchy_depth_inter specifies the maximum hierarchy depth for transform units of coding units coded in inter prediction mode.

max_transform_depth_intra_chroma specifies the maximum hierarchy depth for transform blocks of chroma coding blocks coded in intra prediction mode.

chroma_pred_from_luma_enabled_flag equal to 1 specifies the intra chroma prediction process using the reconstructed luma block is applied according to the intra chroma prediction mode.

…

7.3.8 Transform tree syntax

| transform_tree( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx, chromaFlag ) { | Descriptor |
| --- | --- |
| …… | …… |
| if( residualDataPresentFlag) { |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && entropy_coding_mode_flag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| …… | …… |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| …… | …… |
| transform_tree( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0, chromaFlag ) |  |
| transform_tree( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1, chromaFlag ) |  |
| transform_tree( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2, chromaFlag ) |  |
| transform_tree( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3, chromaFlag ) |  |
| } else if( entropy_coding_mode_flag ) { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize && PredMode != MODE_INTRA ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth – 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth – 1] |  |
| if ( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( PredMode = = MODE_INTRA ) |  |
| if( log2TrafoSize > Log2MinTrafoSize && chromaFlag ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| chromaFlag = 0 |  |
| } else if( blkIdx = = 0 && chromaFlag ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth − 1 ] | ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth − 1 ] | ae(v) |
| chromaFlag = 0 |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.9 Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| …… | …… |
| if(log2TrafoWidth > Log2MaxTrafoSize){ |  |
| leafTrafoDepth =1 |  |
| }else{ |  |
| leafTrafoDepth =0 |  |
| } |  |
| If( ( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) ¦ ¦ (leafTrafoDepth > trafoDepth && cIdx != 0 ){ |  |
| …… | …… |
| transform_coeff( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| …… | …… |
| if ( entropy_coding_mode_flag ) |  |
| if ( scanIdx = = 0 ) |  |
| scanIdx = 3 |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, cIdx ) |  |
| } |  |
| } |  |
| } |  |