| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G284 |
| --- | --- |

| Title: | CE11: Extended Mode Dependent Coefficient Scanning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *#Xin Zhao, *Xun Guo, *Mei Guo, *Shawmin Lei, #Siwei Ma, #Wen Gao, , *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 | Email: | {x.zhao, xun.guo, mei.guo, shawmin.lei}@mediatek.com {swma, wgao}@pku.edu.cn |
| Source: | *MediaTek Inc., #Peking University |  |  |

_____________________________

# Abstract

In JCTVC-F124, an extended mode dependent coefficient scanning (MDCS) method was proposed for TU sizes of 16x16 and 32x32. In this contribution, the simulation results of the extended MDCS are reported on top of HM4.0. Two sets of modified horizontal and vertical scanning patterns for 16x16 and 32x32 are tested, including arbitrary scanning patterns (M0) derived from theoretical analysis, and its simplified version (M1). It is reported that by using M0, the BD-rate reductions for AI-HE and AI-LC are -0.2% and -0.6%, respectively. And by using M1, the BD-rate reductions for AI-HE and AI-LC are -0.1% and -0.5%, respectively.

# Introduction

In HM4.0, MDCS [1] is used in intra coding. In high efficiency (HE) configuration, three scanning patterns are used to code the significance map and coefficients level including diagonal, horizontal and vertical scans. In low complexity (LC) configuration, zigzag scan is used instead of diagonal scan. Therefore, totally four scanning patterns are used in HM4.0 as illustrated in Figure 1.

(0)			(1)			(2)			(3)

Figure 1 Four scanning patterns in HM4.0: zigzag (0), horizontal (1), vertical (2) and diagonal (3).

The selection of the scanning patterns for each transform unit (TU) size is according to intra prediction modes, as shown in table 1. Currently, MDCS is only used for TU sizes of 4x4 and 8x8.

Table 1 LUT of scanning patterns for intra luma block in HM 4.0

| Mode TU | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 16x16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8x8 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 4x4 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 2x2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

# Proposed methods

In this contribution, a modified MDCS is proposed for TU sizes of 16x16 and 32x32. In the proposed method, two sets of modified horizontal and vertical scanning patterns are used instead of the existing ones. The first set includes arbitrary horizontal and vertical scans, denoted as M0, which are derived based on the theoretical analysis presented in JCTVC-F124 [2]. The second set is the simplified version of M0, denoted as M1.

## Modified Scanning Patterns

The horizontal scanning patterns of M0 for 16x16 and 32x32 are presented in Appendix A. The vertical scanning patterns can be achieved by transposing the horizontal ones. From Appendix A, it can be seen that the scanning patterns of M0 are arbitrary, and do not present certain regular patterns, which would be more complex to both hardware and software. To further simplify the arbitrary scans, new scans are proposed as shown in Figure. 2 and Appendix B, denoted as M1.

Figure 2 Proposed M1 scan with horizontal direction.

As shown in Figure 2, the scanning pattern of M1 is controlled by a constant parameter N. For both 16x16 and 32x32, N is set as 9. Compared with M0, M1 is friendly to CABAC coding. That is, the scanning patterns of M1 can partially support parallel context formation between two consecutive significance map bits in scan order if the two bits are not in the same row or column.

## New Mapping Table

To include the proposed scanning patterns for 16x16 and 32x32, a new LUT of scanning patterns is proposed, as shown in Table 2. In this contribution, 16x16 and 32x32 scans share the same mapping to minimize the storage complexity.

Table 2 Proposed modified LUT of scanning patterns

| Mode TU | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16x16, 32x32 | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 | 0 | 0 | 0 |
| 4x4, 8x8 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |

# Simulations and results

The proposed extended MDCS is implemented on top of HM 4.0. Simulations are performed under all intra configurations defined in [3]. Both proposed M0 and M1 are tested, and the BD Rate reductions are reported in Table 3, and 4, respectively. From the experimental results, scan M0 achieves BD-Rate reductions of 0.2% and 0.6% for AI-HE and AI-LC configurations respectively. And scan M1 achieves BD-Rate reductions of 0.1% and 0.5% for AI-HE and AI-LC configurations respectively. The run time is not changed on average.

Table 3 Experimental results of M0

|  | Intra, HE |  |  | Intra, Loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.4% | -0.3% | -0.9% | -1.0% |
| Class B | -0.2% | -0.8% | -0.8% | -0.9% | -1.7% | -1.7% |
| Class C | -0.1% | -0.3% | -0.3% | -0.2% | -0.6% | -0.5% |
| Class D | -0.1% | -0.4% | -0.4% | -0.3% | -0.6% | -0.5% |
| Class E | -0.4% | -1.2% | -1.0% | -1.2% | -2.3% | -2.0% |
| All Avg | -0.2% | -0.6% | -0.6% | -0.6% | -1.2% | -1.1% |
| Enc Time (%) | 99% |  |  | 100% |  |  |
| Dec Time (%) | 97% |  |  | 97% |  |  |

Table 5 Experimental results of M1

|  | Intra, HE |  |  | Intra, Loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.3% | -0.2% | -0.7% | -0.8% |
| Class B | -0.2% | -0.7% | -0.7% | -0.8% | -1.6% | -1.6% |
| Class C | -0.1% | -0.3% | -0.3% | -0.2% | -0.6% | -0.5% |
| Class D | -0.1% | -0.4% | -0.3% | -0.2% | -0.5% | -0.4% |
| Class E | -0.4% | -0.9% | -0.8% | -0.8% | -1.8% | -1.6% |
| All Avg | -0.1% | -0.5% | -0.5% | -0.5% | -1.0% | -1.0% |
| Enc Time (%) | 100% |  |  | 100% |  |  |
| Dec Time (%) | 97% |  |  | 97% |  |  |

# Conclusion

Extended MDCS is proposed in this contribution, which includes two modifications:

- Modified 16×16 and 32×32 scanning patterns are proposed;

- New mapping table for 16×16 and 32×32 scans.

The proposed method is implanted on top of HM4.0. Experimental results show that by using M0, the BD-rate reductions for AI-HE and AI-LC are -0.2% and -0.6%, respectively. And by using M1, the BD-rate reductions for AI-HE and AI-LC are -0.1% and -0.5%, respectively. Moreover, no run time increase is observed at both encoder and decoder.

# References

[1] Y. Zheng, M. Coban, J. Sole, R. Joshi, M. Karczewicz, “CE11: Mode Dependent Coefficient Scanning”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D393, 4th Meeting: Daegu, KR, 20-28 January, 2011.

[2] X. Zhao, X. Guo, M. Guo, S. Lei, S. Ma, W. Gao, “Extended Mode-Dependent Coefficient Scanning”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F124, 6th Meeting: Torino, IT, 14-22 July, 2011.

[3] F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Patent rights declaration

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A

M0 scanning order for 16×16 coefficient block with vertical prediction,

M0 scanning order for 32×32 coefficient block with vertical prediction,

# Appendix B

M1 scanning order for 16×16 coefficient block with vertical prediction,

M1 scanning order for 32×32 coefficient block with vertical prediction,

# Appendix C

The proposed text changes to the working draft are provided as below,

## Scanning order array initialisation process

Input to this process is a block widthsize blkWidth and a block height blkHeight.

Output of this process is the array ScanOrder[ scanIdx ][ pos ][ comp ]. The array index scanIdx equal to 0 specifies a zig-zag scan as specified in subclause Error: Reference source not found with blkWidth and blkHeight as inputs, scanIdx equal to 1 specifies a horizontal scan, scanIdx equal to 2 specifies a vertical scan, and scanIdx equal to 3 specifies an up-right diagonal scan in subclause Error: Reference source not found. The array index pos specifies the scan position ranging from 0 to ( blkWidth * blkHeight ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

ScanOrder[0] = ZigZag

if ( blkWidth >8 ) N = 9

else N = blkWidth

for(i =0; i < blkWidth * blkHeight; i ++){

m = i/(blkWidth<<1);

n = i%(blkWidth<<1);

if(n<N){

offset[2] = {N, 0}

line = 0

ScanOrder[1][[i] = i;

}

else	{

line = (offset[1- line]==blkWidth) ? line : 1- line

ScanOrder[1][i] = m*(blkWidth<<1)+ line *blkWidth+offset[line]

offset[line]++

}

}

for(i=0; i < blkWidth * blkHeight; i++){

ScanOrder[2][i]= (ScanOrder[1][i] % blkWidth)* blkWidth +( ScanOrder[1][i] / blkWidth);

}

ScanOrder[3] = DiagScan

Table 7-1 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |
| --- | --- | --- |
|  | 0, 1 | 2, 3 |
| 0 | 1 | 1 |
| 1 | 2 | 2 |
| 2-3 | 0 | 0 |
| 4-5 | 1 | 0 |
| 6 | 0 | 0 |
| 7-8 | 2 | 0 |
| 9-10 | 0 | 0 |
| 11-12 | 1 | 0 |
| 13-14 | 0 | 0 |
| 15-16 | 2 | 0 |
| 17-19 | 0 | 0 |
| 20 | 1 | 0 |
| 21-22 | 1 | 1 |
| 23 | 1 | 0 |
| 24-27 | 0 | 0 |
| 28 | 2 | 0 |
| 29-30 | 2 | 2 |
| 31 | 2 | 0 |
| 32-33 | 0 | 0 |
| 34 | 0 | 0 |
| 35 | 0 | 0 |