| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G285 |
| --- | --- |

| Title: | Non-CE11: Methods for Solving the Parsing Issue of MDCS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *Xin Zhao, *Xun Guo, +Chih-Wei Hsu, +Tzu-Der Chuang, +Yu-Wen Huang and +Shawmin Lei, *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 +No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | {x.zhao, xun.guo, cw.hsu, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HEVC, mode dependent coefficient scanning (MDCS) is used for 4x4 and 8x8 transform units (TU) in intra coding, in which different scan patterns are selected according to intra prediction modes. For CABAC coding, the context formation of MDCS highly depends on the scan patterns, which needs to acquire the intra prediction modes at the parsing stage. This may reduce the parsing throughput of the decoder. In this contribution, two methods are proposed to solve this parsing issue: a modified context index derivation method and a pre-scan method. It is reported that, by using the first method, the average BD-Rate reduction is 0.22% for AI-HE. By using the second method, the average BD-Rate reduction is 0.21% for AI-HE.

# Introduction

In HEVC high efficiency (HE) configuration, MDCS [1] is used for 4x4 and 8x8 TUs in intra coding, which includes three scan patterns as illustrated in Figure 1: Diagonal, Horizontal and Vertical scans. For each TU, one scan pattern is chosen according to the intra prediction mode. The residual coding of MDCS in CABAC highly depends on the intra prediction modes, since the scan patterns are needed to be known in the coding procedure for the mapping between absolute coefficient positions and scan positions. That is to say, the intra prediction modes need to be derived at the parsing stage, which would reduce the parsing throughput at decoder.

To solve the above parsing issue, this contribution proposes two methods. For the first one, the scan positions are used to derive the context index instead of absolute coefficient positions in HM4.0. Moreover, the last significant coefficient is coded always using horizontal scan. For the second one, a pre-scan method is proposed to remap the transform coefficients from the true scans into horizontal scan. After that, horizontal scan is used for the subsequent coefficients coding. The two methods will be described in details in Section 2 and Section 3. The results are shown in Section 4.

Figure 1. Three scanning patterns for CABAC:

Diagonal (left), Horizontal (middle) and vertical (right)

# Proposed method 1

In this method, the significance map coding and last significant coefficient coding are modified as follows.

## Significance map coding

In HM4.0, the context index of significance map for 4x4 and 8x8 blocks is determined according to the absolute position within the blocks. We propose to always use the horizontal scan to derive the position for the context index derivation so that no scanning pattern is required in the parsing process. After that, the context index, ctxIdx, is derived as follows.

For 4×4, ctxIdx = 4×Y + X

For 8×8, ctxIdx = 15+4×(Y>>1) + (X>>1),

where

Y = scanPos/N,

X = scanPos – (Y×N).

Here scanPos means the position in the scan order, which is equal to the absolute position in a horizontal scan order. It’s just an implementation issue.

## Last significant coefficient coding

In HM4.0, the coordinates (X and Y) of last significant coefficient depend on the scanning pattern. We propose to always use horizontal scan to derive X and Y so that the last significant coefficient coding can be independent of the intra prediction modes.

# Proposed Method 2

In this method, a pre-scan procedure is proposed to remove the dependency between coefficients coding and intra prediction modes. On the encoder side, the scanning order for the intra coded coefficients is still selected based on the intra prediction mode as MDCS does. However, instead of coding the scanned coefficients into bitstream, those scanned coefficients are remapped into horizontal scan. Then horizontal scan is always used to code the coefficients into bitstream. Figure 2 illustrates the pre-scan process from diagonal scan to horizontal scan for the 8x8 intra coded block.

Figure 2. Illustration of pre-scan procedure for intra coded coefficients

On the decoder side, bitstream for the transform coefficients of the intra coded block is parsed using horizontal scan without knowing the real intra prediction mode. After the real intra prediction mode is reconstructed and is available, the parsed coefficients can be remapped from horizontal scan back to the true scan. The above-mentioned procedure is called post-scan and is illustrated in Figure 4. In this way, there is no need to perform reconstruction procedure to acquire the intra prediction mode at the parsing stage.

Figure 3. Illustration of post-scan procedure for intra coded coefficients

# Simulations and results

The proposed methods are implemented on top of HM 4.0. Simulations are performed under AI-HE, configuration defined in [2]. The BD-rate reductions are reported in Table 2 and Table 3.

Table 2 Experimental results of proposed method 1

|  | Intra, HE |  |  | Random Access, HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.30% | 0.12% | 0.19% | 0.12% | 0.13% | 0.14% |
| Class B | 0.20% | 0.00% | 0.07% | 0.11% | 0.02% | -0.07% |
| Class C | 0.19% | 0.19% | 0.20% | 0.15% | 0.36% | 0.36% |
| Class D | 0.23% | 0.36% | 0.43% | 0.16% | 0.45% | 0.22% |
| Class E | 0.19% | -0.08% | -0.21% |  |  |  |
| All Avg | 0.22% | 0.12% | 0.15% | 0.13% | 0.23% | 0.15% |
| Enc Time (%) | 99% |  |  | 100% |  |  |
| Dec Time (%) | 97% |  |  | 100% |  |  |
|  | Low Delay B, HE |  |  | Low Delay P, HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.05% | -0.17% | -0.12% | 0.08% | 0.12% | -0.14% |
| Class C | 0.11% | 0.10% | 0.13% | 0.07% | 0.14% | 0.25% |
| Class D | 0.09% | 0.18% | 0.16% | 0.05% | 0.26% | -0.09% |
| Class E | 0.11% | 0.12% | 0.53% | 0.03% | -0.09% | 0.03% |
| All Avg | 0.08% | 0.04% | 0.13% | 0.06% | 0.12% | 0.00% |
| Enc Time (%) | 100% |  |  | 100% |  |  |
| Dec Time (%) | 98% |  |  | 98% |  |  |

Table 3 Experimental results of proposed method 2

|  | Intra, HE |  |  | Random Access, HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.31% | 0.16% | 0.20% | 0.15% | 0.27% | 0.01% |
| Class B | 0.23% | 0.04% | 0.01% | 0.12% | 0.07% | 0.03% |
| Class C | 0.19% | 0.14% | 0.08% | 0.13% | 0.21% | 0.26% |
| Class D | 0.14% | 0.15% | 0.21% | 0.11% | 0.12% | -0.07% |
| Class E | 0.15% | 0.18% | 0.24% |  |  |  |
| All Avg | 0.21% | 0.13% | 0.14% | 0.13% | 0.16% | 0.05% |
| Enc Time (%) | 100% |  |  | 100% |  |  |
| Dec Time (%) | 97% |  |  | 100% |  |  |
|  | Low Delay B, HE |  |  | Low Delay P, HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.05% | 0.04% | -0.45% | 0.06% | 0.11% | -0.04% |
| Class C | 0.10% | -0.11% | 0.02% | 0.08% | 0.20% | 0.06% |
| Class D | -0.01% | 0.63% | -0.45% | 0.13% | 0.27% | -0.36% |
| Class E | 0.03% | -0.49% | 0.74% | 0.08% | -0.03% | 0.16% |
| All Avg | 0.05% | 0.05% | -0.11% | 0.09% | 0.15% | -0.06% |
| Enc Time (%) | 100% |  |  | 100% |  |  |
| Dec Time (%) | 99% |  |  | 98% |  |  |

# Conclusion

In this contribution, two methods are proposed to solve the parsing issue of MDCS: a modified context index derivation method and a pre-scan method. It is reported that, by using the first method, the average BD-Rate reduction is 0.22% for AI-HE. By using the second method, the average BD-Rate reduction is 0.21 for AI-HE.

# References

[1] Y. Zheng, M. Coban, J. Sole, R. Joshi, M. Karczewicz, “CE11: Mode Dependent Coefficient Scanning”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D393, 4th Meeting: Daegu, KR, 20-28 January, 2011.

[2] F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Patent rights declaration

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

### The proposed text changes to the working draft are provided as below,

### 7. 3.10 Residual coding CABAC syntax

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| numCoeff = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 1 ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 1 ][ numCoeff ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| numCoeff++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 1 ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 1 ][ numCoeff ][ 1 ] |  |
| } |  |
| for( n = numCoeff − 1; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| …… | …… |
| } |  |

###### 9.3.3.1.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position scanPos, and scan block position ( xC , yC ) and the transform block size log2TrafoSize.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than or equal to 3, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
yS = scanPos >> log2TrafoSize
xS = scanPos – (yS<< log2TrafoSize)
sigCtx  =  ( shift  *  15 )  +  ( ( yS  >>  shift )  <<  2 )  +  ( xS  >>  shift )	(9-1)

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC	(9-2)

……