| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G286 |
| --- | --- |

| Title: | CE12 Subtest 1: Chroma Deblocking Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | #Qian Huang, #Jicheng An, #Xun Guo, #Shawmin Lei, *Andrey Norkin, *Kenneth Andersson and*Rickard Sjöberg #No. 2, Kexueyuan South Rd., Raycom Infotech Park, Beijing, China 100190 *Färögatan 6, 164 80 Stockholm, Sweden | Email: | {qian.huang, jicheng.an, xun.guo, shawmin.lei}@mediatek.com {andrey.norkin, kenneth.r.andersson, rickard.sjoberg}@ericsson.com |
| Source: | #MediaTek Inc., *Ericsson |  |  |

_____________________________

# Abstract

This contribution presents experimental results for the chroma deblocking filter in CE12 Subtest 2. In specific, chroma deblocking filter with independent filtering decision and 8x8 filtering unit is tested and proposed. It is reported that, average BD-Rate reduction of 0.6% can be achieved for chroma. The run time is reported to be similar to HM4.0. It is also reported that the subjective quality is almost the same as that of HM4.0.

# Introduction

In HM4.0 deblocking filter, filtering decision is only applied on luma boundaries. In order to further improve both objective performance and subjective quality, we have proposed a chroma filtering decision strategy in our previous proposal JCTVC-F118 [1], which is based on a filtering unit according to sampling format. For 4:2:0 format, the chroma filtering is performed on a 4x4 grid. To reduce complexity, 8x8 filtering unit is used in this contribution.

The rest of this contribution is organized as follows. Section 2 describes the proposed method. Section 3 shows and analyzes the experimental results. And section 4 draws our conclusions.

# Proposed Method

Compared with HEVC deblocking filter, the proposed chroma filtering has two major differences: boundary strength (Bs) and filtering decision.

## Chroma Boundary Strength

In HEVC, chroma boundary strength (Bs) values are not calculated for chroma. Only intra boundaries are filtered. To fully exploit the chroma characteristics, we propose to also filter inter boundaries. For that purpose, the Bs for chroma is computed. After that, the boundaries with Bs > 2 will be filtered. For a given block boundary between blocks A and B, the full definitions for chroma Bs are as follows:

- If this boundary is also a coding unit boundary and either A or B is coded with intra prediction mode, BS is set to 4.

- Otherwise, if either A or B is coded with intra prediction mode, BS is set to 3.

- Otherwise, if this boundary is also a transform unit boundary and either A or B contains non-zero transform coefficients, BS is set to 2.

- Otherwise, BS is set to 0.

  - Chroma Filtering Decision

Chroma components can be filtered at the block boundaries with chroma boundary strength BS > 0. For the 4:2:0 sampling format, separate filtering decisions are done on each sub-boundary with length 4. In each sub-boundary with length 4, the 2nd and 3rd lines are used in the filtering decision as follows.

d2 = |p01 – p11| + |q01 – q11| + |p02 – p12| + |q02 – q12|,                 (1)

where the chroma samples are depicted in Figure 1.

|  | p30 | p20 | p10 | p00 | q00 | q10 | q20 | q30 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p31 | p21 | p11 | p01 | q01 | q11 | q21 | q31 |  |
|  | p32 | p22 | p12 | p02 | q02 | q12 | q22 | q32 |  |
|  | p33 | p23 | p13 | p03 | q03 | q13 | q23 | q33 |  |

Figure 1 A sub-boundary 4 pixel lines

The chroma samples will be filtered if the below expression is true:

(d2 < Beta_Chroma) && (On the 8x8 chroma grid).                  (2)

Note that Beta_Chroma is indexed from the Beta table using chroma QP, and the filtering decisions are made independently for U and V components. The filtering operations for chroma remain the same as HEVC. Only one chroma sample at each side of a chroma boundary might be changed.

# Experimental Results

Our simulations are based on HM4.0 software and the common test conditions in F900 [2]. Table 1 shows the experimental results compared with HM4.0. According to the table, average BD-Rate reduction of 0.6% for chroma components can be observed.

Table 1. Performance of compared to HM 4.0

|  | Intra |  |  | Intra_LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | -0.4% | -0.3% | 0.0% | -0.3% | -0.2% |
| Class B | 0.0% | -0.5% | -0.4% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.7% | -0.7% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | -0.5% | -0.6% | 0.0% | -0.1% | 0.0% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |  |  |  |
| All | 0.0% | -0.4% | -0.4% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.4% | -0.4% | 0.0% | -0.1% | -0.1% |
| EncTime[%] | 100.4% |  |  | 100.0% |  |  |
| DecTime[%] | 99.8% |  |  | 99.2% |  |  |
|  | RandomAccess |  |  | RandomAccess_LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | -0.7% | -0.6% | 0.0% | -0.6% | -0.4% |
| Class B | 0.0% | -0.6% | -0.5% | 0.0% | -0.6% | -0.5% |
| Class C | 0.0% | -0.8% | -0.7% | 0.0% | -0.9% | -1.0% |
| Class D | 0.0% | -0.6% | -0.7% | 0.0% | -1.0% | -1.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| All | 0.0% | -0.7% | -0.6% | 0.0% | -0.8% | -0.7% |
|  | 0.0% | -0.7% | -0.6% | 0.0% | -0.7% | -0.7% |
| EncTime[%] | 99.8% |  |  | 99.9% |  |  |
| DecTime[%] | 100.0% |  |  | 99.0% |  |  |
|  | LowDelay |  |  | LowDelay_LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -1.1% | -1.2% | 0.0% | -0.4% | -0.2% |
| Class C | 0.0% | -1.0% | -1.0% | 0.0% | -0.4% | -0.8% |
| Class D | 0.0% | -0.7% | -0.8% | 0.0% | -0.2% | -0.4% |
| Class E | 0.0% | -2.6% | -1.5% | -0.1% | -1.3% | -1.4% |
| Class F |  |  |  |  |  |  |
| All | 0.0% | -1.3% | -1.1% | 0.0% | -0.5% | -0.6% |
|  | 0.0% | -1.3% | -1.2% | 0.0% | -0.5% | -0.6% |
| EncTime[%] | 100.0% |  |  | 100.0% |  |  |
| DecTime[%] | 100.1% |  |  | 98.9% |  |  |

# Conclusion

The contribution presents the experimental results of chroma deblocking filter in CE12 Subtest 1 [1]. Specifically, chroma deblocking filter with independent filtering decision and 8x8 filtering unit is tested. For common test conditions, the average BD-Rate reduction is 0.6% for chroma. The decoding time is almost the same to HM4.0. In addition, the subjective quality is very similar as that of HM4.0.

# References

- A. Norkin, K. Andersson, R. Sjöberg, Q. Huang, J. An, X. Guo and S. Lei, “CE12: Ericsson's and MediaTek's deblocking filter,” JCTVC-F118, JCT-VC meeting, Torino, IT, Jul. 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, JCT-VC meeting, Torino, IT, Jul. 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).