| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G287 |
| --- | --- |

| Title: | CE12: Simplified Chroma Deblocking Filter in JCTVC-G286 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | #Qian Huang, #Jicheng An, #Xun Guo, #Shawmin Lei, *Andrey Norkin, *Kenneth Andersson and*Rickard Sjöberg #No. 2, Kexueyuan South Rd., Raycom Infotech Park, Beijing, China 100190 *Färögatan 6, 164 80 Stockholm, Sweden | Tel: Email: | +86-10-82800818 {qian.huang, jicheng.an, xun.guo, shawmin.lei}@mediatek.com {andrey.norkin, kenneth.r.andersson, rickard.sjoberg}@ericsson.com |
| Source: | #MediaTek Inc., *Ericsson |  |  |

_____________________________

# Abstract

This contribution presents the results of chroma deblocking filter in CE 12 Subtest 1. In specific, chroma deblocking filter with dependent filtering decision on luma and 8x8 filtering unit is tested and proposed. It is reported that an average BD-Rate reduction of 1.0% can be achieved for chroma. For low delay cases, an average BD-Rate reduction of 0.1% can also be achieved for luma. The run time is reported to be similar to HM4.0. It is also reported that the subjective quality is almost the same as that of HM4.0.

# Introduction

In HM4.0 deblocking filter, filtering decision is only applied on luma boundaries. In order to further improve both objective performance and subjective quality, we have proposed a chroma filtering decision method in our previous proposals JCTVC-F118 [1], then a simplified chroma filtering decision method in JCTVC-F120 [2]. This contribution is based on JCTVC-F120. However, in order to reduce complexity, 8x8 filtering unit is used in this contribution instead of previous 4x4.

The rest of this contribution is organized as follows. Section 2 describes the proposed algorithm. Section 3 shows and analyzes the experimental results. And section 4 draws our conclusion.

# Algorithm Description

The proposed chroma filter actually is a simplification of that in JCTVC-G286. The major difference is that chroma boundary would re-use the filtering decision of corresponding luma boundary. Figure 1 gives the flowchart of the proposed method. Beta_Luma and Beta_Chroma are the threshold values derived from the beta table for luma and chroma, respectively. The luma pixel difference d is defined as follows:

d = |p22 - 2*p12 + p02| + |q22 - 2*q12 + q02| + |p25 - 2*p15 + p05| + |q25 - 2*q15 + q05|.         (1)

The chroma samples will be filtered if the below expression is true:

(d < Beta_Chroma) && (On the 8x8 chroma grid).                  (2)

Note that Beta_Luma is exactly the same as in HM 4.0, and Beta_Chroma is defined according to the slice type:

- For I slices, Beta_Chroma is defined as twice of the QP value for luma.

- For B/P slices, Beta_Chroma is the same as Beta_Luma.

Figure 1. Flowchart of the proposed algorithm

# Experimental Results

Our simulations are based on HM4.0 software and the common test conditions in F900 [3]. An average BD-Rate reduction of 1.0% is observed for chroma, and an average BD-Rate reduction of 0.1% is observed for luma in low delays cases. Moreover, the run time of the proposed method is almost the same to that of HM4.0.

Table 1. Performance of compared to HM 4.0

|  | Intra |  |  | Intra_LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | -0.7% | -0.6% | 0.0% | -0.7% | -0.6% |
| Class B | 0.0% | -0.7% | -0.6% | 0.0% | -0.6% | -0.4% |
| Class C | 0.0% | -0.8% | -0.6% | 0.0% | -0.3% | -0.1% |
| Class D | 0.0% | -0.6% | -0.5% | 0.0% | -0.1% | 0.1% |
| Class E | 0.0% | -0.1% | -0.1% | 0.0% | 0.3% | 0.2% |
| Class F |  |  |  |  |  |  |
| All | 0.0% | -0.6% | -0.5% | 0.0% | -0.3% | -0.2% |
|  | 0.0% | -0.6% | -0.5% | 0.0% | -0.3% | -0.2% |
| EncTime[%] | 99.5% |  |  | 99.9% |  |  |
| DecTime[%] | 99.8% |  |  | 99.0% |  |  |
|  | RandomAccess |  |  | RandomAccess_LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | -1.8% | -1.7% | 0.0% | -1.9% | -1.6% |
| Class B | 0.0% | -1.6% | -1.4% | 0.0% | -1.6% | -1.3% |
| Class C | 0.0% | -1.3% | -1.2% | 0.0% | -1.6% | -1.5% |
| Class D | 0.0% | -1.2% | -1.2% | 0.0% | -1.6% | -1.3% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| All | 0.0% | -1.5% | -1.4% | 0.0% | -1.7% | -1.4% |
|  | 0.0% | -1.5% | -1.3% | 0.0% | -1.7% | -1.4% |
| EncTime[%] | 99.7% |  |  | 100.0% |  |  |
| DecTime[%] | 100.2% |  |  | 99.0% |  |  |
|  | LowDelay |  |  | LowDelay_LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -1.7% | -1.9% | 0.0% | -1.1% | -0.8% |
| Class C | 0.0% | -1.3% | -1.1% | -0.1% | -0.6% | -0.8% |
| Class D | -0.1% | -0.9% | -0.8% | 0.0% | -0.4% | -0.2% |
| Class E | 0.0% | -2.3% | -2.4% | -0.1% | -1.6% | -2.1% |
| Class F |  |  |  |  |  |  |
| All | -0.1% | -1.5% | -1.5% | -0.1% | -0.9% | -0.9% |
|  | -0.1% | -1.5% | -1.5% | -0.1% | -0.9% | -0.9% |
| EncTime[%] | 99.8% |  |  | 100.0% |  |  |
| DecTime[%] | 99.5% |  |  | 99.2% |  |  |

# Conclusion

The contribution proposes a simplified solution based on an earlier contribution [2]. Specifically, the filtering will be performed only on 8x8 chroma grid. For common test conditions, the average BD-Rate reduction is 1.0% for chroma and the average decoder running time is slightly reduced compared with that of HM 4.0. In addition, the subjective quality is almost the same as HM4.0.

# References

- A. Norkin, K. Andersson, R. Sjöberg, Q. Huang, J. An, X. Guo and S. Lei, “CE12: Ericsson's and MediaTek's deblocking filter,” JCTVC-F118, JCT-VC meeting, Torino, IT, Jul. 2011.

- Q. Huang, J. An, X. Guo and S. Lei, “Simplification of chroma deblocking filter in JCTVC-F118,” JCTVC-F120, JCT-VC meeting, Torino, IT, Jul. 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, JCT-VC meeting, Torino, IT, Jul. 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).