| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G288 |
| --- | --- |

| Title: | Non-CE13: Complexity Reduction of MVP List Construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong Yu Tim Hellman Broadcom Corporation 200 Brickstone Square Suite 401 Andover, MA 01810 | Tel: Email: Tel: Email: | +1-978-719-1345 yongyu@broadcom.com +1-978-719-1348 timhellman@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution reports experimental results on the compression performance when reducing the number of combined and non-scaled motion vector predictions (MVPs) used for list construction.  The results show a bit rate increase up to 0.1% in the luminance component when the maximum number of combined MVPs is reduced from 5 to 2, and a bit rate increase up to 0.3% when the maximum number of combined MVPs is reduced from 5 to 1.  The results also show very little difference when increasing the scaled MVPs from 1 to 2 and that invoking the scaling process before the combination process produces slightly better results than invoking it afterwards.

# Introduction

The effect of motion vector prediction list construction on CABAC throughput has been discussed in previous meetings [2, 3, 4, 5]. In these proposals, potential alternative motion vector predictors are added to the MVP list when there are not enough valid MVPs due to the elimination of replicated MVPs, intra neighbor CU’s, or slice boundaries.  Previous contributions have suggested generating these alternative MVPs using a combination of existing valid MVPs followed by a simplified scaling process. A secondary pruning process is invoked after these potential MVPs are generated.

The combination process is invoked up to five times before the additional simplified scaling process which is invoked only once. Therefore, the maximum number of total alternative MVPs can be up to six.

This contribution suggests that the second pruning stage adds computational and testing complexity, especially for hardware implementations. The cost of this secondary pruning is significant, especially when the number of total potential MVPs is large.  This is because the vector construction must be completed in very few clock cycles to avoid becoming the worst-case bottleneck in the system.  Checking a large list of candidates for duplicates and pruning them out is an expensive hardware process, therefore it makes sense to limit the number of possible candidates, especially if doing so has no noticeable effect on bitrate.

In this contribution, results on the following experiments are reported:

- Reducing the maximum number of candidates from the combination process;

- Reversing the order of the two MVP generation processes.  Previous contributions have suggested adding potential MVPs by combining existing valid motion vectors first followed by scaling these predictors.  This experiment examines performing the simplified scaling process first followed by the combination process.

# Experimental Results

## Reducing the maximum number of candidates from the combination processes

Table 1 summarizes the results when the number of combination MVPs are reduced from 5 to 1 or 2.  Results are also shown when the number of scaled MVPs is 1 or 2 to understand the potential effect of increasing this from 1 to 2.  See Appendix for additional details on the simulation results.  I-frame only test categories are not affected by the proposed change and hence are not reported in this contribution.

The results show:

- A luminance bit rate increase of up to 0.1% occurs when the combined candidates are reduced from 5 to 2 and the scaled candidates remains fixed at 1.  (Experiment 1C in the table.)

- A luminance bit rate increase of up to 0.3% occurs when the combined candidates is reduced from 5 to 1 and the scaled candidates remains fixed at 1 as highlighted in the table. (Experiment 1A in the table.)

- Very little difference when increasing the scaled MVPs from 1 to 2.  (Experiment 1A vs. Experiment 1B and Experiment 1C vs. Experiment 1D in the table.)

Table 1. Experimental results when the numbers of potential MVPs are reduced (Reference is 5 Combined Candidates and 1 Scaled Candidate)

| Experiment | 1a |  |  | 2a |  |  | 3a |  |  | 4a |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of Combined Candidates | 1 |  |  | 1 |  |  | 2 |  |  | 2 |  |  |
| # of Scaled Candidates | 1 |  |  | 2 |  |  | 1 |  |  | 2 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| RA-HE | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RA-LC | 0.1% | 0.1% | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| LB-HE | 0.2% | 0.2% | 0.3% | 0.2% | 0.4% | 0.7% | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.3% |
| LB-LC | 0.3% | 0.2% | 0.1% | 0.3% | 0.2% | -0.2% | 0.1% | 0.2% | 0.0% | 0.1% | 0.2% | 0.0% |

## Reversing Order of the Two MVP Candidate Generation Process

Table 2 summarizes the results when the simplified scaling process is invoked first followed by the combination process are compared to the results in Table 1 where the combination process is invoked first.  See Appendix for additional details on the simulation results.  I-frame only test categories are not affected by the proposed change and hence are not reported in this contribution.

The results show that invoking the scaling process before the combination process produces up to 0.3% bit rate gains than invoking it afterwards.

Table 2. Experimental results when the numbers of potential MVPs are reduced and the scaling process is performed before the combination process (Reference is scaling performed after instead of before)

| Experiment | 1b |  |  | 2b |  |  | 3b |  |  | 4b |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of Combined Candidates | 1 |  |  | 1 |  |  | 2 |  |  | 2 |  |  |
| # of Scaled Candidates | 1 |  |  | 2 |  |  | 1 |  |  | 2 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| RA-HE | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| RA-LC | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| LB-HE | 0.2% | 0.2% | 0.3% | 0.2% | 0.2% | 0.3% | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.2% |
| LB-LC | 0.3% | 0.2% | 0.1% | 0.3% | 0.2% | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% | 0.2% | 0.0% |

# Conclusions

This contribution shows that there is less than 0.1% bit rate increase when the maximum combined MVPs is reduced from 5 to 2. This simplification results in a significant amount of cost reduction in implementation of the MVP list construction process in ASIC hardware.  Therefore, this contribution recommends reducing the maximum number of combined MVPs from 5 to 2 to achieve a good trade-off between coding efficiency and implementation cost.

This contribution also suggests that reversing the order of the two MVP generation process produces slightly better results.  However, the difference is relatively insignificant.  Therefore this contribution recommends that no change be made.

# References

- J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei,  “CE9: Results of Experiment ROB04”, Document of Joint Collaborative Team on Video Coding, Torino, IT, JCTVC-F052, July 2011

- M. Zhou, V. Sze, Y. Matsuba,” A study on HEVC parsing throughput issue ”, Document of Joint Collaborative Team on Video Coding, Torino, IT, JCTVC-F068, July 2011

- J. Chen, T. Lee, I.-K. Kim, “MVP index parsing with fixed number of candidates”, Document of Joint Collaborative Team on Video Coding, Torino, IT, JCTVC-F402, July 2011

- G. Laroche, P. Onno, T. Poirier, C. Gisquet , “CE9: Description of experiments ROB01 and ROB02”, Document of Joint Collaborative Team on Video Coding, Torino, IT, JCTVC-F474, July 2011

# Appendix

Test 1a experimental results:

|  | RA-HE |  |  |  | RA-LC |  |  |  | LB-HE |  |  | LB-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y |  | U | V | Y | U | V | Y |  |  | U | V |
| Class A | 0.1% | -0.1% | -0.1% |  | 0.1% |  | 0.2% | 0.0% |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% |  | 0.1% |  | 0.0% | 0.1% | 0.2% | 0.2% | 0.1% | 0.4% |  |  | 0.3% | 0.2% |
| Class C | 0.1% | 0.0% | 0.1% |  | 0.1% |  | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.3% |  |  | 0.2% | 0.1% |
| Class D | 0.1% | 0.2% | 0.1% |  | 0.1% |  | 0.0% | 0.0% | 0.3% | 0.0% | 0.3% | 0.3% |  |  | 0.1% | 0.2% |
| Class E |  |  |  |  |  |  |  |  | 0.2% | 0.4% | 0.7% | 0.3% |  |  | 0.2% | -0.2% |
| All | 0.1% | 0.0% | 0.0% |  | 0.1% |  | 0.1% | 0.1% | 0.2% | 0.2% | 0.3% | 0.3% |  |  | 0.2% | 0.1% |
| Enc Time | 93% |  |  | 271% |  |  |  |  | 100% |  |  |  | 101% |  |  |  |
| Dec Time | 99% |  |  | 99% |  |  |  |  | 101% |  |  |  | 100% |  |  |  |

Test 2a experimental results

|  | RA-HE |  |  |  | RA-LC |  |  |  | LB-HE |  |  | LB-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y |  | U | V | Y | U | V | Y |  |  | U | V |
| Class A | 0.0% | -0.1% | -0.1% |  | 0.1% |  | 0.1% | 0.1% |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% |  | 0.1% |  | 0.0% | 0.1% | 0.2% | 0.2% | 0.1% | 0.4% |  |  | 0.3% | 0.2% |
| Class C | 0.0% | 0.0% | 0.1% |  | 0.0% |  | 0.0% | 0.0% | 0.3% | 0.1% | 0.1% | 0.3% |  |  | 0.2% | 0.1% |
| Class D | 0.1% | 0.0% | 0.0% |  | 0.0% |  | -0.1% | 0.1% | 0.3% | 0.0% | 0.3% | 0.3% |  |  | 0.1% | 0.2% |
| Class E |  |  |  |  |  |  |  |  | 0.2% | 0.4% | 0.7% | 0.3% |  |  | 0.2% | -0.2% |
| All | 0.1% | 0.0% | 0.0% |  | 0.1% |  | 0.0% | 0.1% | 0.2% | 0.2% | 0.3% | 0.3% |  |  | 0.2% | 0.1% |
| Enc Time | 92% |  |  | 101% |  |  |  |  | 100% |  |  |  | 104% |  |  |  |
| Dec Time | 77% |  |  | 99% |  |  |  |  | 94% |  |  |  | 97% |  |  |  |

Test 3a experimental results:

|  | RA-HE |  |  |  | RA-LC |  |  |  | LB-HE |  |  | LB-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y |  | U | V | Y | U | V | Y |  |  | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.1% | 0.1% |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |  |  | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |  |  | 0.1% | 0.0% |
| Class D | 0.1% | 0.2% | 0.1% |  | 0.0% |  | 0.0% | 0.0% | 0.1% | 0.3% | 0.5% | 0.1% |  |  | 0.3% | 0.2% |
| Class E |  |  |  |  |  |  |  |  | 0.1% | 0.4% | 0.9% | 0.0% |  |  | 0.1% | -0.5% |
| All | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% | 0.0% | 0.1% | 0.2% | 0.3% | 0.1% |  |  | 0.2% | 0.0% |
| Enc Time | 104% |  |  | 102% |  |  |  |  | 102% |  |  |  | 100% |  |  |  |
| Dec Time | 106% |  |  | 103% |  |  |  |  | 103% |  |  |  | 102% |  |  |  |

Test 4a experimental results

|  | RA-HE |  |  |  | RA-LC |  |  |  | LB-HE |  |  | LB-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y |  | U | V | Y | U | V | Y |  |  | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.1% | 0.1% |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |  |  | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |  |  | 0.1% | 0.0% |
| Class D | 0.1% | 0.2% | 0.1% |  | 0.0% |  | 0.0% | 0.0% | 0.1% | 0.3% | 0.5% | 0.1% |  |  | 0.3% | 0.2% |
| Class E |  |  |  |  |  |  |  |  | 0.1% | 0.4% | 0.9% | 0.0% |  |  | 0.1% | -0.5% |
| All | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% | 0.0% | 0.1% | 0.2% | 0.3% | 0.1% |  |  | 0.2% | 0.0% |
| Enc Time | 92% |  |  | 281% |  |  |  |  | 100% |  |  |  | 101% |  |  |  |
| Dec Time | 97% |  |  | 97% |  |  |  |  | 98% |  |  |  | 96% |  |  |  |

Test 1b experimental results:

|  | RA-HE |  |  |  | RA-LC |  |  |  | LB-HE |  |  | LB-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y |  | U | V | Y | U | V | Y |  |  | U | V |
| Class A | 0.1% | -0.1% | -0.1% |  | 0.1% |  | 0.2% | 0.0% | 0.2% | 0.2% | 0.1% | 0.4% |  |  | 0.3% | 0.2% |
| Class B | 0.1% | 0.1% | 0.1% |  | 0.1% |  | 0.0% | 0.1% | 0.3% | 0.1% | 0.1% | 0.3% |  |  | 0.2% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% |  | 0.1% |  | 0.1% | 0.1% | 0.3% | 0.0% | 0.3% | 0.3% |  |  | 0.1% | 0.2% |
| Class D | 0.1% | 0.2% | 0.1% |  | 0.1% |  | 0.0% | 0.0% | 0.2% | 0.4% | 0.7% | 0.3% |  |  | 0.2% | -0.2% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| All | 0.1% | 0.0% | 0.0% |  | 0.1% |  | 0.1% | 0.1% | 0.2% | 0.2% | 0.3% | 0.3% |  |  | 0.2% | 0.1% |
| Enc Time | 97% |  |  | 105% |  |  |  |  | 100% |  |  |  | 101% |  |  |  |
| Dec Time | 98% |  |  | 105% |  |  |  |  | 99% |  |  |  | 101% |  |  |  |

Test 2b experimental results:

|  | RA-HE |  |  |  | RA-LC |  |  |  | LB-HE |  |  | LB-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y |  | U | V | Y | U | V | Y |  |  | U | V |
| Class A | 0.1% | 0.0% | 0.0% |  | 0.1% |  | 0.1% | 0.0% |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% |  | 0.1% |  | 0.0% | 0.1% | 0.2% | 0.2% | 0.1% | 0.4% |  |  | 0.3% | 0.2% |
| Class C | 0.1% | 0.0% | 0.2% |  | 0.1% |  | 0.0% | 0.2% | 0.3% | 0.1% | 0.1% | 0.3% |  |  | 0.2% | 0.1% |
| Class D | 0.1% | 0.0% | 0.1% |  | 0.1% |  | 0.1% | 0.1% | 0.3% | 0.0% | 0.3% | 0.3% |  |  | 0.1% | 0.2% |
| Class E |  |  |  |  |  |  |  |  | 0.2% | 0.4% | 0.7% | 0.3% |  |  | 0.2% | -0.2% |
| All | 0.1% | 0.0% | 0.0% |  | 0.1% |  | 0.1% | 0.1% | 0.2% | 0.2% | 0.3% | 0.3% |  |  | 0.2% | 0.1% |
| Enc Time | 98% |  |  | 110% |  |  |  |  | 100% |  |  |  | 100% |  |  |  |
| Dec Time | 99% |  |  | 111% |  |  |  |  | 101% |  |  |  | 100% |  |  |  |

Test 3b experimental results:

|  | RA-HE |  |  |  | RA-LC |  |  |  | LB-HE |  |  | LB-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y |  | U | V | Y | U | V | Y |  |  | U | V |
| Class A | 0.0% | 0.0% | -0.1% |  | 0.1% |  | 0.2% | 0.1% |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% |  | 0.0% |  | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |  |  | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% |  | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |  |  | 0.1% | 0.0% |
| Class D | 0.0% | 0.2% | 0.1% |  | 0.0% |  | -0.1% | 0.0% | 0.1% | 0.3% | 0.5% | 0.1% |  |  | 0.3% | 0.2% |
| Class E |  |  |  |  |  |  |  |  | 0.1% | 0.4% | 0.9% | 0.0% |  |  | 0.1% | -0.5% |
| All | 0.0% | 0.1% | 0.1% |  | 0.0% |  | 0.0% | 0.0% | 0.1% | 0.2% | 0.3% | 0.1% |  |  | 0.2% | 0.0% |
| Enc Time | 102% |  |  | 102% |  |  |  |  | 100% |  |  |  | 102% |  |  |  |
| Dec Time | 104% |  |  | 102% |  |  |  |  | 101% |  |  |  | 104% |  |  |  |

Test 4b experimental results

|  | RA-HE |  |  |  | RA-LC |  |  |  | LB-HE |  |  | LB-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y |  | U | V | Y | U | V | Y |  |  | U | V |
| Class A | 0.1% | -0.1% | 0.0% |  | 0.0% |  | 0.2% | -0.1% |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% |  | 0.1% |  | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |  |  | 0.1% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% |  | 0.1% |  | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |  |  | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% | -0.1% | 0.1% | 0.3% | 0.4% | 0.1% |  |  | 0.3% | 0.2% |
| Class E |  |  |  |  |  |  |  |  | 0.1% | 0.3% | 0.8% | 0.0% |  |  | 0.1% | -0.5% |
| All | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.1% | 0.0% | 0.1% | 0.2% | 0.2% | 0.1% |  |  | 0.2% | 0.0% |
| Enc Time | 95% |  |  | 277% |  |  |  |  | 98% |  |  |  | 97% |  |  |  |
| Dec Time | 101% |  |  | 95% |  |  |  |  | 100% |  |  |  | 100% |  |  |  |

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).