| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G289 M21851 |
| --- | --- |

| Title: | Cross-verification of Sharp’s proposal JCTVC-G307 on Bi-prediction restriction in small PUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for Sharp’s proposal JCTVC-G307 on Bi-prediction restriction in small PUs. In their proposal, the bi-dieractional inter prediction for 4x8 and 8x4 partitions are disabled. Instead, bi-prediction is replace with list 1 uni-directional prediction for those small size PUs This effectively cuts the worst case motion compensation memory bandwidth by ~32%.  Compared to HM4.0 anchor, the average BD-rate increase is 0.0% in RA-HE, 0.0% in RA-LC, 0.1% in LB-HE and 0.0% in LB-LC. The results match the ones reported by proponents. The source code is also checked to verify its consistency with the proposal description.

.

# Introduction

This document reports cross-check results for Sharp’s proposal JCTVC-G307 on Bi-prediction restriction in small PUs. The proposal basically disables the bi-prediction for small block size PUs 4x8 and 8x4 to reduce memory bandwidth requirements for motion compensation.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The experimental results are summarized in Table 1. Compared to HM4.0 anchor, the average BD-rate increase is 0.0% in RA-HE, 0.0% in RA-LC, 0.1% in LB-HE and 0.0% in LB-LC. Those results match what proponents have reported.  Please be advised in Table 1 runtime may not be accurate.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | 0.2% | 0.1% | 0.0% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 94% |  |  | 92% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.3% | 0.1% |
| Class D | 0.1% | 0.4% | 0.5% | 0.0% | 0.1% | 0.2% |
| Class E | 0.0% | 0.6% | 1.0% | 0.0% | 0.5% | -0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.3% | 0.0% | 0.2% | 0.1% |
|  | 0.1% | 0.2% | 0.3% | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 89% |  |  | 90% |  |  |
| Dec Time[%] | 92% |  |  | 98% |  |  |

Table 1.  Experimental results

# Comments

# The proposed technique reduces the worst case motion compensation memory bandwidth requirements by 32% (in HM4.0 the worst case is 8x4 or 4x8 bi-prediction, reference block size for 8x8 is 660 = 15*11*2*2, now the worst case is 8x8 bi-prediction, reference block size for 8x8 is 450 = 15*15*2, 220/660 = 31.8%) without harming coding efficiency, this technique is very beneficial for memory bandwidth reduction purpose.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011