| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G295 |
| --- | --- |

| Title: | Non-CE4 Subtest3 : Extension of Adaptation Parameter Sets syntax for Quantization matrix |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junichi Tanaka Yoshitaka Morigami Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610 | Tel: Email: | +81-50-3750-2745 tanaka.junichi@jp.sony.com Yoshitaka.Morigami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

In WD4, Adaptation Parameter Sets(APS) is defined for ALF and SAO parameters[1]. APS has a parameter “aps_id” which is referred to in the slice header. Quantization matrix in HEVC was proposed in JCTVC-F362[2]/F475[3]. In those proposals, quantization matrices are transmitted as new NAL unit. This contribution propose,

- Extension of APS syntax for Quantization matrix.

- Instead of aps_id, new parameters: aps_sao_id, aps_alf_id and aps_qmatrix_id are defined.

- In slice header, aps_sao_id, aps_alf_id and aps_qmatrix_id are signaled individually.

# Problem

According to APS semantics, aps_sample_adaptive_offset_flag is equal to 0 specifies that the SAO is off for slice referred to the current APS. And aps_adaptive_loop_filter_flag is equal to 0 specifies that the ALF is off for slice referred to the current APS. If there is no active APS, the aps_sample_adaptive_offset_flag and aps_adaptive_loop_filter_flag values are inferred to be 0. Quantization matrices which can be changed frame by frame also should be sent in APS. It is necessary to send APS in any time when SAO or ALF parameter sets are changed, even if quantization matrices are not changed.

Figure 1: Example of NAL unit structure (current APS)

# Proposal

Instead of “aps_id”, new parameter: “aps_sao_id”, “aps_alf_id” and “aps_qmatrix_id” are proposed as sub_ids for SAO, ALF and Quantization matrix. In slice header, aps_sao_id, aps_alf_id and aps_qmatrix_id are signaled individually. It is possible that SAO, ALF and Quantization matrix refer to different sub_ids in different APS.

Figure 2: Example of NAL unit structure (proposed APS)

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| quantization_matrix_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Adaptation parameter set RBSP syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| aps_qmatrix_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ¦¦ aps_adaptive_loop_filter_flag ) { |  |
| aps_cabac_use_flag | u(1) |
| if( aps_cabac_use_flag ) { |  |
| aps_cabac_init_idc | ue(v) |
| aps_cabac_init_qp_minus26 | se(v) |
| } |  |
| } |  |
| if( aps_qmatrix_flag ){ |  |
| aps_qmatrix_id |  |
| qmatrix_param() |  |
| byte_align() |  |
| } |  |
| if( aps_sample_adaptive_offset_flag ) { |  |
| aps_sao_id | ue(v) |
| sao_data_byte_count /* to enable skipping past data without parsing it */ | u(8) |
| byte_align () |  |
| sao_param( ) |  |
| byte_align () |  |
| } |  |
| if( aps_adaptive_loop_filter_flag ) { |  |
| aps_alf_id | ue(v) |
| alf_data_byte_count /* to enable skipping past data without parsing it */ | u(8) |
| byte_align () |  |
| alf_param( ) |  |
| byte_align() | u(8) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id |  |
| if( sample_adaptive_offset_enabled_flag) |  |
| aps_sao_id |  |
| if(adaptive_loop_filter_enabled_flag) |  |
| aps_alf_id |  |
| if( quantization_matrix_enabled_flag) |  |
| aps_qmatrix_id |  |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| } |  |

# Conclusion

In APS, sub_ids for SAO, ALF and Quantization matrix are defined. In slice header, those sub_ids are signaled individually. It is possible to reduce the redundant data in APS. This contribution proposes,

- Extension of APS syntax for Quantization matrix.

- Instead of aps_id, new parameters: aps_sao_id, aps_alf_id and aps_qmatrix_id are defined.

- In slice header, aps_sao_id, aps_alf_id and aps_qmatrix_id are signaled individually.

# Reference

- “Adaptation Parameter Set (APS)”
	JCTVC-F747, Torino, July 2011

- “Proposal to support quantization matrix in HEVC”
	JCTVC-F362, Torino, July 2011

- “Enhancement of quantization matrix coding for HEVC”
	JCTVC-F475, Torino, July 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).