| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G296 |
| --- | --- |

| Title: | Picture-adaptive Field/Frame Coding: support for legacy video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Osnat Bar-Nir 10 Ha’amal St. Rosh Ha’ayin, 48092, Israel | Tel: Email: | +972 3 900 7785 Osnat.Bar-Nir@Harmonicinc.com |
| Source: | Harmonic Inc. |  |  |

_____________________________

# Abstract

This contribution proposes support for coding of interlaced video in HEVC standard, in order to improve coding efficiency of legacy interlaced content.

It is proposed to accommodate “field picture” representation on top of the existing “frame picture” representation. Frame or Field coding can be made adaptive at a sequence level and at a picture level.

# Introduction

The HEVC standard has been focusing on its application to new visual services in near-future, but it is clear that most of existing visual contents (up to HDTV), especially those produced in broadcasting area, are captured with interlaced camera system. In order to support this huge mass of legacy video in future broadcasting or package business, the HEVC video codec would need to handle interlace signal in some way [1].

Interlaced video sequences consist of two fields, captured at different time instants. Therefore in some cases, especially in sequences with high motion, encoding field pictures is more efficient than encoding frames which consist of two fields that were captured at different times. It was proposed in [1] to accommodate “field picture” representation to enable field coding of interlaced sources.

This contribution proposes to enable adaptive Field/Frame control in both sequence level and picture level, and it lists the required changes and additions to current syntax.

# Accommodating “field picture”

For simplification, it is proposed that field pictures must be paired. In field coding, each frame of interlaced video is divided into top and bottom fields. Fields have half the height of the original frame, and twice the frame rate.

A frame can be coded as I, P or B.

Two fields of the same frame are coded sequentially. A pair of fields can be coded as a pair of (I I), (I P), (P I), (P P) or (B B) fields.

In order to use field pictures some high level changes are required:

## Add new POC configuration to support field based encoding.

Picture order counts are used to identify pictures, to determine co-located for deriving motion parameters in temporal or spatial direct mode, to represent picture order differences between frames for motion vector derivation in temporal direct mode, for implicit mode weighted prediction in B slices, and for decoder conformance checking [4].

Currently, in WD4, each coded picture is associated with one picture order count, called PicOrderCnt. PicOrderCnt indicates the picture order of the corresponding picture relative to the previous IDR picture in decoding order. Supporting field coding would require the association of two picture order count per frame/pair of fields: TopFieldOrderCount and BottomFieldOrderCount.

- Change in assigning numbers for field based references in the reference lists. For each pair of reference fields, the reference field of the same parity as the current field is given a smaller number than the number of the reference field of the opposite parity, despite the longer temporal difference between fields of the same parity.

Current frame

2           1          0

4   5     2    3    0    1   current field

Time                                                                   Time

Ref. Frame Buffer	                                                           Ref. Frame Buffer

(a) Frame Coding                                            (b) Field Coding

Figure 1: Reference frame numbers

- Update derivation of neighbors and implicit motion vector; skip, merge and motion vector prediction in field coding. Rules for derivation of the above data are currently phrased for frame coding, new rules for field coding should be added where needed.

- Chroma MV adjustment

Chroma motion vector are derived from the corresponding luma motion vector.

Currently, in the derivation of the chroma motion vector (mvCLX), from the luma motion vector (mvLX) the following applies:

mvCLX[ 0 ] = mvLX[ 0 ],    mvCLX[ 1 ] = mvLX[ 1 ]

In case of field coding, when the current picture and the reference picture are fields of opposite parities, the vertical chroma motion vector should be adjusted as follow:

If the reference is a top field and the current is a bottom field then mvCLX[1] = mvLX[1] + 2

If the reference is a bottom field the current is a top field then mvCLX[1]  = mvLX[1] − 2

- A sequence level adaptive field/frame coding requires a sequence level flag to indicate whether the sequence is coded in frame mode or in field mode.

# Picture Level Adaptive Field/Frame

Picture level adaptive field frame (PAFF) guarantees higher coding efficiency, by choosing field/frame coding mode for each individual picture instead of making the decision at sequence level. In a picture level adaptive field/frame each frame can be coded either as a frame or as a pair of two fields.

This mode has a higher encoding efficiency, and it requires some more changes and additions on top of the changes listed in section 2, but the extra changes are relatively minor:

- Picture level field/frame signaling (in PPS).

- Handle mixed references where current picture and reference picture are not both in field mode or both in frame mode.

- Update derivation of neighbors and implicit motion vector; skip, merge and motion vector prediction in mixed field/frame coding. It is required to add some more rules for mixed field/frame coding on top of the rules added for pure field coding.

# Handling 3:2 Pulldown

For completeness, it is proposed to support telecined or 3:2 pulldown video by signaling various picture structures (pic_struct) as defined in AVC specification [2].

Such signaling will enable the encoder to remove duplicated fields and use frame mode for coding. This has no implications on the decoding process itself, while enabling correct display of exactly the same video as at the encoder input.  Simple switching between 24 Hz and 30 Hz video sequences does not always enable proper representation of the video at transistions between telecine and nontelecine.

# Conclusions

In this contribution it is proposed to use field coding mode for improving the efficiency of interlaced video coding. It is also proposed to use picture level adaptive field/frame instead of sequence level adaptive field/frame, to farther improve coding efficiency.  This requires only a relatively small and high-level addition, on top of sequence level field coding.

# Patent rights declaration(s)

Harmonic Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

# References:

[1] Shun-ichi Sekiguchi, Kazuo Sugimoto, Hiroharu Sakate, “Proposal on the support of interlace format in HEVC”, JCTVC-F194, July 2011.

[2] ISO/IEC 14496-10 | ITU-T Recommendation H.264

[3] Limin Wang, Krit Panusopone, Rajeev Gandhi, Yue Yu and Ajay Luthra, “Adaptive frame/field coding for JVT Video Coding”, JVT-B071, February 2002.

[4] Benjamin Bross et al, “WD4: Working Draft 4 of High-Efficiency Video Coding “, JCTVC-F803_d1_Buffer_Descriptions_display_process_suggestion_r1.doc, July 2011.