| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G299 WG11 Number: m21861 |
| --- | --- |

| Title: | CE8 Subtest h: Cross verification of NTT’s CU-based ALF with NLM filter (JCTVC-G235) by Intel |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of the CU-based ALF with non-local means filter in CE8 subtest h. The software package obtained from the proponent NTT was successfully compiled and tested under the CE8 test conditions defined in JCTVC-F908. The BD rate results match exactly to the data provided by the proponent, and the differences of enc/dec time are within normal variations.

# Algorithm study

The proponent proposed a unification of ALF and non-local means (NLM) approach, as shown in Figure 1. One flag per ALF-CU will be signaled in bitstream to indicate applying ALF or NLM for this CU. And in NLM filtered CU, each sample is adaptively processed as follows based on the local activity (Activity) between the sample and its neighboring samples.

- If Activity <= threshold_1, not filter the sample

- If Activity > threshold_1 && Activity  <= threshold_2, apply NLM filter with pixel template

- If Activity > threshold_2, apply NLM filter with cross-shaped template

Where, threshold_1 and threshold_2 are produced at encoder side and signaled in compressed bitstream.

Figure 1: Comparison of conventional CU-based ALF and proposed one.

# Performance data

The encoder was compiled in Linux OS and tested in a parallel computing system. The decoder was complied with Visual Studio 2008 and tested on a single server with following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM. The accompanied spreadsheet contains the detailed crosscheck data for the above three cases, and the average performance data are shown in following tables. The BD rate results match exactly to the data provided by the proponent, and the differences of enc/dec time are within normal variations.

Table 1 Crosscheck results of CU-based ALF with NLM filter

|  | All Intra HE |  |  |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  | Y | U | V |
| Class A | -0.1 | 0.1 | 0.1 |  | Class A | -0.2 | 0.0 | 0.0 |
| Class B | -0.1 | 0.1 | 0.1 |  | Class B | -0.3 | 0.1 | 0.2 |
| Class C | -0.2 | 0.2 | 0.2 |  | Class C | -0.4 | 0.1 | 0.2 |
| Class D | -0.1 | 0.1 | 0.1 |  | Class D | -0.1 | 0.3 | 0.1 |
| Class E | -0.2 | 0.2 | 0.2 |  | Class E |  |  |  |
| Overall | -0.1 | 0.1 | 0.1 |  | Overall | -0.2 | 0.1 | 0.1 |
| Enc Time[%] | 113% |  |  |  | Enc Time[%] | 107% |  |  |
| Dec Time[%] | 104% |  |  |  | Dec Time[%] | 102% |  |  |
|  | Low delay B HE |  |  |  |  | Low delay P HE |  |  |
|  | Y | U | V |  |  | Y | U | V |
| Class A |  |  |  |  | Class A |  |  |  |
| Class B | -0.3 | 0.0 | -0.1 |  | Class B | -0.5 | 0.0 | -0.1 |
| Class C | -0.5 | 0.2 | 0.2 |  | Class C | -0.6 | 0.1 | 0.1 |
| Class D | -0.3 | 0.4 | 0.5 |  | Class D | -0.4 | -0.2 | -0.8 |
| Class E | -0.3 | -0.1 | 0.5 |  | Class E | -0.7 | 0.1 | -0.1 |
| Overall | -0.3 | 0.1 | 0.2 |  | Overall | -0.5 | 0.0 | -0.2 |
| Enc Time[%] | 106% |  |  |  | Enc Time[%] | 109% |  |  |
| Dec Time[%] | 104% |  |  |  | Dec Time[%] | 105% |  |  |