| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: | Document: JCTVC-G301 |
| --- | --- |

| Title: | Test Results on Context Simplification for Coefficients Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name Xiaoyin Che Wenpeng Ding Jizheng Xu Yunhui Shi Baocai Yin | Tel: Email | wpding@bjut.edu.cn jzxu@microsoft.com syhzm@bjut.edu.cn ybc@bjut.edu.cn |
| Source: | Beijing University of Technology |  |  |

_____________________________

# Abstract

The proposed technique in this contribution is aimed to simplify contexts using in coefficients entropy coding, including two parts, coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. All tests about this contribution is based on HM4.0. The reduction of the used contexts is reported as -60% (from 60 to 24) for either coeff_abs_level_greater1_flag or coeff_abs_level_greater2_flag. The BD-Rate for the high efficiency intra, random access, low delay and low delay(P) configurations are reported as 0.02%, -0.03% , -0.13% and -0.10% respectively.

# Introduction

In HM4.0 high efficiency tools, lots of contexts has been used in coefficients entropy coding. The detailed statistics are showed in Table 1, with “one” and “abs” represent coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag, respectively. After research on these contexts, some of them are found with similar probabilities when using. So they could probably be combined.

Table 1 : Number of used context size for HM4.0 in coefficients coding (high efficiency)

|  | one | abs | all |
| --- | --- | --- | --- |
| intra | 60 | 60 | 120 |
| random access | 120 | 120 | 240 |
| low delay | 120 | 120 | 240 |
| Low delay (P) | 120 | 120 | 240 |

# Proposed Solution

In coefficients entropy coding in HM4.0, a whole set of contexts are used for intra frames only, and for inter frames, another set of contexts of same size are used. To simplify the contexts of coefficients entropy coding, new context assignment are proposed below. According to the different situation when the coding tool chooses contexts, 4 independent context assignments are designed for Intra Luma, Intra Chroma, Inter Luma and Intra Chroma respectively.

coeff_abs_level_greater1_flag

The 30 contexts using for coeff_abs_level_greater1_flag for luma coding (there are another 30 contexts for chroma, and the total number for coeff_abs_level_greater1_flag is 60) can be divided into 6 groups, according to the stats showed by previous blocks, with 5 contexts respectively. After simplification, only 12 contexts have been retained. And these 12 retained contexts, which are not continuous in physical memory in original HM4.0, will be reordered as 0~11, because the other 18 contexts will be directly deleted. Details showed in Table 2~5.

Table 2: Context simplification on coeff_abs_level_greater1_flag for Intra Luma coding

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 1 | 2 | 3 | 4 | 0 | 5 | 4 | 6 | 6 | 7 | 8 | 8 | 8 | 9 |
| Original | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| Proposed | 5 | 10 | 10 | 10 | 1 | 6 | 3 | 3 | 3 | 3 | 0 | 9 | 11 | 11 | 11 |

Table 3: Context simplification on coeff_abs_level_greater1_flag for Intra Chroma coding

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 6 | 4 | 7 | 8 | 9 | 10 | 5 | 9 |
| Original | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| Proposed | 3 | 1 | 1 | 1 | 1 | 4 | 11 | 3 | 3 | 11 | 9 | 7 | 4 | 7 | 7 |

Table 4: Context simplification on coeff_abs_level_greater1_flag for Inter Luma coding

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 1 | 1 | 1 | 2 | 3 | 4 | 4 | 2 | 2 | 5 | 6 | 6 | 7 | 8 |
| Original | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| Proposed | 2 | 9 | 9 | 9 | 10 | 8 | 2 | 2 | 1 | 1 | 0 | 7 | 11 | 11 | 8 |

Table 5: Context simplification on coeff_abs_level_greater1_flag for Inter Chroma coding

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 1 | 2 | 3 | 4 | 5 | 4 | 4 | 4 | 6 | 7 | 8 | 9 | 0 | 10 |
| Original | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| Proposed | 11 | 1 | 1 | 1 | 1 | 6 | 3 | 3 | 3 | 3 | 8 | 6 | 8 | 11 | 4 |

coeff_abs_level_greater2_flag

The proposed solution on coeff_abs_level_greater2_flag is quite similar with coeff_abs_level_greater1_flag, with 4 independent context assignment tables as follows.

Table 6: Context simplification on coeff_abs_level_greater1_flag for Intra Luma coding

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 1 | 2 | 3 | 4 | 5 | 1 | 6 | 2 | 7 | 2 | 2 | 3 | 3 | 4 |
| Original | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| Proposed | 8 | 9 | 10 | 11 | 6 | 0 | 10 | 5 | 11 | 6 | 1 | 6 | 6 | 6 | 2 |

Table 7: Context simplification on coeff_abs_level_greater1_flag for Intra Chroma coding

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 1 | 2 | 3 | 4 | 5 | 1 | 6 | 2 | 7 | 8 | 6 | 2 | 9 | 4 |
| Original | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| Proposed | 10 | 0 | 5 | 11 | 8 | 0 | 5 | 5 | 11 | 1 | 11 | 1 | 1 | 8 | 2 |

Table 8: Context simplification on coeff_abs_level_greater1_flag for Inter Luma coding

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 1 | 2 | 3 | 4 | 5 | 1 | 6 | 7 | 4 | 6 | 2 | 7 | 3 | 4 |
| Original | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| Proposed | 8 | 9 | 0 | 10 | 11 | 0 | 5 | 10 | 1 | 11 | 11 | 11 | 6 | 6 | 7 |

Table 9: Context simplification on coeff_abs_level_greater1_flag for Inter Chroma coding

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 1 | 2 | 3 | 4 | 5 | 1 | 6 | 2 | 4 | 6 | 6 | 2 | 7 | 4 |
| Original | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| Proposed | 8 | 9 | 0 | 5 | 6 | 0 | 5 | 1 | 1 | 6 | 10 | 5 | 5 | 11 | 1 |

After all, the whole simplification proposal is showed in Table 10 below.

Table 10: Numbers of context simpification for coefficients coding

|  | one | abs | All |
| --- | --- | --- | --- |
| Original | 60 | 60 | 120 |
| Proposed | 24 | 24 | 48 |
| Reduced | 36 | 36 | 72 |
| Reduce Ratio | 60.0% | 60.0% | 60.0% |

# Results

The proposed method’s performance relative to the HM4.0 software. The computing platform is Intel(R) Xeon(R) CPU X5570 @2.93GHz, Microsoft Windows Server 2003 R2 Enterprise x64 Edition Service Pack 2. Table 11 shows the test result of coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag.

Table 11 : Test result with coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag

|  | Intra |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.01 | -0.05 | -0.04 | -0.03 | 0.01 | -0.14 |
| Class B | 0.03 | 0.01 | 0.01 | -0.05 | -0.07 | -0.18 |
| Class C | 0.00 | 0.01 | 0.00 | -0.04 | -0.21 | -0.2 |
| Class D | 0.01 | 0.02 | -0.05 | -0.02 | -0.01 | -0.41 |
| Class E | 0.02 | 0.00 | 0.00 |  |  |  |
| All | 0.02 | 0.00 | -0.01 | -0.03 | -0.09 | -0.19 |

|  | Low delay |  |  | Low delay (P) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.16 | -0.26 | -0.58 | -0.16 | -0.04 | -0.24 |
| Class C | -0.14 | -0.21 | -0.44 | -0.16 | -0.39 | -0.36 |
| Class D | -0.06 | -0.07 | -0.1 | -0.02 | -0.05 | -0.80 |
| Class E | -0.16 | 0.49 | 0.57 | -0.04 | 0.72 | -0.17 |
| All | -0.13 | -0.06 | -0.21 | -0.10 | 0.01 | -0.40 |

# 4 Text for Working Draft

###### Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

….

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater1Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  5 )  +  min( 4, greater1Ctx )	(9-1)

ctxIdxInc  =  abs_level_greater1_map[IsChroma][ctxIdxInc]	(9-21)

###### Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

…

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater2Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  5 )  +  min(4, greater2Ctx )	(9-3)

ctxIdxInc  =  abs_level_greater2_map[IsChroma] [ctxIdxInc]	(9-46)

# Patent rights declaration(s)

Beijing University of Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)