| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G305r1 |
| --- | --- |

| Title: | Non-CE9: Bi-prediction for low delay coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshinori Suzuki Akira Fujibayashi 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +81 46 840 3517 suzukiyos@rd.nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

In this contribution the additional option of bi-prediction for low delay conditions and key pictures of random access conditions is proposed. In the proposed method, the motion vector difference for List 1 is not signaled and it sets to (0, 0) for GPB pictures (reference picture list 1 is identical to reference picture list 0). The average Y BD-rate gains for low delay conditions and random access conditions are 0.9% and 0.2%, respectively. Additionally, it is confirmed that the average Y BD-rate gains for random access conditions is 0.4% when GOP size is 4.

- Introduction

In the common conditions of HM4.0 [1], B slice is always utilized for inter coding even if for the low delay conditions, and it achieves a significant coding gain as shown in Table 1. Since a backward prediction is not used for low delay conditions, a bi-prediction produces such a gain.

The bi-prediction is employed for both merge mode and inter coding mode. Table 2 and Table 3 show the effect of bi-prediction in merge mode and inter coding mode for low delay conditions, respectively. In these tables, three coding conditions, that are “Low delay P”, “Low delay B” and “Low delay B, but uni-prediction is selected at encoder when merge_flag=0”, are compared. They indicate that the Y BD rate improvement by bi-prediction in merge mode is larger than in inter coding mode.

In this contribution, the improvement of bi-prediction in inter coding mode for GPB pictures (reference picture list 1 is identical to reference picture list 0) has been considered. In WD4.0 [2], the bi-prediction for GPB pictures is same as that for non-GPB pictures where bi-directional prediction can be utilized.

Table 1: Effect of B-slices (Reference; HM4.0 Low Delay P conditions, Tested; HM4.0 Low Delay B conditions)

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -7.1% | -7.7% | -7.5% | -10.0% | -7.3% | -7.6% |
| Class C | -6.3% | -5.7% | -6.0% | -6.3% | -4.8% | -5.1% |
| Class D | -6.9% | -6.2% | -6.0% | -6.3% | -5.3% | -5.2% |
| Class E | -6.2% | -6.0% | -5.5% | -9.3% | -6.1% | -5.7% |
| Overall | -6.7% | -6.5% | -6.4% | -8.0% | -6.0% | -6.0% |
|  | -6.8% | -6.5% | -6.4% | -8.1% | -5.9% | -6.1% |
| Enc Time[%] | 154% |  |  | 160% |  |  |
| Dec Time[%] | 114% |  |  | 120% |  |  |

Table 2: Effect of bi-prediction in merge mode (Reference; HM4.0 Low Delay P conditions, Tested; HM4.0 Low Delay B conditions, but uni-prediction is selected at encoder when merge_flag=0)

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -5.4% | -5.5% | -4.9% | -8.3% | -5.0% | -5.3% |
| Class C | -4.2% | -3.6% | -3.6% | -4.6% | -3.0% | -3.3% |
| Class D | -4.5% | -3.1% | -3.0% | -4.6% | -2.7% | -3.0% |
| Class E | -4.7% | -4.2% | -3.7% | -8.4% | -5.3% | -4.7% |
| Overall | -4.7% | -4.2% | -3.9% | -6.5% | -4.0% | -4.1% |
|  | -4.8% | -4.2% | -4.0% | -6.5% | -4.0% | -4.2% |
| Enc Time[%] | 109% |  |  | 109% |  |  |
| Dec Time[%] | 112% |  |  | 117% |  |  |

Table 3: Effect of bi-prediction in inter coding mode (Reference; HM4.0 Low Delay B conditions, but uni-prediction is selected at encoder when merge_flag=0, Tested; HM4.0 Low Delay B conditions)

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.0% | -2.5% | -2.9% | -2.0% | -2.5% | -2.6% |
| Class C | -2.1% | -2.2% | -2.5% | -1.8% | -1.9% | -1.8% |
| Class D | -2.6% | -3.4% | -3.2% | -1.8% | -2.7% | -2.3% |
| Class E | -1.7% | -1.9% | -1.9% | -1.0% | -0.9% | -1.0% |
| Overall | -2.1% | -2.5% | -2.7% | -1.7% | -2.1% | -2.0% |
|  | -2.2% | -2.5% | -2.6% | -1.8% | -2.1% | -2.0% |
| Enc Time[%] | 142% |  |  | 147% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

- Proposal

To further improve a coding performance of GPB pictures, the new option on bi-prediction in inter coding mode is proposed. There are four points to be modified in proposed method.

- When the new option is enabled, the difference between a list 1 motion vector components and its prediction is not signaled, and it sets to (0, 0) at decoder. In other words, list 1 motion vector components set to its prediction.

- A flag to identify if this option is utilized is signaled at the slice header. Therefore, the decoder can know easily whether the list 1 motion vector difference is parsed or not. In this contribution, the proposed option on bi-prediction is used when reference picture list 1 is identical to reference picture list 0 (GPB pictures). Other implementation or criteria by the encoder is also possible.

- In the HM4.0 encoder, the motion search of the list 0 motion vector for bi-prediction is conducted on the condition that the list 1 motion vector detected at uni-prediction ME process is fixed. In proposed method, instead of list 1 motion vector detected at uni-prediction ME process, the list 1 motion vector predictor is selected as the list 1 motion vector for bi-prediction.

- To find the best list 1 motion vector predictor which minimizes a prediction error, the best combination of the list 1 reference pictures and motion vector predictor candidates is considered in the uni-prediction ME process.

- Changes to the WD

The modifications of WD text [2] are shown in this section.

1)	One flag to identify if the list 1 motion vector difference is parsed is added to the slice header (cf. 3.1),

2) Prediction unit is modified so that the vector components of list 1 motion vector difference are not parsed and they are set to (0, 0) (cf. 3.2).

## Signaling of mvd_l1_zero_flag

The syntax element for mvd_l1_zero_flag is defined below. It is signaled in slice_header() when slice_type is B-slice.

### Slice header semantics

mvd_l1_zero_flag equals to 1 indicates that difference between a list 1 vector component and its prediction, mvd_l1[][][], is not parsed and it is set equal to 0. This flag equals to 1 indicates that mvd_l1[][][] is parsed.

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
|  |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
|  |  |
| } |  |

## Signaling of mvd_l1

The syntax element for mvd_l1[][][] is not signaled and it is set to 0 when mvd_l1_zero_flag equals to 1. Prediction unit syntax is modified to support this option.

### Prediction unit syntax

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
|  |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( mvd_l1_zero_flag ) { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] = 0 |  |
| mvd_l1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else { |  |
| if( entropy_coding_mode_flag ) { |  |
| mvd_coding_cabac(mvd_l1[ x0 ][ y0 ][ 0 ], mvd_l1[ x0 ][ y0 ][ 1 ]) |  |
| } else { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| } |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Results

Simulations were conducted for the above proposal using the common conditions defined in [1].  Table 4 and Table 5 below show the results for low delay conditions and random access conditions. In this test, the proposed option on bi-prediction is used when reference picture list 1 is identical to reference picture list 0 (GPB pictures).

BD-Rate savings of 0.9% and 0.2% were obtained for low delay conditions and random access conditions, respectively. The proposal does not have any impact on encoding and decoding times for all coding configurations. Additionally, it is confirmed that BD-rate saving of 0.4% was obtained for random access conditions when GOP size is 4 (-g 4 –rg 4) as shown in Table 6.

Table 4: Simulation result for default low delay condition

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -1.2% | -1.3% | -1.0% | -0.7% | -0.8% |
| Class C | -1.0% | -1.0% | -0.9% | -0.8% | -0.6% | -0.9% |
| Class D | -1.0% | -0.6% | -0.8% | -0.8% | -0.3% | -0.6% |
| Class E | -0.9% | -0.8% | 0.0% | -1.0% | -0.8% | -0.4% |
| Overall | -0.9% | -0.9% | -0.9% | -0.9% | -0.6% | -0.7% |
|  | -0.9% | -1.0% | -0.9% | -0.9% | -0.7% | -0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 5: Simulation result for default random access condition

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.4% | -0.5% | -0.5% | 0.1% | -0.1% |
| Class B | -0.3% | -0.4% | -0.3% | -0.3% | -0.3% | -0.3% |
| Class C | -0.2% | -0.3% | -0.2% | -0.1% | -0.2% | -0.1% |
| Class D | -0.2% | -0.2% | -0.3% | -0.1% | -0.3% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% |
|  | -0.2% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6: Simulation result for default random access condition (GOP size is 4)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.3% | -0.6% | -0.7% | -0.1% | -0.4% |
| Class B | -0.4% | -0.5% | -0.5% | -0.5% | -0.4% | -0.6% |
| Class C | -0.4% | -0.3% | -0.3% | -0.3% | -0.2% | -0.3% |
| Class D | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | -0.4% | -0.5% | -0.4% | -0.3% | -0.4% |
|  | -0.4% | -0.4% | -0.5% | -0.4% | -0.2% | -0.4% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Conclusion

This contribution proposes the new option on bi-prediction in inter coding mode for GPB pictures (for GPB pictures (reference picture list 1 is identical to reference picture list 0). In this option, a list 1 motion vector predictor is selected as a list 1 motion vector for bi-prediction and list1 vector components of motion vector difference are not signaled. By executing the motion search for a list 0 motion vector to find the bi-prediction signal of target PU, the encoder can produce the better bi-prediction signal at lower bit cost for inter prediction.

Though the modification of text is minor, it achieves 0.9% and 0.2% BD-rate improvement for low delay conditions and random access conditions, respectively, without any impact on encoding or decoding times for all coding configurations. When GOP size is 4, 0.4% BD-rate improvement is achieved for random access conditions.

It is recommended that the proposal is adapted into the WD5.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6rd Meeting: Torino, IT, 14-22 July, 2011.

- B. Bross et al., “WD4: Working Draft 4 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F803_d5, 6rd Meeting: Torino, IT, 14-22 July, 2011.