| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G307_r1 |
| --- | --- |

| Title: | Bi-prediction restriction in small PU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents bi-prediction restriction to reduce motion compensation complexity. In the proposed method, bi-prediction of 4x8 and 8x4 PU is prohibited with changes of inter_pred_flag syntax and merge motion parameter derivation process. It is reported that BD-rate yields 0.0 %, 0.1 %, 0.0, and 0.0 for RAHE, LBHE, RALC, LBLC condition respectively. Enc Time reduces about 5 % and 7 % compare to HM4.0 anchor in Hier-B case and B case respectively. Dec Time shows no impact. The proposal was cross-checked by TI (JCTVC-G289).

# Introduction

In HM-4.0 [1], the use of 4x4 inter prediction is prohibited when inter_4x4_enabled_flag is disabled [2]. This reduces motion compensation complexity especially for the worst case. However, considering bi-prediction case, the worst case is still large burden.

The motion compensation of small PU requires large computation and data transfer cost for its large window size. At the same time, the motion of small PU is relatively less constant where bi-prediction is not useful very much. Considering these characteristics, the prohibition of bi-prediction in small PU should be examined.

# Proposed method

Bi-prediction of 4x8 and 8x4 PU is prohibited with the following changes. The changes are related to inter_pred_flag syntax and inter prediction parameter derivation process in merge PU.

- inter_pred_flag syntax

When CU size is 8 and PartMode != PART_2Nx2N, bi-prediction is prohibited. In this case, inter_pred_flag is not present in the syntax and only uni-prediction is used.

- inter prediction parameter derivation in merge PU

The inter prediction parameters of merge mode could be inferred from one of neighboring blocks. If the inferred inter prediction parameters used bi-prediction, the parameter is converted to uni-prediction with L1 inter prediction parameters.

When CU size is 8 and PartMode != PART_2Nx2N, the following process is added to the last of the derivation process for spatial / temporal merging candidates.

If both predFlagL0[ xN, yN ] and predFlagL1[ xN, yN ] is 1, predFlagL0[ xN, yN ] is set to be 0

# WD text

The changes of WD text are highlighted as below.

// Changes of syntax

7.3.7 Prediction unit syntax

| prediction_unit( x0, y0 , log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| … |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| NoBipredFlag = (log2CUSize == 3 && PartMode != PART_2Nx2N) |  |
| if( !entropy_coding_mode_flag ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef && !NoBipredFlag) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| if (! NoBipredFlag) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| … |  |
| } |  |
| } |  |
| } |  |

combined_inter_pred_ref_idx specifies the values of inter_pred_flag and ref_idx_lX with X being 0, 1 or c. When it is not present, it shall be inferred to be equal to 0. [Ed. (WJ): needs to be improved]

When NoBipredFlag is equal to 1, codeword index mapping update process is not used. Otherwise, codeword index mapping update process is used.

The variables NumPredRefLC, NumPredRefL0 and NumPredRefL1 are specified as

NumPredRefLC = Min( 4, num_ref_idx_lc_active_minus1 + 1 )	(7-1)
NumPredRefL0 = Min( 2, num_ref_idx_l0_active_minus1 + 1 )	(7-2)
NumPredRefL1 = Min( 2, num_ref_idx_l1_active_minus1 + 1 )	(7-3)

The variable MaxPredRef is specified as

- If num_ref_idx_lc_active_minus1 is greater than 0, the following applies.

MaxPredRef = NumPredRefLC		(7-4)

- Otherwise (combined list is not used), the following applies.

MaxPredRef = NumPredRefL0		(7-5)

- When NoBipredFlag is equal to 0, the following applies.

MaxPredRef += NumPredRefL0 * NumPredRefL1		(7-xx)

// Changes of merge mode inter prediction parameter derivation

8.4.2.1.1 Derivation process for luma motion vectors for merge mode

…

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

…

- When slice_type is equal to B and NoBipredFlag is equal to 0, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked…

  - The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked….

8.4.2.1.2 Derivation process for spatial merging candidates

...

When noBipredFlag is equal to 1, predFlagL0[ xN, yN ] is equal to 1 and predFlagL1[ xN, yN ] is equal to 1, predFlagL0[ xN, yN ] is set to be 0

8.4.2.1.3 Derivation process for temporal merging candidates

...

When noBipredFlag is equal to 1, predFlagL0[ xN, yN ] is equal to 1 and predFlagL1[ xN, yN ] is equal to 1, predFlagL0[ xN, yN ] is set to be 0

8.4.2.1.6 Derivation process for zero motion vector merging candidates

    - If all of the following conditions are true,

      - slice_type is equal to P or NoBipredFlag is equal to 1

      - no candidate in mergeCandList has a list0 motion vector equal to (0,0) and reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0),

# Experimental result

The proposed method is implemented on HM-4.0 and simulation is conducted based on the common test condition. Since the proposed method only impacts on inter frame coding with B-frame, Random Access High Efficiency (RAHE), Low Delay B High efficiency (LBHE), Random Access Low Complexity (RALC) and Low Delay B Low Complexity (LBLC) are tested.

Table 1 summarizes the result of the experiment. Average BD-rate of 0.0 %, 0.1 %, 0.0 %, and 0.0 % are observed for RAHE, LBHE, RALC and LBLC respectively.

Table 1: Performance of the proposed method compared to anchor (HM-4.0)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | 0.2% | 0.1% | 0.0% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.04% | 0.04% | 0.05% | 0.03% | 0.02% | 0.07% |
|  | 0.05% | 0.05% | 0.06% | 0.02% | 0.03% | 0.08% |
| Enc Time[%] | 95% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.3% | 0.1% |
| Class D | 0.1% | 0.4% | 0.5% | 0.0% | 0.1% | 0.2% |
| Class E | 0.0% | 0.6% | 1.0% | 0.0% | 0.5% | -0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.07% | 0.19% | 0.33% | 0.04% | 0.20% | 0.08% |
|  | 0.07% | 0.15% | 0.29% | 0.05% | 0.18% | 0.12% |
| Enc Time[%] | 94% |  |  | 93% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 1: Performance of the additional experiments if L0 is used for merge PU

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class D | 0.1% | 0.2% | 0.2% | 0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.05% | 0.05% | 0.05% | 0.03% | 0.02% | 0.09% |
|  | 0.05% | 0.05% | 0.07% | 0.02% | 0.02% | 0.10% |
| Enc Time[%] | 95% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% |
| Class C | 0.1% | 0.2% | -0.1% | 0.1% | 0.2% | 0.0% |
| Class D | 0.1% | 0.1% | 0.4% | 0.1% | -0.1% | 0.3% |
| Class E | 0.1% | -0.3% | 0.6% | 0.0% | 0.4% | -0.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.09% | 0.02% | 0.19% | 0.07% | 0.13% | 0.05% |
|  | 0.09% | -0.02% | 0.15% | 0.07% | 0.11% | 0.08% |
| Enc Time[%] | 93% |  |  | 93% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

This contribution presents bi-prediction restriction to reduce motion compensation complexity. In the proposed method, bi-prediction of 4x8 and 8x4 PU is prohibited with changes of inter_pred_flag syntax and merge motion parameter derivation process. It is reported that BD-rate yields 0.0 %, 0.1 %, 0.0, and 0.0 for RAHE, LBHE, RALC, LBLC condition respectively. Enc Time reduces about 5 % and 7 % compare to HM4.0 anchor in Hier-B case and B case respectively. Dec Time shows no impact.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803 (d3), JCT-VC 6th Meeting, Torino, 2011

[2] J. Jung, B. Bross, P. Chen, and W.-J. Han, “CE9: Summary report of core experiment 9 on MV Coding and Skip/Merge operations” , JCTVC-F029, JCT-VC 6th Meeting, Torino, 2011

[3] M. Zhou, “Cross-verification of Sharp’s proposal JCTVC-G307 on Bi-prediction restriction in small PUs, ” JCTVC-G289,  JCT-VC 7th Meeting, Geneva, 2011

# IPR Statement

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).