| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G308 |
| --- | --- |

| Title: | Complexity reduction of significant map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents a complexity reduction method of significant map coding. In the proposed method, the number of references used to derive context in significant_coeff_flag for high frequency area of 16x16 and 32x32 block is reduced to 0 or 3 from current 5. The proposal can also eliminate clip function and reduce exceptional procedure regarding to block in high frequency area.The ratio of the area where complexity reduction is applied is about 60 % on 16x16 and 32x32 block. It is reported that BD-rate yields 0.0 %, 0.1 %, 0.1, and 0.1% for AIHE, RAHE, LBHE, and LPHE condition respectively. Enc Time and Dec Time shows no impact (100%).

# Introduction

In HM-4.0 [1][2][3], the context of significant_coeff_flag is derived by the number of non-zero coefficients surrounding the target coefficient. The number of reference is 5. Because the significant_coeff_flag must be coded whether the coefficient is non-zero or not, the complexity can be critical in the case the last non-zero coefficient lies in high frequency position. This worst-case complexity should be addressed. The proposal reduces the number of references, and thus reduces the worst case complexity significantly.

# Proposed method

In the proposed method, the coefficients of 16x16 and 32x32 transform are classified into 5 regions, R0 to R4 as Figure 1. In Table 1, the condition of the classification and its context derivation method is described.

In the proposal, in R3 region the number of references decreases to 3 and in R4 that is eliminated to zero where HM4.0 uses 5. In R3, the clip function (for max value 4) is also eliminated and the exceptional procedure regarding to block also can be reduced. The categories R2 and R3 share the common contexts while the region R4 uses new one context. In total, the number of contexts becomes 45 from current 44.

Figure 1: Definitions of the categories for context derivation of significant_coeff_flag 
(the number of reference)

Table 1: Classification of coefficients and its context derivation method

| Region | Condition | Derivation method (HM4.0) | Derivation method (Proposal)l | Number of coeffs in 16x16 (ratio) | Number of coeffs in 32x32 (ratio) |
| --- | --- | --- | --- | --- | --- |
| R0 | xC + yC <= 1 | position | position (0 reference) | 3 | 3 |
| R1 | xC + yC < 5 | 5 references | 5 references | 7 | 7 |
| R2 | xC + yC < TU size * 7 / 8 | 5 references | 5 references | 95 | 243 |
| R3 | xC + yC < TU size + 6 |  | 3 references | 106 (41 %) | 106 (29 %) |
| R4 | Otherwise |  | 0 references | 45(26%) | 325(32%) |

where TU size is the width of transform block

# WD text

The changes of WD text are highlighted as below.

9.3.3.1.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

// R0

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC	(9-1)

// R1

- Otherwise if xC + yC is less than 5, sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC ][ yC + 1 ]  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-2)
sigCtx  =  34  +  Min( 4, temp )

// R2

- Otherwise if xC + yC is less than ( 1 << log2TrafoSize )  * 7 / 8 ), sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC ][ yC + 1 ]  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-3)
sigCtx  =  39  +  Min( 4, temp )

// R3

- Otherwise (xC  +  yC is less than ( 1 << log2TrafoSize )  + 6), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  39		(9-4)

- When xC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-5)

- When xC and yC are less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-6)

- When xC is less than ( 1 << log2TrafoSize ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-7)

- When yC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-8)

- When yC is less than ( 1 << log2TrafoSize ) − 2 and sigCtx is less than 43, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-9)

// R4

- Otherwise (xC  +  yC is equal to or greater than ( 1 << log2TrafoSize )  + 6), sigCtx is derived as follows.

sigCtx  =  44		(9-xx)

# Experimental result

The proposed method is implemented on HM-4.0 and simulation is conducted based on common test condition. Since the proposed method only impacts on CABAC using condition, All Intra High Efficiency (AIHE), Random Access High Efficiency (RAHE), and Low Delay B High Efficiency (LBHE), and Low Delay P High Efficiency (LPHE) are tested.

Table 2 summarizes the result of the experiment. Average BD-rate loss of 0.0 %, 0.1 %, 0.1 %, and 0.1 % are observed for AIHE, RAHE, LBHE and LPHE respectively.

Table 1: Performance of the proposed method compared to anchor (HM-4.0)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.3% | 0.3% |
| Class C | 0.1% | 0.2% | 0.3% |
| Class D | 0.0% | 0.2% | 0.2% |
| Class E | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% |
|  | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | -0.1% |
| Class B | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | 0.1% | 0.2% |
| Class D | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.1% | -0.1% |
| Class C | 0.1% | 0.1% | -0.1% |
| Class D | 0.1% | -0.1% | -0.4% |
| Class E | 0.0% | -0.7% | 0.8% |
| Class F |  |  |  |
| Overall | 0.1% | -0.1% | 0.0% |
|  | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.3% | 0.2% |
| Class C | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.1% | -0.4% |
| Class E | -0.1% | -0.3% | -0.2% |
| Class F |  |  |  |
| Overall | 0.1% | 0.1% | -0.1% |
|  | 0.1% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

This contribution presents a complexity reduction method of significant map coding. In the proposed method, the number of references used to derive context in significant_coeff_flag for high frequency area of 16x16 and 32x32 block is reduced to 0 or 3 from current 5. The proposal can also eliminate clip function and reduce exceptional procedure regarding to block in high frequency area.The ratio of the area where complexity reduction is applied is about 60 % on 16x16 and 32x32 block. It is reported that BD-rate yields 0.0 %, 0.1 %, 0.1, and 0.1% for AIHE, RAHE, LBHE, and LPHE condition respectively. Enc Time and Dec Time shows no impact (100%).

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803 (d3), JCT-VC 6th Meeting, Torino, 2011

[2] V. Sze, M. Budagavi, “CE11: Parallelization of HHI_TRANSFORM_CODING (Fixed Diagonal Scan) ,” JCTVC-F129, JCT-VC 6th Meeting, Torino, 2011

[3] J. Sole, R. Joshi, M. Karczewicz, “CE11: Unified scans for the significance map and coefficient level coding in high efficiency,” JCTVC-F288, JCT-VC 6th Meeting, Torino, 2011

# IPR Statement

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).