| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G309 |
| --- | --- |

| Title: | Modifications of offset classification in SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution describes modifications of offset classification in SAO (Sample Adaptive Offset).  In this proposal, offset classification tables are modified for both offset types: Edge Offset (EO) and Band Offset (BO).  For EO types, the fifth offset is employed on pixels which edgeIdx is 2.  For BO types, a new band-classification table is introduced for chroma values.  It is reported that the proposed method improves coding efficiency of chroma channels by averages of 0.3%, 0.4% and 0.5% for IO_HE, RA_HE and LB_HE condition respectively.

# Introduction

In the current WD [1], the same classification tables are used for luma and chroma channels in SAO.  For EO types, pixels are classified into five classes and offsets are coded four out of five classes.  For BO types, pixels are classified into 17 classes and offsets are coded for 16 classes.  Note that, in either case, one class (class 0) is assigned as a non-filtering class for which offsets are not coded.  The tables for these classifications convert from edge index or  band index to class index.  An edge index represents a category of edge which a pixel and its neighbors form while a band index represents a band to which a pixel value belongs.  However, as for chroma pixels such as Cb and Cr, the value 128 (in an 8-bit depth data) is more important than 0 and 255, because it is a special value of achromatic color.  In view of this, the proposed method modified modifies the classification tables in accordance with the characteristics of chroma pixels.

# Proposed method

## Band Offset

When BO type is used in the current WD4, as shown in Table 1, the table ‘bandTable’ is used to derive an index of SAO offset from two other indexes saoTypeIdx and bandIdx.  The variable saoTypeIdx represents the type of SAO offsets, such as BO_0 and BO_1 while the variable bandIdx represents a range of pixel values.  In the proposed method, a new table ‘bandTableC’ shown in Table 2 is introduced for chroma bands.  As shown in Table 2, bandTableC is designed to give greater importance to the middle range of bands.  In our initial investigations, HM4.0 uses mostly the original EO and BO_0 type in chroma channels but rarely uses BO_1 for almost all classes and conditions.  The maximum ratio of area where that BO_1 is applied is 1.2% in classA of AI_LC condition.  It shows that the achromatic and near-achromatic bands count with BO types in chroma values. Consequently, the new tables are designed so that offsets are could always be applied to the achromatic and near-achromatic bands whenever the SAO type is BO.

Table 1: specifications of array bandTable according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

Table 2: specifications of array bandTableC according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 13 | 14 | 15 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 0 | 0 | 0 | 0 |

## Edge Offset

When EO type is used in the current WD4, like BO types, the table ‘edgeTable’ is used to derive an index of SAO offset from ‘edgeIdx’, which specifies a category of edge on a pixel. The array of edgeTable is defined as follows:

edgeTable[5] = { 1, 2, 0, 3, 4}

In this definition, each elements of array means the offset index of the edge offset. The third element indicates that the pixels of the category ‘edgeIdx=2’ are classified as class 0 (not filtered).  In this proposed method, a new definition of the table ‘edgeTable’ is introduced so that the ‘edgeIdx=2’ pixels are classified into the additional class (i.e. 5) and filtered. The table is defined as follows:

edgeTable[5] = { 1, 2, 5, 3, 4}

This table is used in both luma and chroma channels in order to reduce modification, and the fifth offset for EO types are always coded.

# Experimental result

The proposed method is implemented on HM-4.0 and simulation is conducted based on common test condition.

Table 3 summarizes the result of the experiment. As for the chroma channels, consistent average gains (negative values) from 0.2% to 0.5% are observed in HE conditions. In particular, the gain is 1.8% in Class E of LB_HE condition.  In LC conditions, the gains are relatively low, but the gains of 0.1% are observed on averages expect AI_LC condition. The average loss of 0.2% is observed only for the U-channel of LB_LC condition. As for the luma channel, the EO table modification is 0.0% on average and no impact was obsereved.

Regarding encoding and decoding time, no major differences from anchor were observed.

Appendix includes results of individual modifications and all results with class F.

Table 3: Performance of the proposed method compared to anchor (HM-4.0)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | -0.2% | -0.4% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | -0.4% | -0.4% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.9% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | -0.3% | -0.4% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | -0.2% | -0.3% | 0.1% | 0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.3% | -0.4% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.3% | -0.4% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.7% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.5% | 0.0% | 0.2% | -0.1% |
| Class D | 0.1% | 0.0% | -0.4% | 0.0% | 0.2% | 0.2% |
| Class E | -0.2% | -1.8% | -0.2% | -0.1% | 0.4% | 0.2% |
| Overall | 0.0% | -0.5% | -0.5% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | -0.5% | -0.6% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

This contribution presents a new offset classification in SAO, by using new tables to classify pixels.  It is reported that the proposed method improves coding efficiency in chroma channels by averages of 0.3%, 0.4% and 0.5% for IO_HE, RA_HE and LB_HE condition respectively without any impact on computing time. Based on the result, it is proposed to adopt the proposed method into WD and HM.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803 (d3), JCT-VC 6th Meeting, Torino, 2011

# IPR Statement

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Table 4: Performance of BO modification compared to anchor (HM-4.0, including classF)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | -0.1% | -0.3% | 0.0% | -0.2% | -0.3% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% |
| Class D | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.3% | -0.4% | 0.0% | -0.3% | -0.4% |
| Overall | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.3% |
| Class D | 0.0% | 0.1% | -0.2% | 0.0% | -0.1% | -0.6% |
| Class E | 0.0% | -0.2% | 0.4% | -0.1% | -0.5% | 0.3% |
| Class F | -0.1% | -0.5% | -0.1% | 0.1% | -0.2% | -0.6% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.3% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 5: Performance of EO modification compared to anchor (HM-4.0, including classF)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | -0.4% | -0.4% | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.8% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.1% | -0.3% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.4% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | -0.1% | -0.3% | 0.0% | 0.2% | 0.1% |
| Class D | 0.0% | 0.4% | -0.1% | 0.0% | 0.0% | -0.3% |
| Class E | -0.3% | -1.9% | -0.4% | 0.0% | 0.4% | 0.3% |
| Class F | -0.2% | -0.4% | 0.0% | 0.0% | -0.1% | -0.2% |
| Overall | -0.1% | -0.4% | -0.2% | 0.0% | 0.1% | 0.0% |
|  | -0.1% | -0.4% | -0.3% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6: Performance of the proposed method compared to anchor (HM-4.0, including classF)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | -0.2% | -0.4% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | -0.4% | -0.4% | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | -0.4% | -0.5% | 0.0% | -0.2% | -0.4% |
| Overall | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.9% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | -0.3% | -0.4% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | -0.2% | -0.3% | 0.1% | 0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.5% | -0.6% | 0.0% | -0.3% | -0.4% |
| Overall | 0.0% | -0.3% | -0.5% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.3% | -0.4% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.7% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.5% | 0.0% | 0.2% | -0.1% |
| Class D | 0.1% | 0.0% | -0.4% | 0.0% | 0.2% | 0.2% |
| Class E | -0.2% | -1.8% | -0.2% | -0.1% | 0.4% | 0.2% |
| Class F | -0.2% | -0.5% | -0.8% | 0.1% | 0.1% | -0.2% |
| Overall | -0.1% | -0.5% | -0.5% | 0.0% | 0.2% | 0.0% |
|  | -0.1% | -0.5% | -0.6% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |