| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G310 |
| --- | --- |

| Title: | CE5: CAVLC Adaptation using Difference Counter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports performance of CAVLC adaptation using difference counter in the context of CE5. Difference counter does not require normalization process used in the counter based CAVLC method adopted in HM-4.0. The proposed method could also reduce the number of required counters while keeping coding gain achieved by the current counter based adaptation method.

# Introduction

In the last meeting, difference counter method [1] was proposed, which modifies counter-based adaptation for CAVLC [2] in HM. This contribution reports experimental result of difference counter implemented on latest reference software (HM-4.0) in the context of CE5. Algorithm description is also included for easier reference of the proposed medhod.

# Alogorithm description

In the proposed method, the difference counter is introduced in place of the counter used in HM. Conceptually, the difference counter maintains the difference of occurrences of two successive codeNum. A swap of decoding table occurs when the value of a difference counter is zero. In other words, a swap occurs when the occurrences of the two successive codeNum are the same. Note that this is the similar condition that is used in the counter based method in HM.

In the proposed method, instead of strictly maintaining actual difference of occurrences, a simplified approach is used. Figure 1 illustrates decoding table and counters / difference counters for both the conventional and the proposal method. Figure 2 shows the decoding table adaptation processes used in each method.

As shown in Figure 2, normalization process used in HM is not required in the proposed method. A value stored in a counter in HM could have value of 0…31, which is assured by the periodical normalization process. In contrast, a value stored in a difference counter could have value of 0…3 without normalization. This difference in value range indicates that the proposed method needs fewer bits for each counter. It also should be noted that the proposed method does not require sum counter. In addition, dcnt[0] in Figure.1 (b) is not necessary because its value is always zero.

(a) HM-4.0

(b) Proposed method

Figure 1: Example of decoding table and associated counter / difference counter

(a) HM-4.0

(b) Proposed method

Figure 2: Decoding table adaptation process

# Experimental result

Proposed method has been implemented on HM-4.0 reference software and the experiment was carried out in the context of CE5. The result is summarized in Table 1. It seems that the coding efficiency impact by the proposed method is negligible.

Table 1: BD-rate and encoding/decoding performance of proposed method

# Conclusion

In this contribution, CAVLC adaptation using difference counter is proposed. The proposed method does not require normalization process needed in current HM. In addition, it requires fewer counters. It was verified through experiments that the coding efficiency impact is negligible. Based on these facts, it is recommended to include the proposed method in HM and WD.

# Reference

[1] T. Yamamoto, “CAVLC Adaptation using Difference Counter,” JCTVC-F395, JCT-VC 6th Meeting, Torino, 2011

[2] L. Guo, B. Li, X. Wang, M. Karczewicz, and J. Xu, “CE5: Counter based adaption for LCEC,” JCTVC-E143, JCT-VC 5th Meeting, Geneva, 2011

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).