| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G311 |
| --- | --- |

| Title: | CE6b: Cross-check of Intra Mode Coding (JCTVC-G242 and JCTVC-G243) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides cross-check results of three proposals on intra mode coding in the context of CE6b. Cross-checked proposals are described in JCTVC-G242 and JCTVC-G243. For each proposal, it is confirmed that the bitrate and PNSR values completely match with those provided by the proponents.

# Introduction

In CE6b [1], based on the contributions in the last meeting, several proposals on intra mode coding were evaluated. In this contribution, cross-check results of three of those proposals are reported. First and second proposals are described in JCTVC-G242 from Canon. The first one is based on JCTVC-F269, F091, and F106. Te second one is based on JCTVC-F062, F459, F091, F106, and F269. The third one is JCTVC-G243 jointly from MediaTek, Qualcomm, Sony, LG, and Canon based on JCTVC-F062, F459, F091, F106, and F269. The first and second proposal is refered as Canon-2MPM and Canon-3MPM respectively. And the third one is refered as Unified-IMC in the following part of this contribution.

# Experimental condition

Simulation was carried out based on the condition defined in CE6. The software was provided by proponents. Intra Only case was tested for both High Efficiency and Low Complexity configurations. The computing platform is summarized in Table 1. In the experiments, encoding was done using the grid system composed of variety of PCs, but it was configured so that the same-performance PC is used to encode both anchor and the proposed method for each test point. Decoding was done using the same machine for both anchor and the proposed method.

Table 1: Computing platforms

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Linux (64bit) | Linux (64bit) |
| CPU | Intel Core2 Quad Q9550 2.83GHz Intel Xeon X5667 3.07GHz | Intel Core2 Quad Q9550 2.83GHz Intel Xeon X5667 3.07GHz |
| Compiler | gcc 4.4.3 | gcc 4.4.3 |

# Experimental result

The experimental results are summarized in the tables below. In Table 2, the performance of Canon-2MPM is shown. In Table3, the performance of Canon-3MPM is shown. Table 4 and 5 both shows the performance of Unified-IMC, specifically, Table 4 shows the result of the version when LM related bugfix is applied. Detailed results are included in the attachment. The resulted bitrate and PSNR values completely match with those provided by the proponents. It is also confirmed that encoding and decoding time is similar to those provided by the proponent.

Table 2: Performance of Canon-2MPM compared to anchor (HM-4.0)

Table 3: Performance of Canon-3MPM compared to anchor (HM-4.0)

Table 4: Performance of Unified-IMC with LM bugfix compared to 
anchor (HM-4.0 with LM bugfix)

Table 5: Performance of Unified-IMC without LM bugfix compared to anchor (HM-4.0)

# Conclusion

This contribution provides cross-check results of three proposals on intra mode coding in the context of CE6b. Cross-checked proposals are described in JCTVC-G242 and JCTVC-G243. For each proposal, it is confirmed that the bitrate and PNSR values completely match with those provided by the proponents.

# References

[1] A. Tabatabai, E. Francois, K. Chono, H. Yu, R. Joshi, and J. Lainema, “CE6: Intra Coding Improvements,” JCTVC-F903 (r2), JCT-VC 6th Meeting, Torino, 2011