| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G312 |
| --- | --- |

| Title: | Non-Square Partition Mode Grouping for CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents non-square partition mode grouping for CAVLC. In the proposed method, syntax cu_split_pred_part_mode is modified so that one of its values corresponds to the group of non-square partition modes. Also, additional information is signaled to indicate a selected non-square partition. It is reported that the proposed method improves coding efficiency by 0.3% and 0.6% for RALC and LBLC condition respectively.

# Introduction

In the current WD [1], partition mode (PartMode) is encoded using a syntax cu_split_pred_part_mode jointly with other information such as CU split flag, skip flag, merge flag and CU prediction mode when CAVLC is used. Adaptation to the occurrence of each PartMode is achieved by applying counter based decoding table swap to the syntax. Basically, the adaptation method assigns shorter bits to the PartMode that was recently appeared. After adoption of Asymmetric Motion Partition (AMP) [2] in the last meeting, the number of PartMode increases and that causes longer maximum length of PartMode bit string as well as slow down of the adaptation. For example, after series of appearance of non-2Nx2N partitions, too long bit string is assigned to 2Nx2N partition. And once longer bit string is assigned, many 2Nx2N partitions need to be appeared before shortest bit string is assigned. The proposed PartMode coding method detailed in the following chapter allows shorter maximum length of PartMode bit string as well as faster adaptation of the decoding table by grouping non-square PartMode (i.e. 2NxN/2NxnU/2NxnD/Nx2N/nLx2N/nRx2N).

# Proposed method

In the proposed method, all of non-square PartMode: PART_2NxN, PART_2NxnU, PART_2NxnD, PART_Nx2N, PART_nLx2N, and PART_nRx2N are grouped into one set which is coded as cu_split_pred_part_mode. Association between values of cu_split_pred_part_mode and PartMode is modified as shown in Table 1 and 2. In CU larger than 8x8, the value 4 corresponds to the group of non-square PartMode. Additional syntax horz_part_flag and rem_part_mode is signaled to select one PartMode from the group. In 8x8 CU, the value 3 corresponds to the group and only horz_part_flag is additionally signaled due to lack of AMP.

The value of cu_split_pred_part_mode is encoded using decoding table with counters and truncated unary code as VLC. In this aspect, the proposed method is the same as WD4 including decoding table initialization and number of counters. Only difference is the max value for truncated unary code. The value of rem_part_mode is also encoded with truncated unary code but without decoding table.

The above design has been decided considering the following statistical observations of PartMode;

1) Ratio of Inter PART_2Nx2N is significantly high thus shorter bit string should be assigned.

2) Ratio of Intra CU is relatively high and it is often the case that Intra CUs are successively appeared.

3) Ratio of Non-square PartMode is relatively low. But since they tend to be selected in complex regions, once one of them is selected, probabilities that other non-square PartMode appears also increases.

Regarding item 1) and 2), narrower ranges of cu_split_pred_part_mode is considered to be effective. And regarding item3), grouping non-square PartMode into one value of cu_split_pred_part_mode could well reflect the property.

Table 1: Definition of cu_split_pred_part_mode (CU size larger than 8x8)

| cu_split_pred_ part_mode | horz_part_flag | rem_part_mode | PredMode | PartMode |
| --- | --- | --- | --- | --- |
| 3 | - | - | MODE_INTER | PART_2Nx2N |
| 4 (escape) | 1 | 0 | MODE_INTER | PART_2NxN |
|  |  | 1 |  | PART_2NxnU |
|  |  | 2 |  | PART_2NxnD |
|  | 0 | 0 | MODE_INTER | PART_Nx2N |
|  |  | 1 |  | PART_nLx2N |
|  |  | 2 |  | PART_nRx2N |
| 5 | - | - | MODE_INTRA | PART_2Nx2N |

Table 2: Definition of cu_split_pred_part_mode (CU size equal to 8x8)

| cu_split_pred_ part_mode | horz_part_flag | rem_part_mode | PredMode | PartMode |
| --- | --- | --- | --- | --- |
| 2 | - | - | MODE_INTER | PART_2Nx2N |
| 3 (escape) | 1 | - | MODE_INTER | PART_2NxN |
|  | 0 | - | MODE_INTER | PART_Nx2N |
| 4 (escape) | - | - | MODE_INTRA | PART_2Nx2N |
|  | - | - | MODE_INTRA | PART_NxN |

# Experimental result

The proposed method is implemented on HM-4.0 and simulation is conducted based on common test condition. Since the proposed method impacts only on inter frame coding with CAVLC, Random Access LC (RALC) and Low Delay B LC (LBLC) are tested.

Table 3 summarizes the result of the experiment. Coding gain of 0.3% and 0.6% are observed for RALC and LBLC respectively. The gains are consistent over all classes. Regarding encoding and decoding time, no major differences from anchor were observed.

Table 3: Performance of the proposed method compared to anchor (HM-4.0)

# Conclusion

This contribution presents non-square partition mode grouping for CAVLC in order to take better account of statistics into encoding of PartMode. It is reported that the proposed method improves coding efficiency by 0.3% and 0.6% for RALC and LBLC condition respectively without almost any impact on computing time. Based on the result, it is proposed to adopt the proposed method into WD and HM.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803 (d3), JCT-VC 6th Meeting, Torino, 2011

[2] I.-K. Kim, W.-J. Han, J. Park, X. Zheng, “CE2: Test results of asymmetric motion partition (AMP),” JCTVC-F379, JCT-VC 6th Meeting, Torino, 2011

# IPR Statement

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).