| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G313 |
| --- | --- |

| Title: | CE4 Subtest 1.3.c: Cross-verification of JCTVC-G067 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi 1-9-2 Nakase, Mihama-ku, Chiba, JAPAN | Tel: Email: | +81 43 299 8526 yasugi.yukinobu@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents cross-verification results of CE4 Subtest 1.3.c, which is the study on the scheme of special QP prediction proposed by NEC, Canon and Sony (JCTVC-G067). The implemented program was appropriate and all simulations of cross-verification were done successfully. The results matched those of the proponents.

# Introduction

The proponents, NEC/Canon and Sony have proposed the combination of methods of spatial QP prediction in G067 [1]. According to the description of CE4 [2], it has three points.

- If the top-left-most CU is intra-coded and its intra prediction mode is directional, predicted QP is derived based on intra prediction direction.
If the prediction direction is upward (iIntraDirOrder < 18), predicted QP is derived from the above CU. Otherwise, if the prediction direction is leftward (iIntraDirOrder >= 18), predicted QP is derived from the left CU.

- Otherwise, if at least either of the above CU or the left CU have the same prediction mode as that of the current CU, predicted QP is derived from those CUs. When both the above and left CUs have the same prediction mode, the averaged QP is used.

- Otherwise, if at least either of the above CU or the left CU is available, predicted QP is derived from those CUs. When both the above and left CUs are available, the averaged QP is used.

These three techniques are enough simple and reasonable. As for the codes, the implementation of the method was correct and the number of modified lines seems to be not really a problem.

# Experimental result

Experiments were carried out under the condition of CE4 Subtest 1 [2] with the bug-fixed version of the proposed software. Table 1 shows the summary of results correspond to MaxCUDepthDQP=3.

Table 1: Simulation results of CE4 Subtest 1.3.c (MaxCUDepthDQP=3)

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.3% | -0.3% | -6.5% | -0.2% | -0.2% | -0.2% | -6.6% |
| Class B | -0.4% | -0.4% | -0.4% | -9.8% | -0.4% | -0.4% | -0.4% | -10.2% |
| Class C | -0.5% | -0.5% | -0.4% | -11.3% | -0.6% | -0.5% | -0.5% | -12.3% |
| Class D | -0.5% | -0.4% | -0.4% | -10.8% | -0.5% | -0.5% | -0.5% | -12.0% |
| Class E | -0.8% | -0.8% | -0.8% | -14.7% | -0.8% | -0.8% | -0.8% | -15.3% |
| Overall | -0.5% | -0.4% | -0.4% | -10.4% | -0.5% | -0.5% | -0.5% | -11.0% |
|  | -0.5% | -0.4% | -0.4% |  | -0.5% | -0.5% | -0.5% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | 0.2% | -0.3% | -4.7% | -0.3% | -0.2% | -0.3% | -4.6% |
| Class B | -0.3% | -0.3% | -0.2% | -6.5% | -0.3% | -0.3% | -0.4% | -6.9% |
| Class C | -0.4% | -0.4% | -0.5% | -9.0% | -0.5% | -0.5% | -0.5% | -10.3% |
| Class D | -0.3% | -0.2% | -0.1% | -7.9% | -0.4% | -0.5% | -0.5% | -9.0% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.2% | -0.3% | -7.0% | -0.4% | -0.4% | -0.4% | -7.6% |
|  | -0.3% | -0.2% | -0.3% |  | -0.4% | -0.4% | -0.4% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.3% | -5.4% | -0.3% | -0.4% | -0.4% | -5.2% |
| Class C | -0.4% | -0.5% | -0.3% | -7.8% | -0.6% | -0.6% | -0.4% | -9.0% |
| Class D | -0.4% | -0.3% | -0.3% | -7.0% | -0.4% | -0.3% | -0.7% | -8.0% |
| Class E | -0.4% | -1.0% | -0.6% | -8.4% | -0.5% | -1.1% | -0.7% | -8.4% |
| Overall | -0.4% | -0.4% | -0.3% | -7.0% | -0.4% | -0.5% | -0.5% | -7.5% |
|  | -0.4% | -0.5% | -0.3% |  | -0.4% | -0.5% | -0.6% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |

# Conclusion

Cross-verification of JCTVC-G067 was performed by SHARP. The techniques were studied and the results provided by the proponents were confirmed.

# References

[1] H. Aoki, K. Chono, M. Kobayashi, M. Shima, and K. Sato, “CE4 Subtest1: test 1.3.c,” JCTVC-G067, JCT-VC 7th Meeting, Geneva, 2011

[2] K. Sato, M. Budagavi, M. Coban, H. Aoki, and X. Li, “Description of Core Experiment 4: Quantization,” JCTVC-F904, JCT-VC 6th Meeting, Torino, 2011