| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, November, 2011 | Document: JCTVC-G316_r1 |
| --- | --- |

| Title: | CE8.a.1 2-D mergeable syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Takashi Watanabe1 Takeshi Chujoh1 Ching-Yeh Chen2 Chih-Ming Fu2 Chia-Yang Tsai2 Yu-Wen Huang2 Shawmin Lei2 Marta Karczewicz3 In Suk Chong3 Tomonori Ikai4 | Email: | ichong@qualcomm.com ikai.tomohiro@sharp.co.jp |
| Source: | TOSHIBA Corporation1, MediaTek Inc.2, Qualcomm3 and Sharp4 |  |  |

_____________________________

# Abstract

#### This contribution reports evaluation results of 2-D mergeable syntax technique [JCTVC-F384]. This technique aims to free the restriction on block classification in the HM-4.0 ALF. The coding gains of 0.0 %, 0.1 %, 0.1 % and 0.1 % in HE-AI, RA, LB, and LP were reported. The decoding time ratio was 100 % to 101 % and the encoding time ratio was 99 % to 100 % in HE case. The proposal was cross-checked by Samsung (JCTVC-G649). Harmonization of the other proposal “CE8.a.2: Directional feature calculation on subset of pixels” (JCTVC-G609) is also tested in JCTVC-G649.

# Introduction

In HM-4.0, each 4x4 blocks in a picture is classified into 15 or 16 classes and the classes are merged into 1 to 16 groups, to each of which a set of filter coefficients are associated. The classification is based on two dimensional features: either (x-pos, y-pos) or (activity, direction). However the merge is restricted within 1D scan order. The 2-D mergeable proposal brings not only coding efficiency improvement but also large degree of freedom of encoder algorithm such as top-down splitting, bottom-up merging, some clustering algorithm and one-dimensional algorithm and so on.

In Figure 1, BA class and RA class are shown. Both modes have two-dimensional axes: activity and direction axes in BA class and X and Y axes in RA class. The classes defined in BA and RA can be merged into small groups so that the amount of filter coefficients can be reduced. However, the current syntax only supports one-dimensional merge, that is, the merges of adjoining classes along 1-D scan. For example, two successive classes in 1-D scan order can be merged. But first class and last class in 1-D scan order cannot be merged unless all classes are merged into one group. The proposed syntax resolves the problem.

Figure 1: BA class and RA class description

# Description

The proposed “2-D mergeable syntax” encodes filter index of each class directly. Figure 2 shows an example of indexes. The classes sharing a same index are regarded as a group (or merged class) where corresponding coefficients set is sent.

Figure 2: 2D Flexible class representation

# WD text

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | ae(v) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_region_adaptation_flag | 2 | ae(v) |
| alf_filter_shape_flag | 2 | ae(v) |
| alf_no_filters_minus1 | 2 | ae(v) |
| for (i=1; i < AlfNumRegions; i++) { |  |  |
| if (alf_no_filters_minus1 == 1) { |  |  |
| alf_region_idx[ i ] |  | ae(v) |
| } else if (alf_no_filters_minus1 > 1) { |  |  |
| if( remain_fixed_regions > 0 ) { |  |  |
| prev_alf_region_pred_flag[ i ] |  | ae(v) |
| if( prev_alf_region_pred_flag[ i ] ) |  |  |
| rem_alf_region_pred_idx[ i ] |  | ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| if (AlfNumFilters > 1) |  |  |
| alf_pred_method | 2 | ae(v) |
| alf_min_kstart_minus1 | 2 | ae(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | ae(v) |

alf_region_idx[ i ] specifies the filter index for adaptive loop filter.

prev_alf_region_pred_flag [ i ] and rem_alf_region_pred_idx [ i ] specify the coefficients index CoeffIdx for adaptive loop filter . When prev_alf_region_pred_flag [i ] is equal to 0, the alf_region_idx [ i ] will be inferred from a index of the neighboring classes.

an array CoeffIdx is derived as follows.

Initially,

CoeffIdx[0] is set to be 0.

a value alf_curr_filters is set to be 1.

For each i in the range of 1 to AlfNumRegions -1, the following steps are processed,

If alf_no_filters_minus1 is 0, CoeffIdx[i] is set to be 0.

Otherwise if alf_no_filters_minus1 is 1, CoeffIdx[i] is set to be alf_region_idx[i]

Otherwise if alf_no_filters_minus 1 is larger than 1, CoeffIdx[i] is derived as the following steps

a value of alf_remain_regions is derived as AlfNumRegions - i

a value of alf_remain_filters is derived as AlfNumFilters - alf_curr_filters

a value of remain_fixed_region is derived as alf_remain_regions - alf_remain_filters

if remain_fixed_region is 0, CoeffIdx[i] is inferred to be a value of alf_curr_filters

Otherwise if prev_alf_region_pred_flag[i] is 1, CoeffIdx[i] is inferred to a predicted value mpm_region_idx[i].

Otherwise if rem_alf_region_pred_idx[i] is less than mpm_region_idx[i], CoeffIdx[i] is set to be prev_alf_region_pred_flag[i].

Otherwise CoeffIdx[i] is set to be rem_alf_region_pred_idx[i].

If CoeffIdx[i] is equal to alf_curr_filters, alf_curr_filters is incremented by 1.

mpm_region_idx[i] is derived as follows,

If alf_remain_regions is larger than alf_remain_filters >> 1, mpm_region_idx[i] is derived as alf_curr_filters.

Otherwise if (i % 5) is 0 and alf_region_adaptation_flag is 0, mpm_region_idx[i] is derived as CoeffIdx[i-5]

Otherwise mpm_region_idx[i] is derived as CoeffIdx[i-1]

8.6.3.4 Filtering process for luma sample

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as follows.

recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]

# Simulation results

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.1% | -0.1% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | 0.0% | 0.0% |
| Class D | -0.1% | 0.2% | -0.2% |
| Class E | -0.2% | -0.1% | 0.4% |
| Overall | -0.1% | 0.0% | 0.1% |
|  | -0.1% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | -0.1% | 0.2% |
| Class C | -0.1% | 0.1% | 0.1% |
| Class D | -0.1% | -0.1% | -0.6% |
| Class E | -0.1% | 0.1% | -0.1% |
| Overall | -0.1% | 0.0% | -0.1% |
|  | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusions

This contribution presents 2-D mergeable syntax proposal. This proposal improves coding efficiency and encoder design. The coding gains of 0.0 %, 0.1 %, 0.1 % and 0.1 % in AI, RA, LB, and LP were reported. The decoding time ratio was 100 % to 101 % and the encoding time ratio was 99 % to 100 % in HE case.

# Reference

[1] T. Ikai, Y. Yasugi, T. Yamazaki, I. S. Chong, M. Karczewicz, Marta Karczewicz, “CE8.1:Block based Adaptive Loop Filter with flexible syntax and additional BA mode by Sharp and Qualcomm”, JCTVC-F384, 6th JCT-VC Meeting, Torino, Italy, July 2011.

[2] P. Lai, F. C. A. Fernandes, “CE8 Subtest a: Cross-check of Tool 1 (JCTVC-G316) 2-D mergeable syntax,” JCTVC-G649, 7th JCT-VC Meeting, Geneva, Italy, July 2011.

[3] T. Yamakage, T. Watanabe, T. Chujoh (Toshiba), C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong , T. Ikai, A. Segall, T. Yamamoto , “CE8.a.2: Directional feature calculation on subset of pixels,” JCTVC-G609, 7th JCT-VC Meeting, Geneva, Italy, July 2011.

[4] T. Yamakage, T. Watanabe, T. Chujoh , C. Y. Chen, C. M. Fu, C. Y. Tsai, Y. W. Huang, S. Lei , M. Karczewicz, I. S. Chong , T. Ikai, A. Segall, T. Yamamoto , “Non-CE8: Combination of CE8.a.1 and CE8.a.2,” JCTVC-G649, 7th JCT-VC Meeting, Geneva, Italy, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation, MediaTek Inc., Sharp Corporation, and Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).