| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G320 |
| --- | --- |

| Title: | CE11: Scanning Passes of Residual Data in HE |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A different data coding order is proposed to unify the current scanning process. The goal is to have the same type of scanning for the significance map coding and for the rest of the coefficient level coding. Two variants of the method have been implemented in HM4.0: using passes of 16 coefficients (i.e., all passes like HM4.0 coefficient passes) and using TU-size scan passes (all passes like the HM4.0 significance pass). The encoding/decoding time and BD-rates are very similar to HM4.0, because it consists of just data re-ordering. For the 16 coefficients scan case, the BD-rate for AI-HE, RA-HE and LB-HE configuration is 0.00%, 0.01%, and 0.04%, respectively.

- Introduction

This contribution analyses the scanning processes described in JCTVC-F552 [1] within the scope of CE11.

In HM4.0, the same reverse scan order is used for significance map coding as well as coefficient level coding. The scan starts with the last significant coefficient and proceeds to the DC coefficient in the reverse scanning order. The significance map scan is done for the entire TU (starting from the last significant coefficient). Meanwhile, the level coding is performed in 4 passes on subsets of 16 coefficients in the reverse scan order. The 4 passes are:

- Bin 1 of the absolute value of the coefficient level

- Bin 2 of the absolute value of the coefficient level

- Sign

- Remaining coefficient level

In this contribution, this discrepancy in the scans is fixed, so that the significance map scan is the same as the coefficient level scans. Two variants are provided. In a first case, the significance scan is interleaved with the coefficient level coding for each subset. In the second one, all the scans are done for the entire TU.

- Technical description

In the first case, the concept of subsets is extended to significance map coding. Thus for each subset, significance map coding and 4 passes of coefficient level coding are performed before proceeding to the next subset. Thus for each subset, the following 5 passes are performed:

- Significance

- Bin 1 of the absolute value of the coefficient level

- Bin 2 of the absolute value of the coefficient level

- Sign

- Remaining coefficient level

The interleaving provides some benefits for parallelization. For example, while coefficient level coding is proceeding, the significance map contexts for some of the coefficients from the next subset can be pre-calculated. Similarly, when significance coding pass is proceeding, the context set to be used for coefficient level coding of the current subset can be pre-calculated. Also, the data processing is more localized, and once a subset is processed, the coefficient levels become fully available.

In the second case, the scan passes are done for the entire TU (no sub-sets). There are the 5 scan passes for the TU:

- Significance

- Bin 1 of the absolute value of the coefficient level

- Bin 2 of the absolute value of the coefficient level

- Remaining coefficient level

- Sign

Note that the performance is not very sensitive to the precise subset size chosen. The subset size can be varied from 1 to the TU size with nearly identical performance. The choice of subset size is a trade-off. For instance, larger sizes are better for bypass bins grouping and for speculative coding. Smaller sizes are better for parallelization of contexts and memory requirements.

- Results

The implementation is in HM4.0 and common test conditions are used. Dividing the data in groups of 16 coefficients has a BD-rate for AI-HE, RA-HE, and LB-HE configurations of 0.00%, 0.01%, and 0.04%, respectively. Table 1 shows these results.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.05% | -0.04% | 0.00% | -0.06% | 0.11% |  |  |  |
| Class B | 0.00% | 0.01% | 0.02% | -0.01% | -0.05% | 0.01% | 0.02% | -0.03% | -0.60% |
| Class C | 0.00% | 0.03% | 0.00% | 0.03% | -0.03% | 0.14% | 0.00% | 0.02% | -0.25% |
| Class D | 0.00% | -0.03% | -0.05% | 0.03% | 0.02% | -0.36% | 0.05% | 0.15% | 0.17% |
| Class E | 0.00% | -0.02% | 0.01% |  |  |  | 0.11% | -1.23% | 0.58% |
| Overall | 0.00% | -0.01% | -0.01% | 0.01% | -0.03% | -0.02% | 0.04% | -0.20% | -0.10% |
| Enc Time[%] | 102% |  |  | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 1. BD-rate for residual data partitioning (scans in sets of 16 coefficients)

Doing all the scans for the entire TU has a similar performance, as table 2 shows. The BD-rate for AI-HE, RA-HE, and LB-HE configurations is 0.00%, 0.02%, and 0.05%, respectively.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.02% | 0.02% | 0.05% | -0.02% | -0.23% |  |  |  |
| Class B | 0.00% | 0.02% | -0.01% | -0.02% | 0.03% | 0.07% | 0.03% | -0.02% | -0.34% |
| Class C | 0.00% | 0.00% | -0.01% | 0.00% | 0.01% | 0.05% | 0.00% | -0.30% | -0.03% |
| Class D | 0.00% | -0.03% | 0.02% | 0.05% | 0.17% | 0.03% | 0.05% | 0.47% | -0.23% |
| Class E | 0.01% | -0.04% | 0.02% |  |  |  | 0.18% | -0.09% | -0.09% |
| Overall | 0.00% | -0.01% | 0.01% | 0.02% | 0.05% | -0.01% | 0.05% | 0.02% | -0.19% |
| Enc Time[%] | 103% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  | 106% |  |  |

Table 2. BD-rate for residual data partitioning (scans in sets of TU size)

- Conclusions

This contribution describes unified scan process for the significance map and the coefficient levels. This re-ordering of the data has no impact in performance, while having practical advantages.

- References

- J. Sole, R. Joshi, M. Karczewicz, “Parallel processing of residual data in HE”, JCTVC-F552, 6th JCT-VC Meeting, Torino, IT, July 2011.

# Text for Working Draft

Text for sub-set size 16:

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| numCoeff = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| numCoeff++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| } |  |
| numLastSubset = numCoeff >> 4 |  |
| for( i = numLastSubset − 1; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| for ( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) | ae(v) |
| coeff_abs_level_greater1_flag[ n ] |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) | ae(v) |
| coeff_abs_level_greater2_flag[ n ] |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) | ae(v) |
| coeff_sign_flag[ n ] |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) | ae(v) |
| coeff_abs_level_minus3[ n ] |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

Table for scans for the entire TU:

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| numCoeff = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| numCoeff++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| } |  |
| for( n = numCoeff − 1; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| for( n = numCoeff; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = numCoeff; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = numCoeff; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( coeff_abs_level_greater2_flag[ n ] ) { |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| } |  |
| for( n = numCoeff; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n ] = 0 |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).