| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G321 |
| --- | --- |

| Title: | CE11: Removal of the parsing dependency of residual coding on intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Yunfei Zheng, Wei-Jung Chien, Rajan Joshi, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This proposal modifies the residual coding in high efficiency to remove the dependency of the residual data parsing on the intra mode. Two modifications are introduced to attain this goal. First, last significant coefficient coding assumes that the scan is always diagonal scan. Second, the significance map context for 4×4 and 8×8 blocks are modified to depend on the scan position, not the block-based position as in HM4.0. The BD-rate of both parts combined for AI-HE, RA-HE, and LB-HE is 0.23%, 0.15%, and 0.14%, respectively.

- Introduction

This contribution analyses the parsing issues removal described in JCTVC-F550 [1] within the scope of CE11. In HM4.0, residual coding in CABAC highly depends on the 2-D coefficient location in a block (X and Y coordinates) rather than the 1-D coefficient index relative to the scanning order. The mappings between the 2-D location and the 1-D index are defined by the scanning patterns. In the current design, mode dependent coefficient scan indicates the scan pattern to apply depending on the intra prediction mode. The parsing process for the residual coding cannot be performed without deriving the intra prediction mode, since the mapping between the 2-D coefficient location and the 1-D coefficient index is not known.

There have been efforts [2, 3] to introduce modifications to the intra prediction mode coding in order to remove the extra memory storage for the neighboring partition information during the parsing process. However, the intra prediction mode still needs to be available in order to start the parsing of the residual data. In this contribution, the residual coding is modified to be scan based, so that the parsing can be done without the knowledge of the intra prediction mode.

- Technical description

The residual coding in HE is modified so that there is no need to know the actual intra mode in order to parse the residual data. Two modifications are done to attain the goal:

- Coding of the position of the last significant coefficient based on diagonal scan

- Significance map coding based on the scan position for 4×4 and 8×8 blocks

The changes are done on HM4.0. Simulation results are presented for the high efficiency common test conditions in tables 1 (for case A) and 2 (for A+B). Detailed results, including LP configurations, are provided in the accompanying excel files.

## Last significant coefficient coding

The X and Y coordinates of the last significant coefficient depend on the scanning order. This dependency is removed by assuming that the employed scan is always the fixed diagonal scan. Coordinates X and Y are derived from the scan position of the last coefficient using always the diagonal pattern. This change makes the last coefficient coding independent of the intra mode, while keeping the BD-rate performance (table 1).

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.08% | -0.10% | -0.09% | 0.07% | -0.17% | 0.04% |  |  |  |
| Class B | 0.05% | -0.06% | -0.05% | 0.02% | -0.04% | -0.08% | 0.02% | -0.12% | -0.43% |
| Class C | 0.03% | 0.02% | -0.02% | 0.06% | 0.03% | 0.11% | 0.07% | 0.07% | -0.09% |
| Class D | 0.05% | -0.06% | -0.04% | 0.03% | 0.57% | -0.06% | -0.01% | 0.67% | 0.20% |
| Class E | 0.08% | 0.03% | -0.15% |  |  |  | -0.07% | -0.56% | 0.18% |
| Overall | 0.06% | -0.04% | -0.06% | 0.05% | 0.09% | 0.00% | 0.01% | 0.04% | -0.07% |
| Enc T[%] | 99% |  |  | 104% |  |  | 103% |  |  |
| Dec T[%] | 96% |  |  | 101% |  |  | 99% |  |  |

Table 1. BD-rate for coefficient level coding and last coefficient coding (A)

## Significance map coding

In HM4.0, the context derivation of the significance map for small blocks (4×4 and 8×8) depends on the block-based position of the coded coefficient. This is modified making the context dependant on the scan position. The context index for 4x4 blocks is derived by

ctxIndex[uiScanPos] = uiScanPos

where uiScanPos is the position of the coefficient along the scan line. The context index for 8×8 blocks is derived by a LUT by   ctxIndex[uiScanPos] = auiScanPosToCtx[uiScanPos].

Table 2 shows the overall performance when combining both changes (A+B).

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.30% | 0.04% | 0.20% | 0.12% | 0.09% | 0.62% |  |  |  |
| Class B | 0.21% | 0.07% | 0.11% | 0.14% | 0.07% | 0.06% | 0.07% | 0.23% | -0.06% |
| Class C | 0.22% | 0.21% | 0.21% | 0.16% | 0.30% | 0.39% | 0.19% | 0.52% | 0.23% |
| Class D | 0.23% | 0.18% | 0.29% | 0.16% | 0.23% | -0.10% | 0.16% | 0.47% | 0.41% |
| Class E | 0.23% | -0.04% | -0.20% |  |  |  | 0.14% | -0.25% | 0.68% |
| Overall | 0.23% | 0.10% | 0.14% | 0.15% | 0.16% | 0.23% | 0.14% | 0.27% | 0.27% |
| Enc T[%] | 102% |  |  | 101% |  |  | 101% |  |  |
| Dec T[%] | 101% |  |  | 102% |  |  | 104% |  |  |

Table 2. BD-rate for last coefficient and significance map coding (A+B)

- Conclusions

This proposal introduces changes in the residual coding in high efficiency to remove the dependency of the residual data parsing on the intra mode. Proposed modifications solve the issue at a little performance cost: 0.23%, 0.15%, and 0.14% for AI-HE, RA-HE, and LB-HE, respectively.

- References

- J. Sole, Y. Zheng, W.-J. Chien, R. Joshi, X. Wang, M. Karczewicz, “Removal of the parsing dependency of residual coding on intra mode”, JCTVC-F550, 6th JCT-VC Meeting, Torino, IT, July 2011.

- W.-J. Chien, T. Lee, J. Chen, “Parsing friendly intra mode coding”, JCTVC-F459, 6th JCT-VC Meeting, Torino, Italy, July 2011.

- V. Seregin, T. Lee, J. Chen, “Intra mode parsing without access neighbouring information”, JCTVC-F378, 6th JCT-VC Meeting, Torino, Italy, July 2011.

- Text for Working Draft

Residual coding syntax table

The syntax for coding the residual is found below. The input to the process derive_XY_from_ScanOrder0 is xI and yI coordinates assuming diagonal scan order and the scanIdx of the current scan order. The output is the xO and yO coordinates for the scanIdx scan. The process is defined as follows:

- If scanIdx = 0, xO = xI, yO = yI

- Otherwise, set d = xI + yI

  - If d < blockWidth, set n = ( d * ( d + 1 ) ) >> 1 + xI

  - Otherwise, set

    - di = ( (blockWidth – 1) << 1 ) – d

    - n = blockWidth * blockHeight – blockWidth – 1 – ( di * ( di + 1 ) ) >> 1 – xI

- xO = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n][ 0 ]

- yO = ScanOrder[  log2TrafoSize − 2 ][ scanIdx  ][ n][ 1 ]

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| (last_significant_coeff_x, last_significant_coeff_y) = derive_XY_from_ScanOrder0( last_significant_coeff_x, last_significant_coeff_y, scanIdx ) |  |
| numCoeff = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| numCoeff++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| } |  |
| for( n = numCoeff − 1; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| numLastSubset = numCoeff >> 4 |  |
| for( i = numLastSubset − 1; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

###### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block size log2TrafoSize, and the coefficient position along scan line ScanPos.

Output of this process is ctxIdxInc.

Table  specifies the auiScanPosToCtx associated with context index of log2TrafoSize is equal to 3.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than 3, sigCtx is derived as follows.

sigCtx = ScanPos

- Otherwise if log2TrafoSize is equal to 3, sigCtx is derived as follows.

sigCtx  =  auiScanPosToCtx [ScanPos]		(9-1)

- Otherwise if xC is less than 2 and yC is less than 2, sigCtx is derived as follows.

sigCtx  =  ( yC  <<  1 )  +  xC		(9-2)

- Otherwise if yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  4  + significant_coeff_flag[ xC-1 ][ yC ]  +  significant_coeff_flag[ xC-2 ][ yC ]	(9-3)

- Otherwise if xC is equal to 0, sigCtx is derived as follows.

sigCtx  =  4  + significant_coeff_flag[ xC ][ yC-1 ]  +  significant_coeff_flag[ xC ][ yC-2 ]	(9-4)

- Otherwise (xC is greater than 1 and yC is greater than 1) , the following applies.

- The variable sigCtx is initialized using previously decoded bins of the syntax element significant_coeff_flag as follows.

sigCtx  =  	significant_coeff_flag[ xC-1 ][ yC ]  +  significant_coeff_flag[ xC ][ yC-1 ]  +  
		significant_coeff_flag[ xC-1 ][ yC-1 ]	(9-5)

- When xC is greater than 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC-2 ][ yC ]	(9-6)

- When yC is greater than 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC-2 ]	(9-7)

- The variable sigCtx is dervied as follows.

sigCtx  =  10  +  min( 4, sigCtx )		(9-8)

The context index increment ctxIdxInc is derived using the color component index cIdx, the transform block size log2TrafoSize and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  ( ( 5  −  log2TrafoSize )  *  16 )  +  sigCtx	(9-9)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  64  +  ( ( 4  −  log2TrafoSize )  *  16 )  +  sigCtx	(9-10)

| scanPos | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdx | 0 | 0 | 1 | 1 | 2 | 2 | 3 | 3 | 1 | 1 | 4 | 4 | 4 | 3 | 3 | 3 |
| scanPos | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| ctxIdx | 5 | 5 | 4 | 4 | 6 | 6 | 6 | 7 | 5 | 5 | 8 | 8 | 8 | 8 | 8 | 7 |
| scanPos | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| ctxIdx | 9 | 9 | 10 | 10 | 10 | 11 | 11 | 7 | 12 | 12 | 13 | 13 | 13 | 11 | 11 | 11 |
| scanPos | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| ctxIdx | 14 | 14 | 14 | 13 | 13 | 13 | 15 | 15 | 14 | 14 | 14 | 15 | 15 | 15 | 15 | 15 |

Table 3 Context index of log2TrafoSize = 3, auiScanPosToCtx

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).