| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G322 |
| --- | --- |

| Title: | CE6.c: Harmonization of HE residual coding with non-square block transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

CABAC entropy coding process is not able to handle directly non-square transform blocks, thus requiring mappings from non-square to square blocks and vice versa. This contribution removes this constraint of the entropy coder. SDIP and NSQT non-square transform blocks are coded directly without any mapping. The BD-rate is 0.08%, 0.05% and 0.08% for AI, RA and LB configurations, respectively.

- Introduction

This proposal analyses the harmonization of CABAC entropy coding with non-square transform blocks of [1] within the scope of CE6.

NSQT and SDIP non-square PUs are transformed by the corresponding non-square transforms of the same size. The following transforms are currently applied: 32×8, 8×32, 32×2, 2×32, 16×4, 4×16 and 8×2, 2×8 for SDIP. Entropy coding is applied to the resulting coefficients. However, the HE entropy coding has not been modified to deal with non-square transform coefficients. Instead, a mapping is applied in order to re-use the existing method for non-square blocks. The mapping is defined from a non-square block to a square block with the same number of pixels. The regular entropy encoding and decoding are applied to the square block. Then, prior to the inverse transform, the inverse mapping is applied for from the square shape to the original non-square shape.

This method re-uses existing entropy coding, but fails to exploit the underlying correlation of the coefficients in a non-square block because the mapping ‘breaks’ some of the neighboring relationship and correlation among coefficients. Therefore, the method can be improved by harmonizing the current entropy coding method to code non-square blocks. The mapping between non-square blocks and square blocks is not used in this proposal also because it introduces intermediate steps that add complexity to the coder.

- Technical description

Modifications are in the following parts: scans, coding of the last significant coefficient, and significance map.

## Scans

Similar to the square blocks, three scans for the non-square blocks are used: horizontal, vertical and diagonal scan. The scan depends on the intra mode. A table maps the intra mode to one of the three scans to apply. The blocks that use the three scans are the smaller ones: blocks with 64 coefficients or less. Larger blocks use only one scan. The diagonal scan of HEVC is used and it starts from the long side of the block.

## Last Significant Coefficient Position

For non-square blocks, in the signaling of the last significant coefficient, the context model depends on the length of the side of the coordinate being coded. In other words, the horizontal coordinate (X component of the last coefficient) has a context depending on the width of the block (and the bin of X being coded). Equivalently, the vertical coordinate (Y component) has a context set assigned depending on the height of the block.

The contexts models are the same as the square case except for the following case. For blocks of a side of length 2 in SDIP, an additional CABAC context is used to indicate the last position, since this 2-point transform is not part of the square transforms.

## Significance Map

For the significance map of non-square blocks, blocks with the same number of coefficients share the same contexts. In this way, no additional contexts are introduced. In order to enhance performance, the context derivation and sharing is not done directly, but on the transposed coordinates and values of the higher of the non-square shapes. For example, the 8×2 is coded directly, but the 2×8 is coded as if it was ‘transposed’: coordinates X and Y and the width and height are swapped prior to coding. This exploits the fact that the statistics of the 8×2 block are similar to the statistics of the 2×8 block transposed. Same principle applies to the other block sizes.

Blocks with 64 coefficients or less use contexts for the significance map depending on the position (like in HEVC the 4×4 and 8×8 blocks). The other blocks (32×8/8×32) use the neighbor based derivation equivalent to the larger blocks in HEVC.

- Results

This method is implemented on the top HM4.0+SDIP of CE6. Simulation results are presented for the high efficiency common test conditions. Table 1 shows the results for this method. The BD-rate is 0.08%, 0.05% and 0.08% for AI, RA and LB configurations, respectively. Detailed results including LP configuration and class F are provided in the accompanying excel file.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.15% | -0.29% | -0.51% | -0.08% | -0.31% | -0.42% |  |  |  |
| Class B | -0.15% | -0.15% | -0.26% | -0.10% | -0.04% | -0.21% | -0.11% | -0.19% | -0.03% |
| Class C | 0.01% | -0.06% | -0.09% | -0.01% | -0.11% | -0.10% | -0.04% | 0.00% | -0.08% |
| Class D | 0.02% | 0.02% | -0.04% | 0.00% | -0.25% | -0.34% | -0.03% | -0.15% | -0.56% |
| Class E | -0.14% | -0.54% | -0.36% |  |  |  | -0.14% | -0.92% | -0.96% |
| Overall | -0.08% | -0.19% | -0.25% | -0.05% | -0.17% | -0.27% | -0.08% | -0.27% | -0.35% |
| Enc Time[%] | 98% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |

Table 1. BD-rates

The same technique can be applied on the top of HM4.0 for the NSQT blocks case. Common conditions are used. Table 2 shows the results for this case. The BD-rate is 0.01% and 0.04% for RA and LB configurations, respectively.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | -0.02% | -0.22% | -0.28% |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | -0.04% | -0.01% | -0.04% | -0.07% | -0.11% | -0.17% |
| Class C | 0.00% | 0.00% | 0.00% | -0.02% | -0.07% | 0.01% | -0.03% | -0.09% | -0.11% |
| Class D | 0.00% | 0.00% | 0.00% | 0.03% | -0.02% | 0.10% | 0.00% | 0.13% | -0.20% |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  | -0.06% | 0.38% | 0.52% |
| Overall | 0.00% | 0.00% | 0.00% | -0.01% | -0.08% | -0.05% | -0.04% | 0.05% | -0.03% |
| Enc Time[%] | 106% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  | 100% |  |  |

- Conclusions

A harmonization of CABAC entropy coding with non-square blocks is proposed. Mappings between square and non-square blocks are removed. Coding is applied directly to the non-square block. This simplifies the coding process, while reporting small BD-rate gains.

- References

- G. Van der Auwera, J. Sole, Y. Zheng, X. Wang, I. S. Chong, R. Joshi, M. Karczewicz, “CE6.b: SDIP Harmonization with Deblocking, MDIS and HE Residual Coding”, JCTVC-F556, 6th JCT-VC Meeting, Torino, IT, July 2011.

# Text for Working Draft

###### 9.3.3.1.1.3 Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x and last_significant_coeff_y

Inputs to this process are the binIdx, the color component index cIdx and the transform block size log2TrafoSize of the axis processed, for which Size being either Width or Height.

Output of this process is ctxIdxInc.

Table 9-1 – Specification of lastCtx[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx | 0 | 1 | 2 | 3 | 3 | 4 | 4 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 7 |
| i | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |  |
| lastCtx | 7 | 7 | 7 | 8 | 8 | 8 | 8 | 9 | 9 | 9 | 9 | 10 | 10 | 10 | 10 |  |

The vector lastCtx[ i ] is specified in Table 9-1. For the derivation of ctxIdxInc, the following applies.

–	If log2TrafoSize is equal to 1, ctxIdxInc is derived as follows.

ctxIdxInc =  20		(9-51)

–	If log2TrafoSize is equal to 2, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ binIdx ]		(9-1)

–	Otherwise (log2TrafoSize is greater than 2), ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( 1 << ( log2TrafoSize – 1 ) ) + binIdx ]	(9-2)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  20		(9-3)

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

###### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size (blkWidth, blkHeight).

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than or equal to 3 and blkWidth is equal to blkHeight, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )	(9-4)

- Otherwise, if blkWidth is equal to 2 or blkHeight is equal 2

  - If blkWidth < blkHeight

    - Temp = xC

    - xC = yC

    - yC = xC

offset = xC <  4 ? 0 : 4

sigCtx = offset + xC + (yC <<2)

- Otherwise if blkWidth*blkHeight is equal to 64

  - If blkWidth < blkHeight

- Temp = xC

- xC = yC

- yC = xC

xC = xC – ( ( xC >> 3 ) << 3 )

yC = yC + ( ( xC >> 3 ) << 2 )

sigCtx = 15 + ( ( yC >> 1 ) << 2 ) + ( xC >> 1 )

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC	(9-56)

- Otherwise if xC + yC is less than 5, sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC ][ yC + 1 ]  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-67)
sigCtx  =  34  +  Min( 4, temp )

- Otherwise (xC  +  yC is greater than 4), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  39		(9-78)

- When xC is less than ( blkWidth – 1 ), the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-59)

- When xC is less than ( blkWidth – 1 ) and yC is less than ( blkHeight – 1 ), the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-80)

- When xC is less than ( blkWidth – 2 ), the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-91)

- When yC is less than ( blkHeight – 1 ), the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-102)

- When yC is less than ( blkHeight – 2 ) and sigCtx is less than 43, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-113)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-124)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  44  +  sigCtx		(9-135)

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

### 8.5.2  Inverse scanning process for transform coefficients

Inputs to this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit.

–	a list of (nW)x(nH) values transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ].

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is a variable c containing a two-dimensional array of (nW)x(nH) values.

The variable nS is set equal to Sqrt(nW*nH).

The variable scanIdx, indicating wich scan is used to scan the transform coefficients (zigzag, horizontal or vertical), is derived as follows.

- If PredMode is equal to MODE_INTRA, the following applies.

  - If cIdx is equal to 0, scanIdx is set to ScanType[ Log2( nS >> 2 ) ][ IntraPredMode ].

  - Otherwise (cIdx is equal to 1 or 2), scanIdx is set to ScanType[ Log2( nS >> 2 ) ][ IntraPredModeC ]

- Otherwise (PredMode is not equal to MODE_INTRA), scanIdx is set to 0.

When entropy_coding_mode_flag is equal to 1 (CABAC) and scanIdx is equal to 0 (zigzag), scanIdx is set to 3 (diagonal).

The variable c[ i ][ j ] which is located in the ( i, j ) position in the array c is derived as follows.

- The transform coefficient level at scanning position n is mapped to the position ( i, j ) in the array c using the scan specified by scanIdx as follows.

i = ScanOrder[ scanIdx ] [ Log2( nW)-2 ][ Log2(nH )-2][ n ][ 0 ]			
j = ScanOrder[ scanIdx ] [Log2( nW)-2 ][ Log2(nH )-2 ][ n ][ 1 ]			
c[ i ][ j ] = transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ][ n ],			
with n = 0.. ( nS * nS ) – 1	(8-14)

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).