| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G323 |
| --- | --- |

| Title: | Non-CE11: Diagonal sub-block scan for HE residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A scanning pattern for residual coding in HE is proposed. TU are divided in square sub-blocks. The scan consists in a diagonal scan within sub-blocks and a diagonal scan of the sub-blocks. This scanning pattern is allegedly beneficial for hardware and SIMD implementations. Results for three sub-blocks sizes (4×4, 8×8 and 16×16) are provided in HM4.0. A parallel friendly modification for the neighborhood-based context derivation for the significance map is further incorporated. BD-rate results for all six cases are provided, which remain within 0.1% of the anchor in common test conditions.

- Introduction

During the last meeting, in the context of CE11 on scans, a discussion arose regarding the drawbacks of the zigzag/diagonal scans for large blocks (16×16 and 32×32). Contribution [1] estimated that the hardware complexity of these scans approximately doubles that of the scans based on small sub-blocks. Also, SIMD implementations become more difficult with the large diagonal scans.

- Technical description

This contribution proposes to tackle these issues by using diagonal scans in sub-blocks. Each sub-block is scanned in the diagonal scan of HEVC and the scan between sub-blocks is also the diagonal pattern. This is applied to blocks of sizes 16×16 and 32×32. Figure 1 depicts an example of this scan (for clarity, on an  8×8 block).

Figure 1. Diagonal sub-block scan (sub-block size 4×4)

Three different sub-block sizes are considered in this contribution: 4×4, 8×8 and 16×16. Results are provided for these three cases.

Given these scans and the current context neighborhood for the significance map coding, parallelization of the context derivation might not be adequate. In order to handle this, a modified parallel friendly neighborhood is used for the positions that would otherwise use the previous coefficient value to derive the context. These coefficients are the ones at the top-left corner of a sub-block and the ones right above the right-bottom corner of a sub-block. Figure 2 shows the modified neighborhood.

Figure 2. Parallel friendly neighborhood for the significance map context derivation for the first coefficient in a sub-block

- Results

This method is implemented in HM4.0. Simulation results are presented for the high efficiency common test conditions. Table 1 shows the results for this method for sub-blocks of size 4×4, table 2 for size 8×8 and table 3 for sub-blocks of sizes 16×16.

Then, the parallel friendly neighborhood is introduced. Results are shown in tables 4, 5 and 6 for sub-blocks of sizes  4×4, 8×8 and 16×16, respectively.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.07% | -0.31% | -0.11% | 0.09% | 0.29% | 0.47% |  |  |  |
| Class B | 0.03% | 0.02% | 0.04% | 0.04% | -0.03% | 0.07% | 0.06% | -0.05% | -0.11% |
| Class C | -0.02% | 0.00% | 0.00% | -0.04% | -0.11% | 0.09% | 0.01% | -0.05% | 0.05% |
| Class D | -0.01% | -0.04% | -0.07% | 0.00% | 0.02% | -0.32% | 0.00% | 0.07% | -0.32% |
| Class E | -0.06% | -0.15% | -0.17% |  |  |  | 0.07% | 0.04% | 0.40% |
| Overall | -0.02% | -0.09% | -0.05% | 0.03% | 0.04% | 0.08% | 0.03% | 0.00% | -0.03% |
| Enc Time[%] | 101% |  |  | 99% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |

Table 1. BD-rates for scan sub-blocks of size 4×4

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.28% | 0.05% | 0.28% | 0.27% | 0.24% | 0.57% |  |  |  |
| Class B | 0.11% | 0.17% | 0.19% | 0.02% | 0.11% | 0.00% | 0.02% | 0.03% | -0.03% |
| Class C | -0.01% | 0.12% | 0.03% | 0.03% | -0.07% | 0.02% | 0.00% | -0.06% | 0.02% |
| Class D | 0.02% | 0.00% | -0.04% | 0.03% | 0.11% | -0.14% | -0.02% | 0.45% | 0.11% |
| Class E | 0.21% | 0.10% | 0.17% |  |  |  | 0.14% | -1.14% | -0.18% |
| Overall | 0.12% | 0.09% | 0.13% | 0.08% | 0.10% | 0.11% | 0.03% | -0.11% | -0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 96% |  |  |

Table 2. BD-rates for scan sub-blocks of size 8×8

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.20% | -0.24% | -0.10% | 0.27% | -0.41% | -0.88% |  |  |  |
| Class B | 0.02% | 0.08% | 0.08% | 0.00% | 0.02% | 0.05% | 0.03% | 0.00% | -0.03% |
| Class C | -0.02% | 0.02% | 0.01% | -0.03% | -0.11% | -0.07% | -0.01% | -0.09% | 0.10% |
| Class D | 0.00% | 0.03% | 0.01% | 0.02% | 0.32% | 0.20% | -0.03% | -0.08% | 0.16% |
| Class E | 0.11% | 0.27% | 0.22% |  |  |  | 0.22% | -0.14% | -0.11% |
| Overall | 0.06% | 0.02% | 0.04% | 0.06% | -0.04% | -0.16% | 0.04% | -0.07% | 0.03% |
| Enc Time[%] | 101% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  | 95% |  |  |

Table 3. BD-rates for scan sub-blocks of size 16×16

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.01% | -0.24% | -0.01% | 0.13% | 0.14% | 0.05% |  |  |  |
| Class B | 0.08% | 0.11% | 0.11% | 0.07% | 0.00% | 0.12% | 0.06% | 0.11% | -0.32% |
| Class C | 0.01% | 0.01% | 0.03% | -0.01% | -0.02% | -0.02% | 0.06% | 0.11% | 0.01% |
| Class D | 0.00% | 0.02% | 0.02% | 0.06% | -0.04% | -0.14% | -0.03% | -0.40% | -0.23% |
| Class E | 0.00% | -0.09% | -0.06% |  |  |  | 0.13% | -0.05% | 0.61% |
| Overall | 0.02% | -0.03% | 0.03% | 0.06% | 0.02% | 0.01% | 0.05% | -0.05% | -0.04% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 93% |  |  | 94% |  |  |

Table 4. BD-rates for scan sub-blocks of size 4×4 with parallel context

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.30% | 0.08% | 0.27% | 0.33% | 0.66% | 0.86% |  |  |  |
| Class B | 0.12% | 0.18% | 0.23% | 0.03% | 0.16% | 0.00% | 0.04% | 0.05% | -0.26% |
| Class C | 0.00% | 0.10% | 0.02% | 0.00% | -0.01% | 0.07% | 0.01% | 0.02% | -0.22% |
| Class D | 0.02% | 0.02% | -0.02% | 0.05% | 0.09% | 0.04% | -0.04% | 0.03% | -0.19% |
| Class E | 0.22% | 0.15% | 0.21% |  |  |  | 0.10% | -1.43% | -0.28% |
| Overall | 0.13% | 0.11% | 0.14% | 0.10% | 0.22% | 0.23% | 0.03% | -0.24% | -0.24% |
| Enc Time[%] | 102% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  | 104% |  |  |

Table 5. BD-rates for scan sub-blocks of size 8×8 with parallel context

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.20% | -0.20% | -0.13% | 0.29% | -0.33% | -0.54% |  |  |  |
| Class B | 0.02% | 0.06% | 0.08% | 0.00% | 0.05% | -0.10% | 0.02% | 0.22% | -0.26% |
| Class C | -0.02% | 0.02% | 0.01% | -0.04% | 0.01% | -0.11% | 0.00% | -0.09% | -0.09% |
| Class D | 0.01% | 0.02% | -0.01% | 0.01% | 0.15% | 0.02% | -0.02% | 0.18% | -0.05% |
| Class E | 0.11% | 0.30% | 0.24% |  |  |  | 0.11% | -0.14% | -0.13% |
| Overall | 0.06% | 0.03% | 0.03% | 0.06% | -0.03% | -0.18% | 0.02% | 0.06% | -0.14% |
| Enc Time[%] | 101% |  |  | 102% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 95% |  |  | 96% |  |  |

Table 6. BD-rates for scan sub-blocks of size 16×16 with parallel context

# Conclusions

This proposal describes sub-block based scans that help hardware and SIMD implementations. Results show that these scans can be incorporated in HEVC at nothing or little performance loss.

- References

- C. Auyeung and T. Suzuki, “CE11: Hardware complexity of large zig-zag scan for level-coding of transform coefficients”, JCTVC-F597, 6th JCT-VC Meeting, Torino, IT, July 2011.

- J. Sole, R. Joshi, M. Karczewicz, “CE11: Scanning Passes of Residual Data in HE”, JCTVC-G320, 7th JCT-VC Meeting, Geneva, CH, November 2011.

# Integration with unified scan process

The sub-block scan process of size 4×4 has been integrated with the unified scan passes of [2]. The scan passes are then all the same and within each sub-block as next table shows:

| Scan Pass | Syntax element | CABAC mode | HM4.0 Scan | Unified |
| --- | --- | --- | --- | --- |
| 1 | significant_coeff_flag | Regular | Entire TU | 4×4 sub-block |
| 2 | coeff_abs_level_greater1_flag | Regular | 16 coefficient sets | 4×4 sub-block |
| 3 | coeff _abs_level_greater2_flag | Regular | 16 coefficient sets | 4×4 sub-block |
| 4 | coeff_sign_flag | Bypass | 16 coefficient sets | 4×4 sub-block |
| 5 | coeff_abs_level_minus3 | Bypass | 16 coefficient sets | 4×4 sub-block |

In this case, the 5 scan passes are interleaved within a sub-block. A sub-block is fully processed and then, it proceeds to the next sub-block. Table 7 shows the BD-rate for the combined solution.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.23% | 0.02% | 0.13% | 0.19% | 0.64% |  |  |  |
| Class B | 0.08% | 0.09% | 0.11% | 0.08% | 0.13% | 0.06% | 0.11% | 0.04% | -0.13% |
| Class C | 0.01% | 0.01% | 0.02% | 0.02% | -0.13% | -0.10% | 0.06% | -0.10% | -0.09% |
| Class D | 0.01% | 0.02% | -0.03% | 0.07% | 0.16% | -0.13% | -0.02% | 0.58% | 0.50% |
| Class E | 0.00% | -0.08% | -0.05% |  |  |  | 0.17% | -1.32% | 0.51% |
| Overall | 0.02% | -0.03% | 0.02% | 0.08% | 0.09% | 0.11% | 0.08% | -0.11% | 0.16% |
| Enc Time[%] | 103% |  |  | 100% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 100% |  |  |

Table 7. BD-rates for scan sub-blocks of size 4×4 with unified passes

# Text for Working Draft

### Up-right diagonal scanning array initialisation process

Input to this process is a block size width blkWidth and a block height blkHeight.

Output of this process is the array DiagScan[ pos ][ comp ]. The array index pos specify the scan position ranging from 0 to ( blkWidthSize * blkHeightSize ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. Depending on the values of blkWidth and blkHeight, the array DiagScan is derived as follows. subblkSize is the size of the sub-block scans.

- If blkWidth* blkHeightSize is less than or equal to 64, the following applies.

DiagScan[ 0 ][ 0 ]  =  0
DiagScan [ 0 ][ 1 ]  =  0
i  =  1
x  =  0
y  =  1
stopLoop  =  ( blkWidth  = = 1 ) ? true : false
while( !stopLoop ) {
	while( y  >=  0 ) {
		if (x  >=  0  &&  x  <  blkWidth  &&  y  >=  0  &&  y  <  blkHeight ) {
			DiagScan [ i ][ 0 ]  =  x
			DiagScan [ i ][ 1 ]  =  y
			i++
		}
		y− −
		x++
	}
	y  =  x
	x  =  0
	if ( i  >=  blkWidth  *  blkHeight ) 
		stopLoop  =  true
}

- Otherwise, the following applies.

x_off = 0

y_off = 0

i_off = 0

stopLoopSubblocks = false

while( ! stopLoopSubblocks) {

i  =  0
			x  =  0
			y  =  0
			stopLoop  =  false
			while( !stopLoop ) {
				while( y  >=  0 ) {
					if (x  >=  0  &&  x  <  subblkSize  &&  y  >=  0  &&  y  <  subblkSize) {
						DiagScan [ i + i_off ][ 0 ]  =  x + x_off
						DiagScan [ i + i_off ][ 1 ]  =  y + y_off
						i++
					}
					y− −
					x++
				}
				y  =  x
				x  =  0
				if ( i  >=  subblkSize *  subblkSize ) 
					stopLoop  =  true
			}

y_off  = y_off  − subblkSize  
			x_off = x_off  + subblkSize

i_off = i_off + subblkSize  * subblkSize

if ( !(x_off  >=  0  &&  x_off   <  blkWidth  &&  y_off  >=  0  &&  y_off  <  blkHeight )) {
	y_off = x_off

x_off = 0	
}

if ( i_off  >=  blkWidth  *  blkHeight ) 
      stopLoopSubblocks =  true

}

###### 9.3.3.1.1.4  Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size log2TrafoSize.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than or equal to 3, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )	(9-1)

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC		(9-2)

- Otherwise if xC + yC is less than 5, sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-3)
- If (xC, yC) is not the top-left corner of a sub-block or (xC, yC+1) is not the bottom-right corner of a sub-block, sigCtx is derived as follows.

sigCtx  =  34  +  temp

- Otherwise,

sigCtx  =  34  +  Min(4, temp + significant_coeff_flag[ xC ][ yC + 1 ])

- Otherwise (xC  +  yC is greater than 4), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  39		(9-4)

- When xC is less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-5)

- When xC and yC are less than ( 1 << log2TrafoSize ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-6)

- When xC is less than ( 1 << log2TrafoSize ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-7)

- When yC is less than ( 1 << log2TrafoSize ) – 1 and (xC, yC) is not the top-left corner of a sub-block or (xC, yC+1) is not the bottom-right corner of a sub-block, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-8)

- When yC is less than ( 1 << log2TrafoSize ) − 2 and sigCtx is less than 43, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-9)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-10)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  44  +  sigCtx		(9-11)

Integration with unified scan process

Regarding the integration with JCTVC-G320 with all the scan passes within sub-blocks of 4×4, there is no interaction between both proposals. The additional text of JCTVC-320 is added below for the sake of completeness.

### 7.3.10 Residual coding CABAC syntax

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| numCoeff = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| numCoeff++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| } |  |
| numLastSubset = numCoeff >> 4 |  |
| for( i = numLastSubset − 1; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| for ( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) | ae(v) |
| coeff_abs_level_greater1_flag[ n ] |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) | ae(v) |
| coeff_abs_level_greater2_flag[ n ] |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) | ae(v) |
| coeff_sign_flag[ n ] |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) | ae(v) |
| coeff_abs_level_minus3[ n ] |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).