| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G324 |
| --- | --- |

| Title: | Modified LPS range and state transition tables for BAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The proposal modifies the binary arithmetic coder to allow a slower probability adaptation process and the coding of more skewed binary distributions than HM4.0. The entries of the range LPS table are changed and the number of probability states increased from 64 to 128. BD-rate for AI-HE, RA-HE and LB-HE configuration is -0.34%, -0.26%, and -0.05%, respectively. When the range LPS table size is divided by 2, the BD-rates are -0.26%, -0.21%, and 0.07% for AI-HE, RA-HE and LB-HE, respectively. A third variant using bit-shift operations that reduces the table size by 25% provides BD-rates of -0.36%, -0.28%, and -0.13%.

- Introduction

The binary arithmetic coding (BAC) in HEVC is coming from H.264/AVC [1]. The tables in BAC were derived taking into consideration low resolution video content. Current HD and beyond content has different statistics than the older CIF sequences. In general, this higher resolution content is more stationary, which means that the adaptation process in BAC doesn’t require its high adaptation speed. This stationarity also implies the existence of larger areas with similar statistics and so, long occurrences of the same symbols. HEVC codec uses large coding and transform units that tend to make a high repetitive use of contexts with very skewed distributions. Therefore, BAC needs to handle more extreme probabilities (closer to 0 and 1) than currently possible.

This contribution modifies current BAC tables (range LPS and next state tables) to account for these characteristics of the content and codec. The modifications address two aspects:

- Slower adaptation process of the state machine

- Handling of more skewed binary distributions

Three variants of the method are provided. First one is keeping the current size of the range LPS table. The second  one has half the current size. Finally, a third variant reduces the range LPS table by 25% and it indicates the bit-shift amount to apply to the current range instead of directly indicating the new range value.

- BAC Background

In the HEVC CABAC, each context has a state assigned. Each state is associated to the probability (pσ) of the Least Probable Symbol (LPS) and the value (0/1) of the Most Probable Symbol (MPS). There are 64 possible LPS probabilities (denoted by the state σ from 0 to 63). The probability for each state is derived as follows:

p σ = a p σ -1

where p0=1/2  and a=0.9493. More precisely, a=(0.01875/0.5)1/63.

CABAC adapts to signal statistics by updating the states through a state transition process. For a given probability state, the update depends on the state index and the value of the coded symbol (LPS or MPS). As a result of the updating process, a new probability state is derived, which consists of a potentially modified LPS probability and, if necessary, a modified MPS value. In HEVC, the NextStateLPS table defines the state transition in case of a LPS occurring. The table has 64 entries, one for each state σ. In case of MPS, the transitions can be obtained by updating the state index σ  to the value min( σ+1, 62 ).

The arithmetic coder computes a new range given the current range and the LPS probability. The new BAC range is derived with the range LPS table. The table provides (roughly) the value of the operation pσ×range. The range LPS table size is 64×4: 64 LPS probability states times 4 quantized range indexes. The range index is obtained by the operation (range>>6) & 3.

- Technical description

The proposal modifies the range LPS table and the NextStateLPS table to attain a slower adaptation process and more skewed distributions. The range LPS table size is kept the same. The number of probability states is increased by 64. Parameter a is set to 0.969 for a slower adaptation (same value as in the update process of [2]). In this case, the probability of each state is derived according to the formula

p σ = a p σ -1

where the state σ is from 0 to 127 and p0=1/2. Then, the lowest possible probability is 0.009 (which is roughly half of the lowest probability in HEVC). Using these parameters, the NextStateLPS table is derived in the same way as in [1].

## Keeping current range table size

In the first case, two probability states share the same LPS table entries. The entry is selected by the right bit-shift operation σ>>1. Therefore, the range LPS table size is still 64×4. The range value is found in the table by rLPSTable[σ>>1][(range>>6) & 3]. The entries of the table are modified to account for the new probabilities described above.

## Half range table size

In the second case, range LPS table size is divided by two. The entry is selected by the right bit-shift operation σ>>2. Therefore, the range LPS table size is 32×4. The range value is found in the table by rLPSTable[σ>>2][(range>>6) & 3].

## Table reduction using bit-shifts

In the third case, the range LPS table size is reduced by 25%. The entry is indexed directly by σ. Each entry has three 4-bit values (s1, s2, s3). Therefore, the range LPS table size is 128×3. Each value is a left bit-shift to apply to the current range in order to get the new LPS range, as follows

uiLPS = ( (m_uiRange<<s1) + (m_uiRange<<s2) + (m_uiRange<<s3) )>>16

Since each entry is 4-bit instead of 8-bit, the table size is reduced. The computation of the 3 bit-shifts can be done in parallel.

The method can also be thought of as a multiplier operation, but using a three bit operand and rounding. If this expression is used, then the table becomes a 1-D table with 128 entries.

- Results

This method is implemented in HM4.0. Simulation results are presented for the high efficiency common test conditions. Table 1 shows the results for this method for the current LPS table size. The BD-rate is -0.34%, -0.26%, and -0.05%, for AI-HE, RA-HE and LB-HE configurations, respectively. Detailed results including LP configuration and class F are provided in the accompanying excel file.

The initialization states have not been changed. Only an offset of 64 has been added to match the states at probability ½. Note that this incurs in performance penalty for the presented method.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.47% | -0.20% | -0.18% | -0.49% | 0.90% | 0.40% |  |  |  |
| Class B | -0.35% | -0.64% | -0.62% | -0.34% | -0.30% | -0.27% | -0.27% | -0.23% | -0.32% |
| Class C | -0.32% | -0.82% | -0.82% | -0.15% | -0.63% | -0.74% | -0.09% | -0.57% | -0.72% |
| Class D | -0.25% | -1.25% | -1.33% | -0.02% | -0.86% | -0.93% | 0.05% | -0.95% | -1.10% |
| Class E | -0.30% | -0.50% | -0.44% |  |  |  | 0.25% | -0.82% | -0.34% |
| Overall | -0.34% | -0.69% | -0.69% | -0.26% | -0.23% | -0.38% | -0.05% | -0.61% | -0.62% |
| Enc Time[%] | 104% |  |  | 102% |  |  | 103% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  | 96% |  |  |

Table 1. BD-rates for current range LPS table size

Table 2 shows the results for this method when having an LPS table of half the current size. The BD-rate is -0.26%, -0.21%, and 0.07%, for AI-HE, RA-HE and LB-HE configurations, respectively.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.41% | -0.36% | -0.36% | -0.48% | 0.39% | 0.18% |  |  |  |
| Class B | -0.26% | -0.92% | -0.91% | -0.30% | -0.52% | -0.57% | -0.18% | -0.56% | -0.98% |
| Class C | -0.24% | -1.05% | -1.00% | -0.12% | -0.77% | -0.79% | 0.05% | -0.66% | -1.01% |
| Class D | -0.15% | -1.42% | -1.50% | 0.07% | -1.09% | -1.37% | 0.16% | -0.80% | -0.84% |
| Class E | -0.21% | -0.83% | -0.75% |  |  |  | 0.37% | -1.08% | 0.61% |
| Overall | -0.26% | -0.92% | -0.91% | -0.21% | -0.50% | -0.63% | 0.07% | -0.74% | -0.65% |
| Enc Time[%] | 101% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  | 100% |  |  |

Table 2. BD-rates for half-size range LPS table

Table 3 shows the results when applying the bit-shift operations. The BD-rate is -0.36%, -0.28%, and -0.13%, for AI-HE, RA-HE and LB-HE configurations, respectively.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.46% | 0.10% | 0.09% | -0.50% | 0.82% | 0.85% |  |  |  |
| Class B | -0.37% | -0.30% | -0.27% | -0.31% | 0.04% | -0.01% | -0.24% | 0.07% | 0.07% |
| Class C | -0.36% | -0.58% | -0.54% | -0.21% | -0.52% | -0.33% | -0.20% | -0.34% | -0.58% |
| Class D | -0.29% | -0.99% | -1.01% | -0.11% | -0.58% | -1.12% | -0.16% | -0.80% | -0.74% |
| Class E | -0.33% | -0.09% | 0.04% |  |  |  | 0.18% | 0.75% | 0.21% |
| Overall | -0.36% | -0.38% | -0.35% | -0.28% | -0.06% | -0.14% | -0.13% | -0.12% | -0.27% |
| Enc Time[%] | 102% |  |  | 103% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 96% |  |  | 97% |  |  |

Table 3. BD-rates for bit-shifts range LPS table

- Conclusions

A straightforward modification of LPS range and state transition tables in HEVC is proposed. The probability update process is more tuned to the statistics using 128 states. Performance is increased even when the LPS range table size is divided by 2.

- References

- D. Marpe, H. Schwarz, and T. Wiegand “Context-Based Adaptive Binary Arithmetic Coding in the H.264/AVC Video Compression Standard“, IEEE Trans. Circuits and Systems for Video Technology, vol. 13, no. 7, July 2003.

- “Description of video coding technology proposal by France Telecom, NTT, NTT DOCOMO, Panasonic and Technicolor”, JCTVC-A114, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- A. Alshin and E. Alshina, “Multi-parameter probability update for CABAC”, JCTVC-F254, 6th JCT-VC Meeting, Torino, IT, July 2011.

# Text for Working Draft

Flowchart in figure 9.2, the following has to be changed in each of the 3 presented cases.

In the second box, lines

qCodeIRangeIdx = (codIRange>>6) & 3

codIRangeLPS = rangeTabLPS[pStateIdx][qCodeIRangeIdx]

have to be modified as

qCodeIRangeIdx = (codIRange>>6) & 3

codIRangeLPS = rangeTabLPS[pStateIdx>>1][qCodeIRangeIdx]

in the first case (keeping the current table size). For the second case (half size table), they have to be modified as

qCodeIRangeIdx = (codIRange>>6) & 3

codIRangeLPS = rangeTabLPS[pStateIdx>>2][qCodeIRangeIdx]

For the third case, they have to be replaced by

codIRangeLPS = (codIRange  << rangeTabLPS[pStateIdx][0] + codIRange << rangeTabLPS[pStateIdx][1] + codIRange << rangeTabLPS[pStateIdx][2] ) >> 16

Table 9.53 specifying the range of LPS is changed for each of the 3 cases. For the first case, the table is:

const UChar TComCABACTables::sm_aucLPSTable[64][4] =

{

{128, 174, 206, 238}, 	{128, 164, 194, 224}, 	{126, 154, 182, 210}, 	{118, 145, 171, 197},

{111, 136, 161, 185}, 	{104, 128, 151, 174}, 	{98, 120, 142, 164}, 	{92, 112, 133, 154},

{86, 106, 125, 144}, 	{81, 99, 117, 136}, 	{76, 93, 110, 127}, 	{72, 88, 104, 120},

{67, 82, 97, 112}, 		{63, 77, 91, 105}, 	{59, 73, 86, 99}, 	{56, 68, 81, 93},

{52, 64, 76, 87}, 		{49, 60, 71, 82}, 	{46, 56, 67, 77}, 	{43, 53, 63, 72},

{41, 50, 59, 68}, 		{38, 47, 55, 64}, 	{36, 44, 52, 60}, 	{34, 41, 49, 56},

{32, 39, 46, 53}, 		{30, 36, 43, 50}, 	{28, 34, 40, 47}, 	{26, 32, 38, 44},

{25, 30, 36, 41}, 		{23, 28, 34, 39}, 	{22, 27, 31, 36}, 	{20, 25, 30, 34},

{19, 23, 28, 32}, 		{18, 22, 26, 30}, 	{17, 21, 24, 28}, 	{16, 19, 23, 27},

{15, 18, 22, 25}, 		{14, 17, 20, 23}, 	{13, 16, 19, 22}, 	{12, 15, 18, 21},

{12, 14, 17, 19}, 		{11, 13, 16, 18}, 	{10, 13, 15, 17}, 	{10, 12, 14, 16},

{9, 11, 13, 15}, 		{8, 10, 12, 14}, 		{8, 10, 12, 13}, 		{7, 9, 11, 13},

{7, 9, 10, 12}, 		{7, 8, 10, 11}, 		{6, 8, 9, 10}, 		{6, 7, 8, 10},

{5, 7, 8, 9}, 		{5, 6, 7, 9}, 		{5, 6, 7, 8}, 		{5, 6, 7, 8},

{4, 5, 6, 7}, 		{4, 5, 6, 7}, 		{4, 5, 5, 6}, 		{4, 4, 5, 6},

{3, 4, 5, 6}, 		{3, 4, 5, 5}, 		{3, 4, 4, 5}, 		{3, 3, 4, 5}

};

For the second case, the table is:

const UChar TComCABACTables::sm_aucLPSTable[32][4] =

{

{128, 174, 206, 238}, 	{126, 154, 182, 210}, 	{111, 136, 161, 185}, 	{98, 120, 142, 164},

{86, 106, 125, 144}, 	{76, 93, 110, 127}, 	{67, 82, 97, 112}, 	{59, 73, 86, 99},

{52, 64, 76, 87}, 		{46, 56, 67, 77}, 	{41, 50, 59, 68}, 	{36, 44, 52, 60},

{32, 39, 46, 53}, 		{28, 34, 40, 47}, 	{25, 30, 36, 41}, 	{22, 27, 31, 36},

{19, 23, 28, 32}, 		{17, 21, 24, 28}, 	{15, 18, 22, 25}, 	{13, 16, 19, 22},

{12, 14, 17, 19}, 		{10, 13, 15, 17}, 	{9, 11, 13, 15}, 		{8, 10, 12, 13},

{7, 9, 10, 12}, 		{6, 8, 9, 10}, 		{5, 7, 8, 9}, 		{5, 6, 7, 8},

{4, 5, 6, 7}, 		{4, 5, 5, 6}, 		{3, 4, 5, 6}, 		{3, 4, 4, 5}

};

For the third case, the table indicating the bit-shifts is:

const UChar TComCABACTables::sm_aucLPSTable[128][3] =

{

{13, 13, 14},	{13, 12, 14},	{13, 12, 14},	{13, 12, 14},

{13, 12, 14},	{13, 11, 14},	{13, 10, 14},	{13, 9, 14},

{12, 12, 14},	{12, 12, 14},	{12, 11, 14},	{12, 11, 14},

{12, 10, 14},	{12, 9, 14},	{11, 11, 14},	{11, 10, 14},

{11, 9, 14},		{11, 5, 14},	{10, 9, 14},	{9, 9, 14},

{8, 7, 14},		{12, 12, 13},	{12, 12, 13},	{12, 11, 13},

{12, 11, 13},	{12, 11, 13},	{12, 11, 13},	{12, 10, 13},

{12, 10, 13},	{12, 9, 13},	{12, 3, 13},	{11, 11, 13},

{11, 10, 13},	{11, 10, 13},	{11, 9, 13},	{11, 8, 13},

{10, 10, 13},	{10, 9, 13},	{10, 8, 13},	{10, 6, 13},

{9, 8, 13},		{9, 3, 13},	{7, 7, 13},	{11, 11, 12},

{11, 11, 12},	{11, 11, 12},	{11, 10, 12},	{11, 10, 12},

{11, 10, 12},	{11, 9, 12},	{11, 9, 12},	{11, 8, 12},

{11, 5, 12},		{10, 10, 12},	{10, 9, 12},	{10, 9, 12},

{10, 8, 12},		{10, 7, 12},	{9, 9, 12},	{9, 8, 12},

{9, 8, 12},	             {9, 6, 12},         {8, 7, 12},		{8, 4, 12},

{7, 4, 12},		{3, 2, 12},	{10, 10, 11},	{10, 10, 11},

{10, 9, 11},		{10, 9, 11},	{10, 9, 11},	{10, 8, 11},

{10, 8, 11},		{10, 7, 11},	{10, 5, 11},	{9, 9, 11},

{9, 8, 11},		{9, 8, 11},	{9, 7, 11},	{9, 6, 11},

{9, 3, 11},		{8, 7, 11},	{8, 7, 11},	{8, 5, 11},

{7, 6, 11},		{7, 4, 11},	{6, 4, 11},	{3, 3, 11},

{9, 9, 10},		{9, 9, 10},	{9, 8, 10},	{9, 8, 10},

{9, 8, 10},		{9, 7, 10},	{9, 7, 10},	{9, 6, 10},

{9, 4, 10},		{8, 8, 10},	{8, 7, 10},	{8, 7, 10},

{8, 6, 10},		{8, 6, 10},	{8, 3, 10},	{7, 7, 10},

{7, 6, 10},		{7, 4, 10},	{6, 6, 10},	{6, 4, 10},

{5, 4, 10},		{3, 2, 10},	{8, 8, 9},	{8, 8, 9},

{8, 7, 9},		{8, 7, 9},	{8, 7, 9},	{8, 6, 9},

{8, 6, 9},		{8, 5, 9},	{8, 4, 9},	{7, 7, 9},

{7, 6, 9},		{7, 6, 9},	{7, 6, 9},	{7, 5, 9},

{7, 3, 9},		{6, 6, 9},	{6, 5, 9}

};

The values fo table 9.54 specifying the state transition tables are the following ones for the first and second case:

const UChar ContextModel::m_aucNextStateMPS[ 256 ] =

{

2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17,

18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33,

34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49,

50, 51, 52, 53, 54, 55, 56, 57, 58, 59, 60, 61, 62, 63, 64, 65,

66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, 77, 78, 79, 80, 81,

82, 83, 84, 85, 86, 87, 88, 89, 90, 91, 92, 93, 94, 95, 96, 97,

98, 99, 100, 101, 102, 103, 104, 105, 106, 107, 108, 109, 110, 111, 112, 113,

114, 115, 116, 117, 118, 119, 120, 121, 122, 123, 124, 125, 126, 127, 128, 129,

130, 131, 132, 133, 134, 135, 136, 137, 138, 139, 140, 141, 142, 143, 144, 145,

146, 147, 148, 149, 150, 151, 152, 153, 154, 155, 156, 157, 158, 159, 160, 161,

162, 163, 164, 165, 166, 167, 168, 169, 170, 171, 172, 173, 174, 175, 176, 177,

178, 179, 180, 181, 182, 183, 184, 185, 186, 187, 188, 189, 190, 191, 192, 193,

194, 195, 196, 197, 198, 199, 200, 201, 202, 203, 204, 205, 206, 207, 208, 209,

210, 211, 212, 213, 214, 215, 216, 217, 218, 219, 220, 221, 222, 223, 224, 225,

226, 227, 228, 229, 230, 231, 232, 233, 234, 235, 236, 237, 238, 239, 240, 241,

242, 243, 244, 245, 246, 247, 248, 249, 250, 251, 252, 253, 254, 255, 254, 255

};

const UChar ContextModel::m_aucNextStateLPS[ 256 ] =

{

1, 0, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13,

12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27,

28, 29, 30, 31, 32, 33, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41,

42, 43, 44, 45, 46, 47, 48, 49, 48, 49, 50, 51, 52, 53, 54, 55,

56, 57, 58, 59, 60, 61, 60, 61, 62, 63, 64, 65, 66, 67, 68, 69,

70, 71, 70, 71, 72, 73, 74, 75, 76, 77, 78, 79, 78, 79, 80, 81,

82, 83, 84, 85, 84, 85, 86, 87, 88, 89, 90, 91, 90, 91, 92, 93,

94, 95, 96, 97, 96, 97, 98, 99, 100, 101, 102, 103, 102, 103, 104, 105,

106, 107, 106, 107, 108, 109, 110, 111, 110, 111, 112, 113, 114, 115, 114, 115,

116, 117, 116, 117, 118, 119, 120, 121, 120, 121, 122, 123, 122, 123, 124, 125,

124, 125, 126, 127, 128, 129, 128, 129, 130, 131, 130, 131, 132, 133, 132, 133,

134, 135, 134, 135, 136, 137, 136, 137, 138, 139, 138, 139, 140, 141, 140, 141,

140, 141, 142, 143, 142, 143, 144, 145, 144, 145, 146, 147, 146, 147, 146, 147,

148, 149, 148, 149, 148, 149, 150, 151, 150, 151, 150, 151, 152, 153, 152, 153,

152, 153, 154, 155, 154, 155, 154, 155, 156, 157, 156, 157, 156, 157, 158, 159,

158, 159, 158, 159, 158, 159, 160, 161, 160, 161, 160, 161, 160, 161, 162, 163

};

For the third case, the values of table 9.54 are:

const UChar ContextModel::m_aucNextStateMPS[ 256 ] =

{

2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17,

18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33,

34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49,

50, 51, 52, 53, 54, 55, 56, 57, 58, 59, 60, 61, 62, 63, 64, 65,

66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, 77, 78, 79, 80, 81,

82, 83, 84, 85, 86, 87, 88, 89, 90, 91, 92, 93, 94, 95, 96, 97,

98, 99, 100, 101, 102, 103, 104, 105, 106, 107, 108, 109, 110, 111, 112, 113,

114, 115, 116, 117, 118, 119, 120, 121, 122, 123, 124, 125, 126, 127, 128, 129,

130, 131, 132, 133, 134, 135, 136, 137, 138, 139, 140, 141, 142, 143, 144, 145,

146, 147, 148, 149, 150, 151, 152, 153, 154, 155, 156, 157, 158, 159, 160, 161,

162, 163, 164, 165, 166, 167, 168, 169, 170, 171, 172, 173, 174, 175, 176, 177,

178, 179, 180, 181, 182, 183, 184, 185, 186, 187, 188, 189, 190, 191, 192, 193,

194, 195, 196, 197, 198, 199, 200, 201, 202, 203, 204, 205, 206, 207, 208, 209,

210, 211, 212, 213, 214, 215, 216, 217, 218, 219, 220, 221, 222, 223, 224, 225,

226, 227, 228, 229, 230, 231, 232, 233, 234, 235, 236, 237, 238, 239, 240, 241,

242, 243, 244, 245, 246, 247, 248, 249, 250, 251, 252, 253, 252, 253, 254, 255

};

const UChar ContextModel::m_aucNextStateLPS[ 256 ] =

{

1, 0, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 10, 11,

12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27,

28, 29, 30, 31, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41,

42, 43, 44, 45, 46, 47, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55,

56, 57, 58, 59, 58, 59, 60, 61, 62, 63, 64, 65, 66, 67, 68, 69,

68, 69, 70, 71, 72, 73, 74, 75, 76, 77, 76, 77, 78, 79, 80, 81,

82, 83, 82, 83, 84, 85, 86, 87, 88, 89, 88, 89, 90, 91, 92, 93,

94, 95, 94, 95, 96, 97, 98, 99, 98, 99, 100, 101, 102, 103, 104, 105,

104, 105, 106, 107, 108, 109, 108, 109, 110, 111, 110, 111, 112, 113, 114, 115,

114, 115, 116, 117, 118, 119, 118, 119, 120, 121, 120, 121, 122, 123, 122, 123,

124, 125, 126, 127, 126, 127, 128, 129, 128, 129, 130, 131, 130, 131, 132, 133,

132, 133, 134, 135, 134, 135, 136, 137, 136, 137, 138, 139, 138, 139, 138, 139,

140, 141, 140, 141, 142, 143, 142, 143, 144, 145, 144, 145, 144, 145, 146, 147,

146, 147, 146, 147, 148, 149, 148, 149, 148, 149, 150, 151, 150, 151, 150, 151,

152, 153, 152, 153, 152, 153, 154, 155, 154, 155, 154, 155, 156, 157, 156, 157,

156, 157, 156, 157, 158, 159, 158, 159, 158, 159, 158, 159, 160, 161, 254, 255

};

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).