| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G327 |
| --- | --- |

| Title: | CE11: Cross-check of TI’s reduction in contexts used for coefficient level (JCTVC-G121) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The contribution confirms the results presented in the CE11 contribution JCTVC- G121 on the reduction of the number of contexts employed for the absolute value of the level coding with CABAC.

- Introduction

Software implementing JCTVC-F132 / JCTVC-G121 [2] on top of HM4.0 basis was released by TI as part of CE11.

Following files are modified:

- TypeDef.h, TEncSbac.cpp, TDecSbac.cpp and TComTrQuant.cpp

Code changes are little. The change is to limit the maximum count number for Larger than 1 and Larger than 2, which are set to 3 and 2 (instead of 4), respectively. Complexity of the algorithm is the same as the anchor.

These changes reduce the luma number of contexts by 6 and 12 for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag, respectively. Chroma contexts reduction is the same. However, the code should be further modified to fully reflect these changes: the corresponding context initialization tables in ContextTables.h and macros should also be modified.

- Results

Coding results using common conditions (High-Efficiency settings) are summarized in the tables below. HM4.0 is the anchor. Note that decoding times are not reliable since different types of machines were used to decode.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.04% | 0.02% | 0.06% | 0.00% | 0.17% | 0.14% |  |  |  |
| Class B | 0.07% | 0.08% | 0.08% | 0.02% | 0.02% | 0.09% | 0.02% | -0.17% | -0.07% |
| Class C | 0.05% | 0.10% | 0.08% | 0.02% | -0.15% | -0.07% | 0.02% | -0.11% | -0.27% |
| Class D | 0.08% | 0.10% | 0.09% | 0.09% | 0.28% | -0.22% | 0.03% | 0.57% | 0.09% |
| Class E | 0.03% | 0.05% | 0.09% |  |  |  | 0.03% | -0.50% | 0.08% |
| Overall | 0.06% | 0.07% | 0.08% | 0.03% | 0.08% | -0.01% | 0.02% | -0.03% | -0.05% |
|  | 0.06% | 0.07% | 0.07% | 0.03% | 0.04% | -0.05% | 0.03% | -0.03% | -0.12% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  | 103% |  |  |

Results for class F are the following:

| Class F | 0.11% | 0.14% | 0.26% | 0.07% | 0.10% | -0.09% | -0.11% | -0.29% | 0.36% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Additionally, common conditions with RDOQ off were considered to be a testing point. Results for that case are found in the table below.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.02% | 0.07% | 0.08% | 0.01% | -0.02% | 0.08% |  |  |  |
| Class B | 0.05% | 0.05% | 0.06% | 0.05% | -0.02% | 0.20% | 0.00% | -0.11% | -0.10% |
| Class C | 0.04% | 0.06% | 0.06% | -0.02% | 0.00% | 0.07% | 0.03% | 0.28% | 0.13% |
| Class D | 0.07% | 0.04% | 0.04% | 0.00% | 0.07% | -0.01% | 0.09% | -0.07% | -0.26% |
| Class E | 0.03% | 0.17% | 0.04% |  |  |  | -0.03% | -0.27% | -0.22% |
| Overall | 0.04% | 0.07% | 0.06% | 0.01% | 0.01% | 0.09% | 0.02% | -0.03% | -0.11% |
|  | 0.04% | 0.07% | 0.05% | 0.01% | -0.01% | 0.03% | 0.02% | 0.02% | -0.10% |
| Enc Time[%] | 104% |  |  | 98% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  | 107% |  |  |

- References

- V. Sze, “Reduction in contexts used for significant_coeff_flag and coefficient level”, JCTVC-F132, 6th JCT-VC Meeting, Torino, IT, July 2011.

- V. Sze, “CE11: Reduction in contexts used for coefficient level”, JCTVC-G121, 7th JCT-VC Meeting, Geneva, CH, Geneva 2011.

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.