| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G332 WG11 Number: m21894 |
| --- | --- |

| Title: | Multiple Adaptation Parameter Sets Referring |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming LI Ping WU No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | +86-25-88014641 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In this document, a multiple adaptation parameter sets (APSs) referring approach is proposed. Since APS tends to carry the coding parameters which are more likely to be changeable from picture to picture, or even from slice to slice, therefore, such a referring design is proposed to be at slice layer. In a slice layer encoding/decoding, multiple APS IDs can be made available, and each coding tool, such as SAO or ALF, etc., is initialized by activating only once in one APS of the several referred ones. Thereby, the encoder is allowed to code SAO and/or ALF parameters by directly utilizing whole or partial information already presented in the existing APSs. This facilitates flexible configuration of the coding tools and saves bits when re-using the former tool parameters while coding the current slice. The proposed scheme can also serve both the case when parameters change from slice to slice, such as in video coding tool with Weighted Prediction, and the case when parameters intend to be unchanged for the whole sequence.

# Introduction

At the 6th JCT-VC Meeting, adaptation parameter set (APS) was developed and adopted to HEVC based on the contribution JCTVC-F747 [1] by Vidyo, MediaTek and ZTE, which is intended for picture-adaptive data. Currently, some parameters (i.e. slice layer ON/OFF flags, filter coefficients, etc) of two kinds of loop filters, i.e. sample adaptive offset (SAO) and adaptive loop filter (ALF), are coded in APS and an APS ID is presented in slice header. When decoding a slice, the referred APS is activated and the filter information is used to initialize both SAO and ALF.

Although APS provides an effective approach to achieve slice level common information sharing, the approach that one slice can only refer to one APS to configure both ALF and SAO could bring a problem of duplicating of the coded APS information. Take a scenario that the following slice uses the same ALF parameters as the former coded slice but different SAO parameters for example. In this case, the encoder needs to generate a new APS containing new SAO information but repeated ALF information, where such ALF information leads to the waste of bits.

Moreover, when SAO and ALF are allowed but are not used in coding a picture, it costs quite a few bits to send such OFF information. For instance, an APS containing SAO and ALF OFF flags can be generated and referred to in the corresponding slices, which requires extra bits for APS itself and several other bits for APS ID in slice header besides two valid control flags.

To tackle this problem, a new slice layer APS referring method is proposed. This method allows a slice to refer to multiple APSs to get the parameters of the coding tools before decoding, however, a tool can only be initialized by activating once from the referred APSs and taking the corresponding information from activated APS. To configure another tool, a different APS may be activated. The multiple APSs can be activated at the same time; but for each corresponding coding tool parameters, they can only be activated once for a slice. Thereby, the proposed method equips an encoder with the ability to directly employ whole or partial parameters already in one or several existing APSs to code SAO and/or ALF for the current picture, which saves bits from coding repeated information. From another perspective, an encoder can flexibly re-use the former applied SAO and ALF separately in coding the current picture.

This proposal is organized as follows. In section 2, based on the APS syntax in JCTVC-F747, the first proposed approach is described. In section 3, two flags are added in the APS, and then another slice layer multiple APSs referring method is designed using the modified APS syntax. Finally, conclusions are given in section 4. Both proposed methods in this document provide complete solution to the highlighted problem independently.

# Slice Layer Multiple APSs Referring Approach (I)

## Modifications

The modifications of syntax and semantics for approach I are listed in the table below.

Table 2.1  Modifications for approach I

| APS Syntax | No |
| --- | --- |
| Slice Header | Adding new syntax elements signaling multiple APSs’ IDs. |
| Others | Adding decoding rules. |

## Modified Syntax and Semantics

The modified slice header is presented in Table 2.2, where the added syntax elements are highlighted in yellow bars.

Table 2.2  Modified slice header syntax for approach I

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) { |  |
| number_of_valid_aps | u(v) |
| if (number_of_valid_aps != 0 ) { |  |
| for (i = 0; i < number_of_valid_aps; i++) { |  |
| aps_id[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| …… |  |
| } |  |
| …… |  |
| } |  |

The semantics of the new syntax elements are presented as below.

number_of_valid_aps specifies the number of valid APSs in decoding the slice. The value of number_of_valid_aps shall be in the range of [0, MaxNumberValidAps], inclusively. The value of MaxNumberValidAps is specified in Profile/Level. If number_of_valid_aps does not exist, its value is inferred to be 0. If number_of_valid_aps is equal to 0, it is inferred as no active APSs.

aps_id[i] specifies the valid APSs IDs in decoding the slice, where i shall be in the range of [0, number_of_valid_aps-1], inclusively.

## Decoding Rules

When the number_of_valid_aps value equals to 0 or is not presented, i.e. no APS is referred to in slice header, the flags of aps_sample_adaptive_offset_flag and aps_adaptive_loop_filter_flag in decoding this slice shall be set to 0, and neither SAO nor ALF shall be applied in decoding the slice.

When the number_of_valid_aps value equals to 1, i.e. only one APS is referred to in slice header, the flags of aps_sample_adaptive_offset_flag and aps_adaptive_loop_filter_flag and the tool parameters of SAO and ALF in decoding this slice shall be set equal to the values of the corresponding syntax elements presented in this referred APS.

When number_of_valid_aps is larger than 1, i.e. when multiple APSs approach is applied, for SAO, if all the aps_sample_adaptive_offset_flag elements in the referred APSs are equal to 0, the aps_sample_adaptive_offset_flag in decoding this slice shall be set to 0 and SAO shall not be applied in decoding the slice. Otherwise, if one and only one APS contains the aps_sample_adaptive_offset_flag equal to 1, the aps_sample_adaptive_offset_flag in decoding this slice shall be set to 1 and SAO with the parameters parsed from sao_param() in this APS shall be applied in decoding this slice. Otherwise, if there are more than one APS containing aps_sample_adaptive_offset_flag equal to 1, the aps_sample_adaptive_offset_flag in decoding this slice shall be set to 0 and SAO shall not be applied in decoding the slice.

When number_of_valid_aps is larger than 1, i.e. when multiple APSs approach is applied, for ALF, if all the aps_adaptive_loop_filter_flag elements in the referred APSs are equal to 0, the aps_adaptive_loop_filter_flag in decoding this slice shall be set to 0 and ALF shall not be applied in decoding the slice. Otherwise, if one and only one APS contains the aps_adaptive_loop_filter_flag equal to 1, the aps_adaptive_loop_filter_flag in decoding this slice shall be set to 1 and ALF with the parameters parsed from alf_param() in this APS shall be applied in decoding this slice. Otherwise, if there are more than one APS containing aps_sample_adaptive_offset_flag equal to 1, the aps_adaptive_loop_filter_flag in decoding this slice shall be set to 0 and ALF shall not be applied in decoding the slice.

In other words, when multiple APSs are referred by a slice, for each tool, the APS that is the only one APS containing ON/OFF flag of this tool (i.e. aps_sample_adaptive_offset_flag or aps_adaptive_loop_filter_flag) equal to 1 among the referred APSs is activated to initialize this tool before decoding this slice.

# Slice Layer Multiple APSs Referring Approach (II)

## Modifications

The modifications of syntax and semantics for approach II are listed in the table below.

Table 3.1  Modifications for approach II

| APS | Adding two new elements signaling whether SAO or ALF parameters are presented in this APS; Moving SAO and ALF ON/OFF control flags to the “if(sao/alf_data_present_flag)” branches. |
| --- | --- |
| Slice Header | The same modifications as those in section 2. |
| Others | Adding decoding rules. |

## Modified Syntax and Semantics

The modified APS syntax is presented in Table 3.2, where the added syntax elements are highlighted in yellow bars.

Table 3.2  Modified APS Syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_sample_adaptive_offset_data_present_flag | u(1) |
| aps_adaptive_loop_filter_data_present_flag | u(1) |
| if( aps_sample_adaptive_offset_data_present_flag ¦¦ aps_adaptive_loop_filter_data_present_flag ) { |  |
| aps_cabac_use_flag | u(1) |
| if( aps_cabac_use_flag ) { |  |
| aps_cabac_init_idc | ue(v) |
| aps_cabac_init_qp_minus26 | se(v) |
| } |  |
| } |  |
| /* Insert non-CABAC stuff above this line */ |  |
| if( aps_adaptive_loop_filter_data_present_flag ) { |  |
| aps_adaptive_loop_filter_flag | u(1) |
| if (aps_adaptive_loop_filter_flag) { |  |
| alf_data_byte_count /* to enable skipping past data without parsing it */ | u(8) |
| /* byte_align() this byte align to happen between the non-CABAC and CABAC parts of the alf_param() Once there is an all CABAC alf_param(), enable this byte_align() */ |  |
| alf_param( ) |  |
| byte_align() |  |
| } |  |
| } |  |
| /* insert CABAC stuff below this line; make sure its byte-aligned */ |  |
| if( aps_sample_adaptive_offset_data_present_flag ) { |  |
| aps_sample_adaptive_offset_flag | u(1) |
| if ( aps_sample_adaptive_offset_ flag ) { |  |
| sao_data_byte_count /* to enable skipping past data without parsing it */ | u(8) |
| byte_align () |  |
| sao_param( ) |  |
| /* byte_align() this final byte align unnecessary as being taken care of by rbsp_trailing_bits() */ |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The semantics of the new elements in APS syntax are presented below.

aps_sample_adaptive_offset_data_present_flag equal to 1 specifies that the SAO parameters exists in this APS, equal to 0 specifies that the SAO parameters does not exist in this APS, where the SAO parameters refer to the SAO enable flag and SAO parameters when SAO enable flag is 1.

aps_sample_adaptive_offset_flag equal to 1 specifies that the SAO is on for slices referred to the current APS; equal to 0 specifies that the SAO is off for slices referred to the current APS. If there is no active APS or the aps_sample_adaptive_offset_flag value equals to 0, the aps_sample_adaptive_offset_flag value is inferred to be 0.

aps_adaptive_loop_filter_data_present_flag equal to 1 specifies that the ALF parameters exist in this APS, equal to 0 specifies that the ALF parameters does not exist in this APS, where the ALF parameters refer to the ALF enable flag and ALF parameters when ALF enable flag is 1.

aps_adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for slices referred to the current APS; equal to 0 specifies that the ALF is off for slices referred to the current APS. If there is no active APS or the aps_adaptive_loop_filter_data_present_flag value equals to 0, the aps_adaptive_loop_filter_flag value is inferred to be 0.

Other slice layer modified syntax and corresponding semantics for this approach is the same as those described in section 2.2.

## Decoding Rules

The decoding rules for the cases when the number_of_valid_aps value equals to 0 or 1 are the same as the corresponding rules specified in section 2.3.

When number_of_valid_aps is larger than 1, the APS with its ID equal to aps_id[0] can be hypothetically taken as the basic APS in decoding the current slice, while the other APSs with their IDs equal to aps_id[1], aps_id[2], … aps_id[number_valid_aps -1] as the amendment APSs. The presented information in the amendment APSs with its APS ID of aps_id[i] (i > 0), which refers to the tool parameters (i.e. aps_sample_adaptive_offset_flag and sao_param() for SAO and/or aps_adaptive_loop_filter_flag and alf_param() for ALF) presented in an APS when aps_sample_adaptive_offset_data_present_flag is 1 and/or aps_adaptive_loop_filter_data_present_flag is 1, overwrites the information of the corresponding tool(s) formerly and temporally determined by the APSs with their APS ID of aps_id[i-1], …, aps_id[0]. Thereby, the finally obtained value of aps_sample_adaptive_offset_flag and the parameters in sao_param() are used for implementing SAO in decoding the current slice, and the finally obtained aps_adaptive_loop_filter_flag and the parameters in alf_param() are used for implementing ALF in decoding the current slice.

In other words, when multiple APSs are referred by a slice, for each tool, according to the presentation order of the APS IDs, the last APS that contains the data_present_flag (i.e. aps_sample_adaptive_offset_data_present_flag or aps_adaptive_loop_filter_data_present_flag) of this tool equal to 1 is activated to initialize this tool before decoding this slice.

# Conclusions

A slice layer multiple APSs referring approach is proposed. When coding a slice, for each video coding tool whose parameters are specified in APSs, one activated APS is determined following the proposed decoding rules to initialize each tool. Thereby, the former used SAO and/or ALF parameters and their controlling flags can be separately re-used while coding the current picture without introducing much additional overhead bits. Meanwhile, the encoder is also enabled to flexibly configure and utilize the video coding tools specified in APSs. The recommendation is to adopt slice layer multiple APSs referring design described in this document to HEVC.

# References

- Stephan Wenger, Jill Boyce, Yu-Wen Huang, Chia-Yang Tsai, Ping Wu and Ming Li, “Adaptation Parameter Set (APS)”, JCT-VC Document JCTVC-F747, July 2011, Torino, Italy.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).